3GPP TSG-RAN WG4 Meeting #116							         	       	R4-2509243

# Bengaluru, India, 25th – 29th August, 2025

Title: 	MPR input for PC1 NB-IoT-NTN

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.6.3.3.1	Tx requirements	[NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In RAN#115, we provided a preliminary evaluation in [1] of the PC2 NB-IoT-NTN MPR for the corner case allocations. A way forward [2] captured a few agreements on the 12-tone and 1-tone 15KHz SCS edge and center allocations. It was observed that most of the allocations have not been evaluated. In this contribution, we provide the MPR evaluations for all the applicable allocations.

# Discussion

## 2.1 Way forward input on PC1 NB-IoT-NTN

The way forward [2] lists the open items for PC1 NB-IoT based IoT-NTN MPR and A-MPR for single tone and multi-tone allocations, and is copied below in italics.

Issue 1-1-3: MPR requirements for NB-Iot based Iot-NTN PC1

- Agreement:

  - Try to agree the following parameters and tentative requirements as following (as high lightened in the summary table, with already two sets of measurements and one set of simulation):

    - Tone positions for 1 Tone allocation for 15KHz SCS:

      - Using option 1, which divided to three region in square brackets [0~2], [3~8], [9~11]

    - MPR requirements for 1 Tone allocation for 15KHz SCS

      - [1.5~2dB] for edge regions, and [0.5dB] for center region

      - Unified requirement for pi/2 BPSK and pi/4 QPSK

    - MPR requirements for 12 Tone allocation for 15KHz SCS

      - [5-5.5dB]

  - These requirements are targeted for all form factors.

  - The remaining requirements (3/6 tones allocation, 1Tone allocation for 3.75kHz SCS) would be discussed in next meeting.

  - Using following structure as starting point. Further refinements on tables and wording can be made later

Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [8-11] | [0-TBD] | TBD | [TBD-47] |
| MPR | ≤[1.5~2] dB | [0.5] dB | ≤[1.5~2] dB | ≤ TBD dB | [0.5] dB | ≤ TBD dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |  |  |

Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤TBD dB | TBD dB |  | ≤TBD dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤TBD dB |  | ≤ TBD dB |  |
| [Tone positions for 12 Tones allocation] | 0-11 |  |  |  |
| [MPR] | ≤ [5-5.5] dB |  |  |  |
| Note 1: For multi-tone allocation, only QPSK and 15KHz SCS can be supported |  |  |  |  |

Issue 1-1-4: Further MPR/A-MPR assumptions for NB-Iot based Iot-NTN in further evaluation;

- Agreement:

  - [3] dB post-PA insertion losses for calibration purpose

  - For PC2, PC3 NR PA can also be considered for reference architecture

    - Calibration point for PC2 (PC3 NR PA) (further update based on offline common view):

      - DFT-s-OFDM QPSK 20 MHz 100RB0

      - 4dB post PA loss, 1dB MPR

      - Carrier leakage: 28 dBc, IQ image: 28 dBc, CIM3: 60 dBc

      - NR ACLR: 30dBc, other number is not precluded

    - Calibration based on NB-IOT signal is not precluded.

  - For PC1, consider adding [-34] dBc IQ-image and LO-leakage and 60 dBc CIM3 as another option for evaluation

    - Calibration points for PC1 (based on NR PC2 PA) (further update based on offline common view):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 / [-34]dBc

      - CIM3: 60 dBc

      - NR ACLR: 31dBc, other number is not precluded

    - Calibration based on NB-IOT signal is not precluded.

As observed, except for the 12-tone and 1-tone 15KHz SCS edge and center allocations, most of the allocations have not been evaluated. In this contribution, we provide the MPR and A-MPR evaluations for all the applicable allocations.

## 2.2 PC1 NB-IoT-NTN measurements based on GSM and PC2 NR PAs

Further measurements were performed on the same GSM and NR PC2 PA used in [1] to include the missing single-tone and multi-tone allocations The PA calibrations compared to [1] were also reconsidered and measured at the nominal VCC for both PAs which is more appropriate for the low PAPR waveforms:

- QPSK 3, 6 and 12-tone allocations. Only the allocations on the left side of the carrier were measured.

- Single-tone Pi/4 QPSK and Pi/2 BPSK 15kHz all positions (0 to 11).

- Single-tone Pi/4 QPSK and Pi/2 BPSK 3.75kHz every 4 positions (1,4,8,12,16,20,23).

- Post-PA losses of 3dB (with 0.5dB correction for the NR PC2 PA since it has an integrated post PA switch).

- With 28dB and 34dB image impairments to check the influence with the baseline carrier leakage, CIM3 and CIM5.

- Performed tests: Occupied BW, UTRA ACLR, NB-IoT SEM, 1dB compression point and compression level at SEM limit.

- IBE could not be tested directly but for 15kHz tone the in-band spectrum in 15kHz bins can be measured and represent a worst-case versus IBE which is after FFT and BB corrections.

Observation on occupied BW: In our measurement campaign we found that the 12-tone input waveform from the measurement instrument would not meet the occupied BW and that there was no impact from the PA. Since the BB waveform should be designed to meet the occupied BW by applying the proper digital filtering, we ignored this fail as it is not related to the PA.

The raw data for all the cases and the two PAs can be found in the Annex 1.

Observations on raw results:

- In all the cases, the limitation comes from the NB-IoT SEM limit.

- Most of the inner single-tones allocations can reach up to 31dBm but requires saturating the PA (into >4dB compression).

- Other than for the 3.75kHz edge allocations, the improved image rejection has little benefit.

- There is a slight difference between Pi/4 QPSK and Pi/2 BPSK

Given the little impact of image rejection we focussed our evaluation on 28dB image rejection numbers which summarized in Table 1.

Table 1: evaluated NR PC2 PA and GSM PA MPR per waveforms

|  | Tone position and MPR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SC# 3.75kHz | 0-3/44-47 | 4-7/40-43 | 8-11/36-39 | 12-15/32-35 | 16-19/28-31 | 20-27 |
| 1 tone Pi/2 BPSK PC2 NR PA/GSM PA | 3/3.5 | 1/1 | 0.5/0.5 | 0.5/0.5 | 0.5/0.5 | 0/0.5 |
| 1 tone Pi/4 QPSK PC2 NR PA/GSM PA | 3/3 | 1/1.5 | 0.5/0.5 | 0.5/0.5 | 0.5/0.5 | 0.5/0.5 |
| SC# 15kHz | 0/11 | 1/10 | 2/9 | 3/8 | 4/7 | 5/6 |
| 1 tone Pi/2 BPSK PC2 NR PA/GSM PA | 2/2.5 | 1/1 | 0.5/1 | 0.5/1 | 0.5/0.5 | 0.5/0.5 |
| 1 tone Pi/4 QPSK PC2 NR PA/GSM PA | 1.5/2 | 1/1.5 | 0.5/1 | 0.5/1 | 0.5/0.5 | 0.5/0.5 |
| SC# 15kHz | 0-2/9-11 |  |  | 3-5/6-8 |  |  |
| 3 tone QPSK PC2 NR PA/GSM PA | 2/2.5 |  |  | 1.5/2 |  |  |
| SC# 15kHz | 0-5/6-11 |  |  |  |  |  |
| 6 tone QPSK PC2 NR PA/GSM PA | 2.5/3 |  |  |  |  |  |
| SC# 15kHz | 0-11 |  |  |  |  |  |
| 12 tone QPSK PC2 NR PA/GSM PA | 4/4.5 |  |  |  |  |  |

Observations:

- The GSM PA needs slightly higher backoff (about 0.5dB due to rounding effects) compared to the PC2 NR PA. However, it is measured at 3.5V instead of 3.8V. Since they could both use 3.8V supply they in fact could achieve the same performance.

- Other than for some edge allocations, reaching close to 31dBm is feasible for most 1-tone allocation

- For Pi/4 QPSK vs Pi/2 BPSK, outside some 0.5dB rounding, the difference in MPR is small.

- For 3.75kHz versus 15KHz single tone, the main difference is for the edge allocations (PSD)

## 2.3 Estimation of IBE

In band emissions for NB-IoT could not be tested directly in our setup, but for 15kHz SCS single-tone and multi-tone cases, the in-band spectrum in 15kHz bins can be measured. Although it does not provide the exact IBE value that is calculated after FFT and BB corrections, it represents a worst-case evaluation of IBE and permits to investigations into whether this might be a limiting factor for MPR instead of the NB-IoT SEM. The evaluation can be done for 3 and 6-tone QPSK and 15kHz SCS 1-tone Pi/4. The Table 2 below provides the power ratio [dB] between non-allocated tones and allocated tones for:

- QPSK 3-tone edge, 3-tone centre and 6-tone

- Pi/4 QPSK 15kHz 1-tone position 0 to 5

- At the maximum measured power

Table 2: Power ratio [dB] between non-allocated tones and allocated tones

| Non-Allocated | Tone position |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| 3tone 0-2 | 0.2 | -1.0 | 0.8 | 9.4 | 15.9 | 18.9 | 20.7 | 23.0 | 24.0 | 23.9 | 24.7 | 25.7 |
| 3tone 3-5 | 19.6 | 16.2 | 9.4 | -0.3 | -0.7 | 1.0 | 9.1 | 15.5 | 18.7 | 21.2 | 23.1 | 24.5 |
| 6tone 0-5 | 0.4 | -0.5 | 0.2 | -0.5 | -0.1 | 0.5 | 8.6 | 14.9 | 17.0 | 18.5 | 19.8 | 21.7 |
| 1tone 0 | 0.0 | 9.5 | 17.9 | 21.6 | 24.0 | 24.6 | 25.5 | 27.4 | 28.7 | 29.8 | 30.5 | 28.9 |
| 1tone 1 | 9.4 | 0.0 | 9.4 | 18.2 | 21.9 | 23.8 | 25.5 | 27.1 | 28.0 | 29.0 | 27.7 | 30.9 |
| 1tone 2 | 18.2 | 9.4 | 0.0 | 9.4 | 18.1 | 21.5 | 23.9 | 26.3 | 27.4 | 26.8 | 28.9 | 30.4 |
| 1tone 3 | 21.9 | 18.0 | 9.4 | 0.0 | 9.5 | 17.9 | 21.6 | 24.3 | 25.0 | 27.5 | 29.0 | 29.7 |
| 1tone 4 | 24.3 | 21.8 | 18.1 | 9.2 | 0.0 | 9.5 | 17.8 | 21.4 | 24.1 | 26.2 | 27.7 | 29.0 |
| 1tone 5 | 26.4 | 24.5 | 22.1 | 18.1 | 9.3 | 0.0 | 9.4 | 18.2 | 21.9 | 24.4 | 26.3 | 27.9 |

Observation: This Table can be used to compare with other IBE input in the meeting.

## 2.4 MPR proposal

Although our evaluation shows that many allocations can reach close to 31dBm some aspects may need to be considered:

- For above 30.5dBm the PA need to be pushed in hard compression (>3dB compression) but below this the compression level is reasonable.

- The NR PC2 PA has less gain than the GSM PA (one less stage), this requires enough drive from the transceiver

- The output power needs to be properly controlled for the different allocations to meet the SEM (Similar to the GSM PA power control)

- To reach IBE and EVM at close to 31dBm, specific waveform design (CFR, spectrum shaping) may be required for 1-tone allocations.

Given that the GSM PA could use the same supply voltage as the NR PA, our MPR proposal is based on the NR PC2 PA at nominal voltage. To simplify the MPR table we also merged the MPR values for 15KHz and 3.75kHz SCS, and Pi/2 BPSK and Pi/4 QPSK.

Proposal on MPR table: The following table is used for PC1 NB-IoT-NTN requirement:

Table 2: Proposed baseline MPR for PC1 NB-IoT-NTN

| Waveform SCS and number of tones | Tone position and MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| SC# 15kHz/3.75kHz | 0 and 11/0-3 and 44-471 | 1 and 10/4-7 and 40-43 | 2-9/8-39 |
| 1 tone Pi/2 BPSK or Pi/4 QPSK | 2 | 1 | 0.5 |
| SC# 15kHz | 0-2 and 9-11 |  | 3-5 and 6-8 |
| 3 tone QPSK | 2 |  | 1.5 |
| SC# 15kHz | 0-5 and 6-11 |  |  |  |
| 6 tone QPSK | 0 |  |  |  |
| SC# 15kHz | 0-11 |  |  |  |
| 12 tone QPSK | 4 |  |  |  |
| Note 1: except for 0-3 and 44-47 3.75kHz tone positions where MPR is 3dB |  |  |  |  |

# Conclusion

In this contribution, we evaluated the emissions for the missing allocations in our earlier measurements in [1] and make the following proposals.

Proposal on MPR table: The following table is used for PC1 NB-IoT-NTN requirement:

Table 2: Proposed baseline MPR for PC1 NB-IoT-NTN

| Waveform SCS and number of tones | Tone position and MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| SC# 15kHz/3.75kHz | 0 and 11/0-3 and 44-471 | 1 and 10/4-7 and 40-43 | 2-9/8-39 |
| 1 tone Pi/2 BPSK or Pi/4 QPSK | 2 | 1 | 0.5 |
| SC# 15kHz | 0-2 and 9-11 |  | 3-5 and 6-8 |
| 3 tone QPSK | 2 |  | 1.5 |
| SC# 15kHz | 0-5 and 6-11 |  |  |  |
| 6 tone QPSK | 0 |  |  |  |
| SC# 15kHz | 0-11 |  |  |  |
| 12 tone QPSK | 4 |  |  |  |
| Note 1: except for 0-3 and 44-47 3.75kHz tone positions where MPR is 3dB |  |  |  |  |

# References

[1] R4-2505834 NR and GSM PAs measurements for NB-IoT-NTN HPUE, Skyworks Solutions Inc., RAN4#115

[2] R4-2508122 WF on NR_IoT_NTN_HPUE_part2, Vivo, RAN4#115

# Annex 1: Raw measurement data

Table A1 provides the RAW measured data for the backoff needed to meet the NB-IoT SEM mask which is the limiting factor for all the measured single-tone and multi-tone allocations, modulations and SCS, and the associated PA compression level. Data is available for 28 and 34dB image rejection.

Table A 1: Backoff and compression level at SEM limit for 28 and 34dB image rejection vs waveforms for the PC2 NR PA and the GSM PA

| Waveform |  |  |  | PC2 NR PA 3.8V |  |  |  | GSM PA 3.5V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 28dB image |  | 34dB image |  | 28dB image |  | 34dB image |  |
| Mod | SCS | #Tone | Tone# | comp | Backoff | comp | Backoff | comp | Backoff | comp | Backoff |
| QPSK | 15kHz | 3 | 0 | 1.2 | 1.8 | 2 | 1.8 | 1.7 | 2.4 | 1.5 | 2.4 |
|  |  | 3 | 3 | 1.5 | 1.4 | 1.3 | 1.4 | 1.5 | 1.9 | 1.6 | 1.8 |
|  |  | 6 | 0 | 1.5 | 2.2 | 1.5 | 2.2 | 1 | 3 | 1.5 | 2.7 |
|  |  | 12 | 0 | 0.6 | 3.8 | 0.4 | 3.8 | 0.3 | 4.7 | 0.4 | 4.6 |
| Pi/2 BPSK |  | 1 | 0 | 1.7 | 1.8 | 1.9 | 1.7 | 1.8 | 2.2 | 2 | 2.1 |
|  |  | 1 | 1 | 1.5 | 0.8 | 1.3 | 0.7 | >1.5 | 1 | 1.5 | 0.9 |
|  |  | 1 | 2 | 2 | 0.5 | 2.4 | 0.4 | 2 | 0.8 | 2.5 | 0.7 |
|  |  | 1 | 3 | 2.4 | 0.3 | 2 | 0.3 | 2.5 | 0.8 | 2.6 | 0.6 |
|  |  | 1 | 4 | 3.3 | 0.2 | 3.3 | 0.2 | 3 | 0.4 | 3.2 | 0.4 |
|  |  | 1 | 5 | 3.5 | 0.1 | 3.8 | 0.1 | 4.5 | 0.3 | 3.8 | 0 |
| Pi/4QPSK |  | 1 | 0 | 1.8 | 1.4 | 2.1 | 1.4 | 2.1 | 1.8 | 2 | 1.7 |
|  |  | 1 | 1 | 2 | 0.5 | 1.2 | 0.4 | 1.1 | 1.5 | 1.4 | 1.2 |
|  |  | 1 | 2 | 1.9 | 0.4 | 1.8 | 0.5 | 1.6 | 1 | 1.7 | 0.9 |
|  |  | 1 | 3 | 2.2 | 0.3 | 2 | 0.3 | 2 | 0.5 | 1.8 | 1 |
|  |  | 1 | 4 | 2.3 | 0.4 | 2.3 | 0.3 | 2.3 | 0.6 | 2.3 | 0.5 |
|  |  | 1 | 5 | 2.4 | 0.3 | 2.2 | 0.3 | 2.8 | 0.3 | 2.3 | 0.4 |
| Pi/2 BPSK | 3.75kHz | 1 | 0 | 1 | 3 | 2.4 | 2.4 | 1 | 3.4 | 2.3 | 2.5 |
|  |  | 1 | 4 | 1.6 | 0.6 | 2 | 0.4 | 1.5 | 1 | >2 | 0.6 |
|  |  | 1 | 8 | 3.3 | 0.2 | >2 | 0.2 | 3.5 | 0.6 | 3.5 | 0.2 |
|  |  | 1 | 12 | 2.7 | 0.1 | 3 | 0 | 3.4 | 0.2 | 3.5 | 0.2 |
|  |  | 1 | 16 | 4 | 0.1 | 4.3 | 0 | 4.5 | 0.3 | 4.5 | 0.1 |
|  |  | 1 | 20 | 5.5 | 0 | 4.2 | 0 | 5 | 0.2 | 4.7 | 0.2 |
|  |  | 1 | 23 | 4.5 | 0 | >5 | 0 | 5 | 0.3 | 4.8 | 0.1 |
| Pi/4QPSK |  | 1 | 0 | 1.5 | 3 | 3.8 | 2.1 | 1.5 | 3.1 | 5 | 2.1 |
|  |  | 1 | 4 | 1.4 | 0.7 | 1.8 | 0.5 | 2 | 1.2 | 1.5 | 1 |
|  |  | 1 | 8 | 3 | 0.2 | 2.1 | 0.3 | 3.5 | 0.1 | 2.5 | 0 |
|  |  | 1 | 12 | >3 | 0.1 | 2 | 0.1 | 2 | 0 | 3 | 0 |
|  |  | 1 | 16 | 3.6 | 0 | 3.4 | 0 | 4 | 0 | 3.4 | 0 |
|  |  | 1 | 20 | 3.8 | 0.1 | 4 | 0.1 | 3.1 | 0 | 4 | 0 |
|  |  | 1 | 23 | 4 | 0 | 3.8 | 0.1 | 3.3 | 0 | 3.2 | 0.1 |