3GPP TSG-RAN WG4 Meeting #116                                                  	R4-2509245

# Bengaluru, India, 25th – 29th August, 2025

# Agenda item:			7.7.6 / 7.7.5

# Source:					Eutelsat Group

# Title:						Intra-SAN (own-system interference) ACLR analysis for Ku band

# Document for:		Discussion

# Overview

This contribution considers the case of intra-operator, intra-SAN service link (SAN – UE) and derives the SAN and VSAT ACLR values required to achieve an acceptable capacity loss.

Previous work in Rel-18 considered SAN ACLR from the perspective of TN-NTN co-existence. Furthermore, for Ka band based on FR2 numerology, it could be assumed that wide bandwidths (50 MHz – 400 MHz) the SAN to UE link could be implemented as a single RF carrier. For Ku band, especially (but not limited to the case) when FR1 numerology is employed, multiple RF carriers may be activated in the same spot beam with or without CA.

The figures derived herein are intended to inform the selection of minimum requirements in addition to other considerations outside the scope of this contribution (e.g., international regulations).

# Discussion

## Method and assumptions

Figure 1: NTN-NTN Coexistence Scenario: multiple adjacent channel transmissions from a single SAN.

We consider the performance of a victim NTN cell co-existing with transmissions in the two adjacent channels as shown in Figure 1.  This represents downlink or uplink transmissions across multiple adjacent channels from a single SAN. These may, or may not, be component carriers to be aggregated (CA is not supported in Rel-19 but is expected to be introduced in future) but CA does not impact the analysis.

We model cell throughput in both ‘ideal’ cell (with no adjacent channel interference) and a ‘victim’ cell experiencing adjacent channel interference caused by adjacent channel leakage and limited adjacent channel selectivity. We fix ACS and vary ACLR and observe the impact on throughput loss - the reduction in the victim’s cell throughput compared to the ideal cell.

- Cell Throughputs are calculated as follows:

  - For each of 10 UEs in an ideal cell

    - Pick a random percentile from the range 0-100%

    - Read that UE’s Geometry SINR from a CDF. See Figure 2 (DL) and Figure 7 (UL) below.

    - Calculate that UE’s Spectral Efficiency (SE) from its SINR using the SINR to throughput mapping shown in Figure 3

    - To model UE SINR in the victim cell, we additionally model the degradation caused by ACLR and ACS as follows:

      - SINRvictim = 1 / (1/ SINRGeometry + 2/ACLR + 2/ACS)      (linear terms, not dB)

      - The 2/ACLR and 2/ACS represents two adjacent channels

    - Schedule a percentage of the cell’s bandwidth to each UE in proportion to 1/(SE^F), where F=0.5, representing a ‘fair’ scheduler.

  - Sum up the ten UE Tputs to calculate Cell Tput. The absolute channel bandwidth is common to both ideal and victim cells and thus cancels out in the calculation of relative throughput loss in percent.

- Cell throughput loss = 1 – Cell_Tput_Victim/Cell_Tput_ideal

- The 10 UEs per cell simulation is repeated 1000 times to average the result, and we observed that repeated runs gave less than 1% variation in the Cell Tput loss figure.

To derive proposed minimum requirements, we assume a 5% throughput loss per component carrier with multiple adjacent carriers (compared with the throughput of a single isolated component carrier) is acceptable.

Figure 2: DL Geometry SINR in the ideal cell (used for calculations in this contribution)

Figure 3: SINR to throughput mapping from section 5.2.7 in TR 38.803, used by other NTN coexistence studies [1]

## Downlink Results to derive SAN ACLR

SINR (dB)

Figure 4: UE DL SINR CDFs across all UEs simulated, ideal and victim with ACLR=12dB and ACS=25dB

Figure 4 shows UE SINR distributions in the ideal and victim cells for ACS=25dB and ACLR=12dB [3]. We can see how adjacent channel interference most impacts UEs with the highest SINRs, and has little impact for those with low SINR.

We can cross-check the SINR degradation by comparing to a simple analysis of the way SINR is degraded by ACLR and ACS as a function of SINR shown in figure 5, where

- Victim SINR with one adjacent channel = 1 / (1/SINR + 1/ACLR + 1/ACS)

- Victim SINR with two adjacent channels = 1 / (1/SINR + 2/ACLR + 2/ACS)

The highest geometry SINRs in the simulation are around 15dB. We see from Figure 5 that a 15dB geometry SINR is degraded in the two adjacent channel case to around 8dB, which correlates with the peak victim SINR of 8dB in Figure 4.

Figure 5:  How Adjacent Channel Leakage and Limited Selectivity degrade SINR as a function of SINR

Figure 6 shows the main result, DL Cell Throughput loss as a function of ACLR.  Here we fix VSAT ACS at the 25dB figure used for Ka band [2] and vary ACLR, which being much lower, dominates ACIR.

Figure 6: DL Cell Throughput loss as a function of ACLR, for ACS=25dB. Two adjacent channels.

Observation 1: Assuming we keep the VSAT ACS at 25dB (Ka band figure), SAN ACLR would need to be 19dB or better to ensure Cell Throughput loss remains below a 5% threshold.

Proposal 1: For SAN operating in Ku band, the ACLR value should be set at a minimum of 19 dB to ensure less than 5% capacity loss.

## Uplink results to derive UE ACLR

The methodology for uplink is the same as for downlink, but the assumed SINR distribution is changed to match the Ericsson geometry calibration result for LEO with 45x45cm antenna [1], which can be seen in Figure 7.

The victim SINR in this example is based on the Ka figures of UE ACLR of 14dB [2] and SAN ACS of 27.7dB [3].  As with the downlink, ACIR is dominated by the much lower ACLR figure, so we keep ACS at 27.7 and vary ACLR to find the 5% throughput loss threshold, shown in Figure 8

SINR (dB)

Figure 7: UE UL SINR CDFs across all UEs simulated, ideal and victim with ACLR=14dB and ACS=27.7dB

Figure 8: UL Cell Throughput loss as a function of VSAT UE ACLR, for SAN ACS=27.7 dB

Observation 2: Assuming we keep the SAN ACS at 27.7dB (Ka band figure), VSAT ACLR would need to be 18 dB or better to ensure Cell Throughput loss remains below a 5% threshold.

Proposal 2: For VSAT operating in Ku band, the ACLR should be set at a minimum of 18 dB to ensure less than 5% capacity loss.

# Conclusions

Analysis of the impact of adjacent channel interference on NTN cell throughput has provided a way to confirm that ACLR and ACS specifications strike a balance between the equipment performance, feasibility/ practicality of the requirements and spectral efficiency.

Observation 1: Assuming we keep the VSAT ACS at 25dB (Ka band figure), SAN ACLR would need to be 19dB or better to ensure Cell Throughput loss remains below a 5% threshold.

Proposal 1: For SAN operating in Ku band, the ACLR value should be set at a minimum of 19 dB to ensure less than 5% capacity loss.

Observation 2: Assuming we keep the SAN ACS at 27.7dB (Ka band figure), VSAT ACLR would need to be 18 dB or better to ensure Cell Throughput loss remains below a 5% threshold.

Proposal 2: For VSAT operating in Ku band, the ACLR should be set at a minimum of 18 dB to ensure less than 5% capacity loss.

To sum-up, if we reuse calibration results and other assumptions from Ku coexistence simulations and limit Cell throughput loss to 5%, whilst keeping ACS figures the same as for Ka band, we find that Ku requires the following:

For downlink

SAN ACLR ≥ 19dB

Assuming: VSAT ACS = 25dB, as per Ka band

For Uplink

VSAT ACLR ≥ 18dB

Assuming: SAN ACS = 27.7dB, the same as Ka band

# References

[1] R4-2508774, “Ku band co-existence simulation assumptions (running document)”, Ericsson

[2] TS 38.101-5	NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements, v19.1.0, June 2025

[3] TS 38.108 	NR; Satellite Access Node radio transmission and reception, v19.1.0, June 2025