3GPP TSG-RAN WG4 Meeting #116                                                                R4-2509260

Bengaluru, India, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on 6RX release independent issue

Agenda item:	7.1.2.3.2

Document for:	Approval

- Introduction

In RAN4 #115 meeting, a WF [1] on 6Rx and SRS IL imbalance was agreed. However, there are still some remaining issues need to be discussed. In this contribution, we focus on the 6RX release independent issue, share some views and proposals.

- Discussion

The agreement of 6RX release independent issue has been made and shown as below:

## <Sub-topic 2-1> Release independency for 6Rx UE Agreement: - Option 1: - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18 - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19 - Option 2: - Support 6Rx from Rel-19 - Option 3: - Support 6Rx from Rel-18

In principle, if new signal is introduced in new release, it should not be supported in earlier releases. Obviously, t3r6 should be introduced in release 19 although t1r6 and t2r6 is also existed.

-------------------------------------------------------------TS 38.306-----------------------------------------------------------------------

srs-AntennaSwitchingBeyond4RX-r17

Indicates whether the UE supports SRS Antenna switching for more than 4 Rx. The capability signalling comprises the following parameters:

-	supportedSRS-TxPortSwitchBeyond4Rx-r17 indicates a combination of supported xTyRs. It includes 11-bit bitmap, where starting from the leading / leftmost bit (bit 0), each bit corresponds to {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}. For any indicated value, x shall be equal to or smaller than the one associated with the largest y.

-------------------------------------------------------------TS 38.306-----------------------------------------------------------------------

In order to not further split a feature and UE capability, we propose to support 6RX from Rel-19.

Proposal 1: Support 6RX from release 19.

- Conclusion

This contribution provides our views on 6RX release independent issue. The following proposal is concluded as follows:

Proposal 1: Support 6RX from release 19.

- References

- R4-2508086, WF on 6Rx and SRS IL imbalance