3GPP TSG-RAN WG4 Meeting #116                                                                R4-2509261

Bengaluru, India, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on FWA terminology

Agenda item:	7.1.2.3.1

Document for:	Approval

- Introduction

In RAN4 #115 meeting, a WF [1] on 6Rx and SRS IL imbalance was agreed. However, there are still some remaining issues need to be discussed. In this contribution, we focus on the FWA terminology issue, share some views and proposals.

- Discussion

The agreement of 6RX FWA terminology issue has been made and shown as below:

## <Sub-topic 1-1> FWA terminology Agreement: - Remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx, and use “FWA CPE” or “FWA” for ΔRIB,6R requirement in the note of the table. - Consider adding the definition of FWA CPE or FWA, and industrial UE in the specifications - Focus on modifying the notes of the receiver requirements for 4Rx, 6Rx and 8Rx for Rel-19 specifications in RAN4#115 - FFS on whether and how to align the existing notes between 4Rx, 6Rx and 8Rx since the notes were specified according to the separate WIDs. - Further modifications will be done in the future meetings - Address the similar issues for previous releases under the maintenance agenda - Address the similar issue for transmission requirements under the maintenance agenda.

There is an agreement that remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx. It is a good start to clarify and modify the unclear description in the spec. However, we think the descriptions for the same thing need to be coordinated in the whole specification, so we also check other parts of the spec. FWA and form factor are used in many places in TS 38.101-1and in most cases, they are used together to descript the device with larger size than handheld UE, known as CPE. But CPE is only used once in Table 7.3.2-2a without any definition.

----------------------------------------------------------TS 38.101-1-----------------------------------------------------------------------

FWA	Fixed Wireless Access

----------------Next Section-----------------------

Two Rx antenna port XR UE: A non-(e)RedCap XR UE that is equipped with only two Rx antenna ports in frequency band(s) where 4 Rx antenna ports are required. The UE is intended to be worn on human head. When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

----------------Next Section-----------------------

For a terminal that supports inter-band Carrier Aggregation (CA) with UL MIMO or Tx diversity operation, the requirements are targeted for FWA form factor in current version of specification.

----------------Next Section-----------------------

[Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)] NOTE 12:	The implementation with 4 antennas is targeted for FWA form factor for this band combination.

----------------Next Section-----------------------

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n13, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n1, n3, n7, n25 | -4.5 |
| n34, n39, n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

----------------------------------------------------------TS 38.101-1-----------------------------------------------------------------------

As CPE is only used once in the specification without any definition, it is proposed to remove CPE and not introduce the definition in current and later specifications.

Proposal 1:  It is proposed to remove CPE and not introduce the definition in current and later specifications.

It is also noted that the same thing should be coordinated between different specifications in specification series. In TS 38.101-2, FWA is also used but with some differences as below, we suggest a consistent usage between different specifications because it seems these two spec readers are the same and we should avoid misunderstanding between them.

----------------------------------------------------------TS 38.101-2-----------------------------------------------------------------------

FWA UE: A UE intended to be used in fixed wireless access scenario.

----------------Next Section-----------------------

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |
| Note: Any power class can be used for Redcap type devices as long as the device can meet the core requirements that are applicable to Redcap devices as defined in clause 4.2.21.1 from TS38.306 [14] |  |

----------------------------------------------------------TS 38.101-2-----------------------------------------------------------------------

TS 38.101 series are “User Equipment (UE) radio transmission and reception”, so in this framework, UE include all kinds of user devices. So it is proposed to use “FWA UE” to substitute “FWA form factor’ to be aligned with 38.101-2.

Proposal 2: It is proposed to use “FWA UE” to substitute “FWA form factor” to be aligned with 38.101-2, this change should cover all places in TS 38.101-1 not only REFSENS chapter.

- Draft CR proposal only for REFSENS chapter

----------------------------------------------------------TS 38.101-1-----------------------------------------------------------------------

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWAUE, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n13, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA UE. NOTE 2: When 4Rx operation is supported by handheld UE. |  |

For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n1, n3, n7, n25 | -4.5 |
| n34, n39, n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA / Vehicular /Industrial UE. |  |

----------------------------------------------------------TS 38.101-1-----------------------------------------------------------------------

- Conclusion

This contribution provides our views on SRS test. The following proposals are concluded as follows:

Proposal 1:  It is proposed to remove CPE and not introduce the definition in current and later specifications.

Proposal 2: It is proposed to use “FWA UE” to substitute “FWA form factor” to be aligned with 38.101-2, this change should cover all places in TS 38.101-1 not only REFSENS chapter.

- References

- R4-2508086, WF on 6Rx and SRS IL imbalance