3GPP TSG-RAN WG4 Meeting #116	R4-2509275

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on less than 5MHz for NTN RRM performance requirements

Source: 	CATT

Agenda item:	7.6.4.5

Document for:	Discussion

- Introduction

In the last RAN4 meeting, RAN4 has reached good progress on performance requirements and the CR split work has been done [1]. In this contribution, we continue to discuss and provide our views on the performance requirements based on the agreed core requirements.

- Discussion

For NTN with less than 5MHz, RAN4 reached a lot of agreements on the test cases in the last meeting, but there were still some FFS for the test configurations and the agreements/WF are copied as below [1]:

RAN4#114-bis： Agreements/WF: The key testing configuration parameters for the agreed test cases are summarized in the table below. Companies are encouraged to input your views on the FFS parameters in the table below in the next meeting. Test case; Requirements; Configurations; Recommend WF/ Companies views. Common configuration; 15KHz, 3MHz BW, 12 PRB SSB, GSO/NGSO. HO-1:; 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover; unknown cell. Intra-frequency. Same testing procedure as NTN can be reused. Other aspects; FFS. CHO-1; 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover or 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements; With L3 measurement/ without L3 measurements; FFS. Time based. Unknown target cell. Intra-frequency. Other aspects. SatSwitch-1; 6.1C.3 NR SAN Satellite switching with re-synchronization; Hard switch. Unknown target cell. Other aspects: e.g. RACH based; FFS. RLM-1; 8.1C Radio Link Monitoring for Satellite Access; RLM OOS. FR1 PCell only. non-DRX/DRX; FFS. Other aspects; FFS. RLM-2; 8.1C Radio Link Monitoring for Satellite Access; RLM IS. FR1 PCell only. non-DRX/DRX; FFS. BFR-1; 8.5C Link Recovery Procedures for Satellite Access; SSB based BFD. FR1 PCell only. non-DRX/DRX; FFS. Other aspects; FFS. MR-1; 9.2C NR intra-frequency measurements for SAN; with SSB index detection. Non-DRX/DRX; FFS. No gap; FFS. Other aspects; FFS. MR-2; 9.3C NR inter-frequency measurements for SAN; With SSB index detection. Non-DRX/DRX; FFS. No gap; FFS. Other aspects; FFS. IDLE-1; 4.2C Cell Re-selection for NR UE for Satellite Access; FFS.

In our view, in order to avoid a large number of similar test cases, we suggest implementing different configurations in different test cases as much as possible. For the undecided test configurations, we suggest to only define the following for NTN with less than 5MHz:

•	CHO-1: with L3 measurement

•	SatSwitch-1: RACH-based

•	RLM-1: RLM OOS for non-DRX

•	RLM-2: RLM IS for DRX

•	BFR-1: BFR for non-DRX

•	MR-1: Intra-frequency without gap measurements for non-DRX

•	MR-2: Inter-frequency with gap measurements for DRX

For cell Re-selection, in legacy test cases for TN with less than 5MHz, there was a test case for cell reselection to FR1 intra-frequency NR, which defined in Clause A.6.1.1.9.

In the above test case, different values of TSI-NR (1320 ms) is assumed compared to those in legacy TN (1280 ms). TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell, which is up to RAN2 spec and is not reflected in the RAN4 core requirement.

Therefore, even though there was no new requirement for core requirements, we think that RAN4 can follow the same logic and to define a new test case for cell reselection to FR1 intra-frequency NR case in IDLE mode (IDLE-1).

Proposal 1: For the undecided test configurations, we suggest to only define the following for NTN with less than 5MHz:

•	CHO-1: with L3 measurement

•	SatSwitch-1: RACH-based

•	RLM-1: RLM OOS for non-DRX

•	RLM-2: RLM IS for DRX

•	BFR-1: BFR for non-DRX

•	MR-1: Intra-frequency without gap measurements for non-DRX

•	MR-2: Inter-frequency with gap measurements for DRX

Observation 1: In legacy, there was a test case for cell reselection to FR1 intra-frequency NR for TN with less than 5MHz, which defined in Clause A.6.1.1.9.

Proposal 2: For NTN with less than 5MHz, RAN4 to define new test case for cell reselection to FR1 intra-frequency NR case for satellite access only in IDLE mode.

- Conclusion

This document discussed the issues for RRM performance requirements for R19 NTN with less than 5MHz and presented following observation and proposals:

Proposal 1: For the undecided test configurations, we suggest to only define the following for NTN with less than 5MHz:

•	CHO-1: with L3 measurement

•	SatSwitch-1: RACH-based

•	RLM-1: RLM OOS for non-DRX

•	RLM-2: RLM IS for DRX

•	BFR-1: BFR for non-DRX

•	MR-1: Intra-frequency without gap measurements for non-DRX

•	MR-2: Inter-frequency with gap measurements for DRX

Observation 1: In legacy, there was a test case for cell reselection to FR1 intra-frequency NR for TN with less than 5MHz, which defined in Clause A.6.1.1.9.

Proposal 2: For NTN with less than 5MHz, RAN4 to define new test case for cell reselection to FR1 intra-frequency NR case for satellite access only in IDLE mode.

- Reference

[1]	R4-2508261, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi