3GPP TSG-RAN WG4 Meeting #116	R4-2509284

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on RRM performance requirements for Rel-19 NES for NR

Source: 	CATT

Agenda item:	7.23.4

Document for:	Discussion

- Introduction

In last RAN4 meeting, most of issues reached the agreements [1]. In this contribution, we discuss and provide our initial views on the performance requirements based on the agreed core requirements.

- Discussion

  - 2.1 OD-SSB SCell operation

For the test scenario, since case 1/case 2 and scenario #2/scenario #2A has been introduced in core part and different requirements have been defined for these scenarios, so RAN4 need to define the test cases for both case 1/case 2 and scenario #2/scenario #2A for OD-SSB SCell operation. For other scenarios, it depends on core part progress.

Proposal 1: RAN4 to define the test cases for both case 1 and case 2 for OD-SSB SCell operation.

Proposal 2: RAN4 to define the test cases for both scenario #2 and scenario #2A for OD-SSB SCell operation.

For OD-SSB based SCell activation, since known condition has been defined and the core requirements include known and unknown SCell cases, so RAN4 to define the related test case for known SCell and unknown SCell.

In addition, since RAN4 has agreed to define the requirements in the case of multiple OD-SSB Scells, so the test case for OD-SSB based multiple SCell activation need to be introduced.

Proposal 3: RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

In core part, RAN4 defined the T2 and discussed UE’s behaviour for re-activating the OD-SSB transmission. However, we think that the test case can mainly focus on single OD-SSB transmission, and not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Proposal 4: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Based on the above, and according to the core part requirements, we propose that at least the following test cases need to be defined:

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

- OD-SSB based SCell Activation in Case 1

- OD-SSB based SCell Activation in Case 2

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement for deactivated SCell in Case 1

- Intra-frequency measurement for deactivated SCell in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

Proposal 5: RAN4 at least to define the following test cases for OD-SSB SCell operation:

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

- OD-SSB based SCell Activation in Case 1

- OD-SSB based SCell Activation in Case 2

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement for deactivated SCell in Case 1

- Intra-frequency measurement for deactivated SCell in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

  - 2.2 SSB adaptation

According to the core part requirements, we propose that at least the following test cases need to be defined for SSB adaptation:

- L1-RSRP measureent for SSB adaptation

- L1-SINR measureent for SSB adaptation

- CBD for SSB adaptation

- L3 measurement for SSB adaptation

Proposal 6: RAN4 at least to define the following test cases for SSB adaptation:

- L1-RSRP measureent for SSB adaptation

- L1-SINR measureent for SSB adaptation

- CBD for SSB adaptation

- L3 measurement for SSB adaptation

  - OD-SIB1

In the last meeting, RAN4 has reached the following agreements for OD-SIB1 [2]:

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

Therefore, RAN4 need to define a new test case to verify the OD-SIB1 procedure in IDLE mode with longer TSI-NR.

For the test procedure, the UE need to evaluate cell reselection based on measurements towards SSB of target NES cell, and the following steps can be a reference:

- UE transmit UL-WUS transmission and using PRACH

- UE receive and decoding RAR

- UE receive OD-SIB1

Proposal 7: RAN4 need to define a new test case to verify the OD-SIB1 procedure in IDLE mode with longer TSI-NR.

Proposal 8: For following test procedure can be a reference:

- UE transmit UL-WUS transmission and using PRACH

- UE receive and decoding RAR

- UE receive OD-SIB1

- Conclusion

This document discussed the issues for Rel-19 NES performance requirements and presented following proposal:

Proposal 1: RAN4 to define the test cases for both case 1 and case 2 for OD-SSB SCell operation.

Proposal 2: RAN4 to define the test cases for both scenario #2 and scenario #2A for OD-SSB SCell operation.

Proposal 3: RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

Proposal 4: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Proposal 5: RAN4 at least to define the following test cases for OD-SSB SCell operation:

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

- OD-SSB based SCell Activation in Case 1

- OD-SSB based SCell Activation in Case 2

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement for deactivated SCell in Case 1

- Intra-frequency measurement for deactivated SCell in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

Proposal 6: RAN4 at least to define the following test cases for SSB adaptation:

- L1-RSRP measureent for SSB adaptation

- L1-SINR measureent for SSB adaptation

- CBD for SSB adaptation

- L3 measurement for SSB adaptation

Proposal 7: RAN4 need to define a new test case to verify the OD-SIB1 procedure in IDLE mode with longer TSI-NR.

Proposal 8: For following test procedure can be a reference:

- UE transmit UL-WUS transmission and using PRACH

- UE receive and decoding RAR

- UE receive OD-SIB1

- Reference

[1] 	R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#115

[2]      R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, RAN4#115