3GPP TSG-RAN WG4 Meeting #116	R4-2509296

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.27.5.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part2 was approved [2]. Most of remaining issues have been solved. This document will further discuss the RRM impact related LS from RAN2 on multiple SMTCs with different periodicities and give our understanding and proposals [1].

- Discussion

## Issue 1-0: RRM impact related LS from RAN2 on multiple SMTCs with different periodicities

In the last meeting, RAN4 discussed the LS from RAN2 on multiple SMTCs with different periodicities for downlink coverage enhancement [1], and RAN4 reached the following agreements [2]:

Agreement: (Ad-hoc) From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. - In the reply LS R4-2508433(to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference: - FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment Agreement: online From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Agreement: online The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of RAN4 RRM core part of this WI.

In our view, for the case of configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, we believe that it just has impact on configuration and there is no additional RAN4 RRM impact. The existing idle/inactive requirements have included the case of more than 4 SMTCs per frequency (i.e. 6) are configured, and the longer measurement delay is expected [3].

Therefore, we support that the UE capability of supporting multiple SMTCs in parallel will not change, and how the UE selects the SMTCs to be measured depends on the UE implementation, and there is no need to update the related RRM requirement.

Observation 1: The existing idle/inactive requirements have included the case of more than 4 SMTCs per frequency (i.e. 6) are configured, and the longer measurement delay is expected.

Proposal 1: There is no need to update the related RRM requirements.

- The UE capability of supporting multiple SMTCs in parallel will not change, and how the UE selects the SMTCs to be measured depends on the UE implementation.

- Kmulti_SMTC/K_satellite for idle/inactive/connected mode

For configuring two different SMTC periodicities (with different offsets) in one frequency layer, since the existing calculation for Kmulti_SMTC/K_satellite didn’t consider the multi SMTC periodicities scenario, so at least the calculation for Kmulti_SMTC/K_satellite for idle/inactive and connected mode neighbor cell measurements need to be discussed, especially for the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion. A simple method is to reuse the same method in RRC_IDLE mode for deriving Kmulti_SMTC/K_satellite as following [3]:

Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

Observation 2: The existing calculation for Kmulti_SMTC/K_satellite didn’t consider the multi SMTC periodicities scenario, for the case of SMTCs partially overlapping with each other, 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion.

Proposal 2: When configuring two different SMTC periodicities (with different offsets) in one frequency layer, RAN4 to reuse the legacy method for deriving Kmulti_SMTC/K_satellite in idle, inactive and connected mode, that is, two SMTCs are considered as overlapping if they overlap in one or more occasions.

- The definition of Kp/Kgap

For the definition of scaling factors Kp and Kgap, if two different SMTC periodicities in one frequency layer are configured, SMTC period in the related requirements is not clear enough. In our view, the existing method can be reused, that is, RAN4 to clarify that if different SMTC periodicities in one frequency layer are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Proposal 3: For the definition of scaling factors Kp and Kgap, RAN4 to clarify that if different SMTC periodicities in one frequency layer are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

- The definition of Trs/TSMTC

For the Handover interruption and RRC Connection Release with Redirection delay requirements, the definition of Trs is copied as follows [3]:

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover/redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

For the RRC Re-establishment delay requirements, the definition of TSMTC/TSMTC,i is as follows [3]:

- TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell. - TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

The definitions of Trs and TSMTC are related to the periodicity of the SMTC occasion. When configuring two different SMTC periodicities (with different offsets) in one frequency layer and has not been provided with an SMTC configuration for target cell, the SMTC configured in the measObjectNR is not clear enough. Also, for intra-frequency carrier and inter-frequency carrier, the periodicity of the SMTC occasion is not clear enough.

Therefore, the definition of Trs and TSMTC/TSMTC,i shall be updated, and the larger SMTC periodicity can be used in the corresponding definition.

Proposal 4: The definition of Trs and TSMTC/TSMTC,i shall be updated, and the larger SMTC periodicity can be used in the corresponding definition.

- Conclusion

This document discussed the issues about RRM impact related LS from RAN2 on multiple SMTCs with different periodicities and the following observations and proposals are presented:

Observation 1: The existing idle/inactive requirements have included the case of more than 4 SMTCs per frequency (i.e. 6) are configured, and the longer measurement delay is expected.

Proposal 1: There is no need to update the related RRM requirements.

- The UE capability of supporting multiple SMTCs in parallel will not change, and how the UE selects the SMTCs to be measured depends on the UE implementation.

Observation 2: The existing calculation for Kmulti_SMTC/K_satellite didn’t consider the multi SMTC periodicities scenario, for the case of SMTCs partially overlapping with each other, 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion.

Proposal 2: When configuring two different SMTC periodicities (with different offsets) in one frequency layer, RAN4 to reuse the legacy method for deriving Kmulti_SMTC/K_satellite in idle, inactive and connected mode, that is, two SMTCs are considered as overlapping if they overlap in one or more occasions.

Proposal 3: For the definition of scaling factors Kp and Kgap, RAN4 to clarify that if different SMTC periodicities in one frequency layer are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Proposal 4: The definition of Trs and TSMTC/TSMTC,i shall be updated, and the larger SMTC periodicity can be used in the corresponding definition.

- Reference

- R2-2503068, LS on SMTC enhancements, RAN2

- R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- 3GPP TS 38.133, NR Requirements for support of radio resource management