3GPP RAN WG4 Meeting #116	R4-2509301

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on testing issues for CSI compression two-sided models

Source: 	CATT

Agenda item:	7.18.3

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 discussed testability and interoperability issues for two-sided model in Rel-19 AI/ML for NR air interface and agreements were captured in [1]. But some issues need further investigating and discussing. In this contribution, we will continue to discuss some open issues and present our views [2].

Discussion

Simulation results and observations

According to the simulation plan for Option 3 captured in the way forward [1], more simulation results are expected to examine the performance of low-complexity encoders with different backbones, i.e., transformer, CNN and MLP, and also to verify whether it is feasible to achieve a common decoder, which is compatible with encoders having different backbones and complexity levels. The observations and conclusions obtained based on this simulation campaign can make the feasibility study more substantial. Detailed simulation assumptions are provided in [3] and the assumed encoder structures for different backbones are used in the simulations. The simulation results are provided in Table 1.

Table 1: Simulation results for AI based CSI compression with low-complexity encoders having different backbones

| Option 3 low-complexity encoders with different backbones |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | （1-to-1）Joint training | New encoder training with (Samsung’s) frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| Encoder-1 | Companies' name | SGCS-0 | SGCS-1 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CATT | 0.6927 | 0.7106 | 0.7017 | 0.6978 |
| Encoder-2 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CATT | 0.6999 | 0.6756 | 0.6932 | 0.6915 |
| Encoder-3 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CATT | 0.6831 | 0.697 | 0.6785 | 0.6773 |
| Encoder-4 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CATT | 0.6344 | 0.6346 | 0.6783 | 0.6769 |

Observation 1: In 1-to1 joint training and separate encoder training with fixed decoder, low complexity encoders with CNN/transformer backbone have comparable performance to that of Encoder 1. MLP encoder has the lowest performance and a 5% SGCS degradation compared to others.

Observation 2: In N-to-1 joint training, low complexity encoders with CNN/transformer/MLP backbone have comparable performance to that of Encoder 1, where the SGCS spans around 2.5%.

Observation 3: N-to-1 joint training provides performance generalization and robustness when different backbones/structures are used for encoders, compared with 1-to-1 training.

Conclusions

This contribution discussed some testability and interoperability issues for AI/ML enabled CSI compression and some observations and proposals are provided:

Observation 1: In 1-to1 joint training and separate encoder training with fixed decoder, low complexity encoders with CNN/transformer backbone have comparable performance to that of Encoder 1. MLP encoder has the lowest performance and a 5% SGCS degradation compared to others.

Observation 2: In N-to-1 joint training, low complexity encoders with CNN/transformer/MLP backbone have comparable performance to that of Encoder 1, where the SGCS spans around 2.5%.

Observation 3: N-to-1 joint training provides performance generalization and robustness when different backbones/structures are used for encoders, compared with 1-to-1 training.

References

[1] R4-2508051, WF on AI/ML part 2, Ericsson, RAN4#115.

[2] R4-2507571, Topic summary for [115][132] NR_AIML_air_part2, Ericsson, RAN4#115.

[3] R4-2508084, Simulation assumptions on AI/ML CSI compression, Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, RAN4#115.