3GPP RAN WG4 Meeting #116	R4-2509306

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on UE to UE CLI handling

Source: 	CATT

Agenda item:	7.21.4.1

Document for:	Discussion

Introduction

In RAN4#115meeting, RAN4 made fruitful progress of the UE to UE CLI requirements in R19 SBFD WI. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss open issues [2] that are related to UE-to-UE CLI handling.

Discussion

Requirement applicability

RAN4#115 discussed the requirement applicability of L1 CLI measurement and the following agreement was achieved:

- RAN4#115: - Issue 1-1-1: Requirement applicability - Agreement (Monday online session) - For method #2, 3 and non-basic UE operation of method #1 - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is configured within active DL BWP - when CLI-RSSI measurement resource is configured within active DL BWP - FFS for basic UE operation for method #1 - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

One general open issue is whether the basic UE operation exists or not. According to the component 2 in the feature group index 60-1 and 60-8 defined in RAN1 UE feature list [3], it is differentiated that whether UE can support reception of CLI-RSSI measurement resource in one UL subband only, in one DL subband only, or across two DL subbands only. And UE can also report whether it is capable of DL reception in two DL subbands in SBFD symbols. Hence, with these reported UE capabilities, UE can clearly declare its DL reception capability and for the UE capable of receiving two DL subbands, they are expected to measure L1-CLI-RSSI measurement resource in two DL subbands simultaneously when the CLI resource is configured across two DL subbands.

We also submitted a companion draft CR to update the L1-CLI-RSSI requirements to reflect the impact of the above UE capability on measurement requirements [4].

RAN1 UE feature list after RAN1#121 meeting: 60. NR_duplex_evo; 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG). ……. 60. NR_duplex_evo; 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS.

Proposal 1: 	RAN4 to confirm that UE can declare its DL reception capability that whether it is capable of receiving two DL subbands simultaneously or not and accordingly update the L1-CLI-RSSI requirements.

L1 CLI measurement in non-SBFD symbols

In RAN4#115 meeting, one proposal was brought up to consider whether a UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation.

RAN4#115: - Proposals - Proposal 1: - RAN4 to wait, and potentially take into consideration, RAN1 decision on whether a UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation. - Recommended WF - Suggest to wait for RAN1 conclusion.

In RAN1#121 meeting, RAN1 made the following agreement and the contents related to the above issue is highlighted in green:

RAN1#121 Agreement: For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

According to RAN1 agreement, when SBFD symbols are not configured for a UE, the UE still need to perform L1 UE-to-UE CLI measurement on non-SBFD symbols based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig. Therefore L1 UE-to-UE CLI measurements on non-SBFD symbols are supported.

Proposal 2: It should be confirmed that 	L1 UE-to-UE CLI measurements on non-SBFD symbols are supported.

Measurement restriction

Collision between L1-SRS-RSRP (Method#2) and L1-CLI-RSSI (Method#1) collision

After some discussions in RAN4#115 meeting, RAN4 thinks that more discussion is still needed when L1-SRS-RSRP and L1-CLI-RSSI measurements collide:

RAN4#115 Agreements: #### Issue 1-2-3: Measurement restriction - Agreement - Take into account following RAN1 agreement in RAN1#121, and further discuss the measurement restriction between Method #1 and Method #2.

Considering that the purpose of CLI measurements is to evaluate the interference level to victim UE in DL subband(s), i.e., network cares the channel quality of DL subbands of victim UE. Therefore, when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide, UE shall prioritize L1-CLI-RSSI measurement.

Proposal 3: UE shall prioritize L1-CLI-RSSI measurement when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide.

Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements

In RAN4#114 meeting, RAN4 achieved the following agreement for the collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements:

- Agreements - Collision between L1-CLI-RSSI measurements with different methods - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

In RAN4#115, companies’ proposals are reproduced below:

Proposals in RAN4#115: - Proposals for collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1 - Proposal 1a (CATT, ZTE) - Measurement requirements for L1-CLI-RSSI apply as default. - Proposal 1b (Samsung) - Measurement requirements for L1-CLI-RSSI apply, when SCS of L1-CLI-RSSI measurement and L1/L3 RRM measurements are same. - Proposal 1c (E////) - Measurement requirements for L1-CLI-RSSI apply, except when L1-CLI-RSSI measurement resources overlap MG - Proposal 1d (HW) - Measurement requirements for L1-CLI-RSSI apply, except when - L1-CLI-RSSI measurement resources overlap MG - L1-CLI-RSSI measurement resources are configured with a different SCS from the SSB used for L1/L3 measurement - Proposal 2 (Nokia) - Measurement requirements for L1-CLI-RSSI do not apply. - Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements in FR1 - Discuss the following 2 options on high level: - Option 1: Measurement requirements for L1-CLI-RSSI apply as default, and define exception cases where requirements do not apply. - Option 2: Measurement requirements for L1-CLI-RSSI do not apply.

In FR1, with the assumption that omnidirectional Rx antenna is used by UE to perform DL receptions, we think UE can perform both L1-CLI-RSSI and L1/L3 RRM measurements simultaneously when the corresponding resources overlap in time domain. However, there are some exceptions brought up by companies, such as L1 CLI measurement resource overlapping with measurement gaps, and different SCS configurations between L1 CLI measurement resource and SSB for L1/L3 RRM measurement. For these exceptions, we can agree L1 CLI measurement requirement do not apply.

Proposal 4: In FR1, L1 CLI measurement requirements apply when L1 CLI measurements and L1/L3 RRM measurements overlaps in time domain, except

      - L1-CLI-RSSI measurement resources overlap MG.

      - L1-CLI-RSSI measurement resources are configured with a different SCS from the SSB used for L1/L3 measurement.

Conclusions

This contribution discussed the impacts of L1 UE-to-UE CLI on RRM requirements and some proposals are provided:

Proposal 1: 	RAN4 to confirm that UE can declare its DL reception capability that whether it is capable of receiving two DL subbands simultaneously or not and accordingly update the L1-CLI-RSSI requirements.

Proposal 2: It should be confirmed that 	L1 UE-to-UE CLI measurements on non-SBFD symbols are supported.

Proposal 3: UE shall prioritize L1-CLI-RSSI measurement when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide.

Proposal 4: In FR1, L1 CLI measurement requirements apply when L1 CLI measurements and L1/L3 RRM measurements overlaps in time domain, except

      - L1-CLI-RSSI measurement resources overlap MG.

      - L1-CLI-RSSI measurement resources are configured with a different SCS from the SSB used for L1/L3 measurement.

References

[1] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#115.

[2] R4-2505568, Topic summary for [115][219] NR_duplex_evo, Moderator (Huawei, HiSilicon), RAN4#115.

[3] R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121, RAN1#121.

[4] R4-2509305, DraftCR to update L1-CLI-RSSI measurement requirements, CATT, RAN4#116.