3GPP RAN WG4 Meeting #116	R4-2509307

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on impacts of SBFD on RRM requirements

Source: 	CATT

Agenda item:	7.21.4.2

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 continued discussing the RRM requirements of R19 SBFD feature. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss the impacts of SBFD on RRM requirements.

Discussion

CSI-RS L1 measurement – puncturing in frequency domain

On this issue, the following agreement was achieved in last meeting:

RAN4#115 Agreements: - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability. Proposals for impact on measurement period due to processing time - Proposal 1 (vivo, OPPO, Samsung, CMCC, E///, HW, MTK): - Add a lower bound on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - 8ms (vivo, CMCC, E///, HW, MTK) - 10ms (OPPO) - Proposal 2 (CATT): - Existing measurement period for apply when the periodicity of CSI-RS is large enough to cover scaled processing time. Otherwise, a longer measurement delay can be expected. - Proposal 3 (Nokia): - For Case 3, if the accuracy requirements is adopted dependent on the X4 = 24 PRBs, the existing measurement delay shall apply i.e. lower bound is not needed. This applies when UE does not support simultaneous DL reception in 2 DL subbands. - For Case 3, if UE supports simultaneous DL reception in 2 DL subbands, a lower bound can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

One open issue is to determine the upper bound of the measurement period due to the scaled processing time.

In our understanding, in most of cases, the periodicity of CSI-RS resource is large enough to cover the scaled CSI processing time and therefore the existing delay requirements can still apply. However, according to the major view in last meeting, most of companies supported to add an extra time, i.e., 8 ms, to the existing measurement period, which allows UE to have more time to process the CSI-RS resource across two DL subbands. Hence, we can also compromise to have this extra processing time duration, i.e., 8 ms.

Proposal 1: For DUD case, add 8 ms to the existing measurement period for CSI-RS based L1 measurement and the requirements apply when UE indicates the CSI processing time is scaled by 2.

Conclusions

This contribution discussed the impacts of SBFD on RRM requirements and some proposals are provided:

Proposal 1: For DUD case, add 8 ms to the existing measurement period for CSI-RS based L1 measurement and the requirements apply when UE indicates the CSI processing time is scaled by 2.

References

[1] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#115.