3GPP TSG-RAN WG4 Meeting # 116	R4-2509309

# Bengaluru, India, 25 August - 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to update the applicability requirements for UE supporting L3 fast beam sweeping operation in FR2-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Update the applicability requirements for UE supporting L3 fast beam sweeping operation in FR2-1. - Revise the solution 1 in clause 9.2.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the applicability requirements by including the triggering conditions, i.e., RSRP and/or RSRQ threshold of serving cell. - Revise the solution 1 in clause 9.2.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability requirements for UE supporting L3 fast beam sweeping operation in FR2-1 are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2, New clause: 3.6.20. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | N/A |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | N/A |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 3.6.20	Applicability of requirements for UE supporting L3 fast beam sweeping operation in FR2-1

The requirements related to the support of [Rel-19 reduced L3 beam sweeping capability] are applicable when

-	the carrier to be measured is the only carrier in the FR2-1 band, which is configured for L3 SSB measurement, and

-	UE is not configured with CA or DC.

-	UE is configured with triggering conditions, i.e., RSRQ and/or RSRQ threshold(s) of the serving cell indicated by fbs-ThresholdP-r19 and/or fbs-ThresholdQ-r19, and the triggering conditions are satisfied.

<End of Change 1>

<Start of Change 2>

9.2.3.2	Requirements for FR2

For UE supporting [CSSF enhancement for one serving CC measurement per-band], if UE receives network indication via [TBD] to enable one serving carrier measurement for each FR2 band,

-	For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

- 	PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in this FR2 band; or

-	PSCC, when UE is configured with EN-DC with PSCC in this FR2 band; or

-	PSCC, when UE is configured with NR-DC with PSCC in this FR2 band; or

-	SCC on which UE is configured to report SSB based measurements, when neither PCC nor PSCC is in the same band and this SCC is the only SCC on which UE is configured to report SSB based measurements in this FR2 band;

-	otherwise

-	SCC determined by UE implementation.

-	UE is not required to perform SS-RSRP, SS-RSRQ, and SS-SINR measurements on serving cell for each of the other intra-frequency layer(s) in the same band.

For UE not supporting [CSSF enhancement for one serving CC measurement per-band], or UE supporting [CSSF enhancement for one serving CC measurement per-band] but does not receive network indication via [TBD] to enable one serving carrier measurement for each FR2 band,

-	For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

-	where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

-	The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

<End of Change 2>