3GPP RAN WG4 Meeting #116	R4-2509315

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on the impacts of low band CA via switching on RRM core requirements

Source: 	CATT

Agenda item:	7.3.4

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 discussed the impacts of low-low band carrier aggregation via switching on RRM core requirements. The achieved agreements are captured in [1]. In this contribution, we will further present our views on the open issues summarized in [2].

Discussion

Necessity of interruption requirement in RRM when more carriers are considered

Some proposals were brought up to consider more carriers/bands in Rel-19 and they are copied here for reference:

RAN4#115 Proposals: - Option 1 (QC): RAN4 does not define interruption requirements to other bands/carriers in Rel-19. - Option 2 (HW): When more carriers are considered, switching between FDD and SDL would interrupt other combined active SCell, the interruption length is ⌈T_switching/(Slot-length-victim)⌉+1. - Option 3 (Nokia): If a third band is configured without switching pattern, no interruption is allowed on the third band due to LBCA-sw operation on PCell and SDL SCell. - Option 4 (MTK): RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

In the summary, an agreement made in RF group in RAN4#114 was provided:

RF agreements in RAN4#114: Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

According to the highlighted contents, more carriers/bands can be considered in the future release and therefore we support to close the issue and focus on other core and performance requirements.

Proposal 1: RAN4 RRM to close the issue of RRM requirements when more carriers are considered.

Requirement design of activated SDL SCell L3 measurement

In RAN4#115, it was agreed to define requirements for activated SDL SCell measurement:

Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

There are two open issues that need further discussion. Regarding the 1st issue, we generally agree with the proposal since there is no SSB occasion of the SDL SCell is contained within the SDL SCell ON period of the LB-CA switching pattern and therefore UE cannot perform SSB-based measurements on SDL SCell. But we think the wording can be more precise and our proposal is provided as follows:

Proposal 2: 	If no any SSB occasion transmitted from the SDL SCell is contained within SDL SCell ON period of the LB-CA switching pattern, the requirements do not apply.

For 2nd issue, there are some common methods that can be adopted to define requirements for activated SDL SCell measurements, such as using new Kp factor, max{DRX cycle, SMTC, pattern periodicity P}, and CSSF as mentioned in the summary. Actually, any of them can achieve the purpose of reflecting the impacts of LB-CA switching pattern in measurement requirements. In our understanding, using max{DRX cycle, SMTC, pattern periodicity P} is a clearer and more straightforward way. Hence, we prefer to use max{DRX cycle, SMTC, pattern periodicity P} method.

Proposal 3: RAN4 to use max{DRX cycle, SMTC, pattern periodicity P} method to reflect the impacts of LB-CA switching pattern in the activated SDL SCell measurement requirements.

Scheduling restriction during SDL SCell activation/deactivation

Another issue is whether and how to define scheduling restriction during SDL SCell activation procedure and a compromised option was provided based on companies’ proposals:

- Recommended WF - Check if following moderator proposal for compromise(considering P1, P2, P3, P4(option 2), P5) is agreeable: - Proposal 6(Moderator): - RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure. - The scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation. - the scheduling restriction length for each CC switching in SDL SCell activation can be defined as {(symbol(s) for switching duration of FDD-to-SDL) + SSB/SMTC duration + (symbol(s) for switching time of FDD-to-SDL)}.

During SDL SCell activation procedure, UE will switch to SDL SCell to receive RS occasions and when UE finishes the receptions it will switch back to PCell for DL reception and/or UL transmission. Apparently, SCell activation procedure has impacts only on the symbols where RS occasions are transmitted and CC switching occurs. Since NW knows the temporal locations of RS occasions of SCell and CC switching, NW can avoid scheduling this UE for DL reception and/or UL transmission on PCell. Therefore we think scheduling restriction needs to be defined to avoid collisions between PCell data transmission between SCell activation procedures.

Regarding the specific scheduling restriction time duration on PCell, the recommended solution in the summary can be illustrated as follows:

Figure 1 Illustration of scheduling restriction on PCell during SDL SCell activation procedure

Based on Figure 1, we support to define scheduling restriction time duration in a symbol level which is derived by the equation:

Scheduling restriction = (symbol(s) for switching duration of FDD-to-SDL) + SSB/SMTC duration + (symbol(s) for switching time of FDD-to-SDL).

Proposal 4: RAN4 to define the scheduling restriction on PCell caused by the measurements during SDL SCell activation procedure, and the scheduling restriction length is derived by:

- (symbol(s) for switching duration of FDD-to-SDL) + SSB/SMTC duration + (symbol(s) for switching time of FDD-to-SDL).

BFD/CBD requirements for SDL SCell

Regarding the BFD/CBD requirements for SDL SCell in LB CA via switching, RAN4 have the following agreement:

RAN4#115 Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

RAN4 agreed that the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. A simple way to define requirement is to count the number of SSB/CSI-RS occasions that are not covered by SDL duration in switch pattern, e.g., X. And the number of measurement sample in the corresponding legacy requirements can be updated by the adding X.

Proposal 5: The BFD/CBD measurement requirement for SDL SCell in LB CA via switching can be defined based on the legacy requirement with the number of measurement sample being updated. The new measurement sample equals to the legacy value plus X, where X is the number of SSB/CSI-RS occasions that are not covered by SDL duration in switch pattern.

Similarly, the requirements for L1-RSRP/SINR measurement requirement for SDL SCell can be defined using the same approach since RAN4 made a similar agreement as follows:

RAN4#115 Agreement: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Proposal 6: RAN4 to use the approach in proposal 5 to define L1-RSRP/SINR measurement requirement for SDL SCell.

Conclusions

This contribution discussed the impacts of LBCA on RRM core requirements and some proposals are provided:

Proposal 1: RAN4 RRM to close the issue of RRM requirements when more carriers are considered.

Proposal 2: 	If no any SSB occasion transmitted from the SDL SCell is contained within SDL SCell ON period of the LB-CA switching pattern, the requirements do not apply.

Proposal 3: RAN4 to use max{DRX cycle, SMTC, pattern periodicity P} method to reflect the impacts of LB-CA switching pattern in the activated SDL SCell measurement requirements.

Proposal 4: RAN4 to define the scheduling restriction on PCell caused by the measurements during SDL SCell activation procedure, and the scheduling restriction length is derived by:

- (symbol(s) for switching duration of FDD-to-SDL) + SSB/SMTC duration + (symbol(s) for switching time of FDD-to-SDL).

Proposal 5: The BFD/CBD measurement requirement for SDL SCell in LB CA via switching can be defined based on the legacy requirement with the number of measurement sample being updated. The new measurement sample equals to the legacy value plus X, where X is the number of SSB/CSI-RS occasions that are not covered by SDL duration in switch pattern.

Proposal 6: RAN4 to use the approach in proposal 5 to define L1-RSRP/SINR measurement requirement for SDL SCell.

References

[1] R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4#115.

[2] R4-2505556, Topic summary for [115][207] NR_LBCA_Sw, Apple, RAN4#115.