3GPP TSG-RAN WG4 Meeting #116				                                                R4-2509318

Bengaluru, India, August 25th – August 29th, 2025

Title:	Discussion on the impacts of A-IoT on RRM core requirements

Source:	CATT

Agenda Item:	7.22.4

Document for:	Discussion

1 Introduction

In RAN4#115 meeting, RAN4 discussed the RRM impacts of ambient IoT. The agreements are captured in the WF [1]. In this contribution, we will further analyse the impacts of A-IoT device 1 deployed in scenario 1 with topology 1 (D1T1-B) on RRM requirements.

2 Discussion

  - UL Timing requirements

In RAN4#115 meeting, UL transmit timing requirements were discussed and the agreements were achieved:

RAN4#115 Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

RAN4 agreed that the value of Te is equal to 10%*TSFO and this principle is used as a starting point for MSG1 X=2. In RAN1#120bis and RAN1#121 meetings, RAN1 made the following agreements that are related to UL timing [2][3]:

RAN1#120bis Agreements: Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. RAN1#121 Agreements: Agreement For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, from the device perspective: - Toffset1 is from a set of predefined values. - FFS: the predefined values - E.g. values within the range [2666.8, 1333.4, 666.6, 333.4, 222.2, 166.6, 133.34, 111.2, 83.4, 55.6, 44.44, 41.6, 30] us - FFS: which values of Toffset1 correspond to which values of the following factors (to be selected with future agreement): - R2D chip length, D2R chip length, padding length, whether FEC is applied Agreement Toffset2=αToffset1+βTmsg1, where - Tmsg1 is the duration of the 1st Msg1 time domain resource - α=0.25 and β=1.25 Agreement Define Toffset3, which is the time interval from the end of a R2D transmission for Msg2 to the starting time of the corresponding Msg3 time domain resource, from the device perspective. Agreement Define Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission, from the device perspective.

RAN1 introduced two new time offsets, Toffset3 and Toffset4 for Msg3 and other D2R transmissions. Since the definitions are quite similar to that of Toffset1, we suggest apply the D2R transmit timing requirements to Toffset3 and Toffset4 as well.

Proposal 1: The defined D2R transmit timing requirements also apply to Toffset3 and Toffset4.

3 Conclusion

In this paper, we discussed the impacts of A-IoT on RRM requirements and the following observations and proposals are given:

Proposal 1: The defined D2R transmit timing requirements also apply to Toffset3 and Toffset4.

- References

- R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC, RAN4#114bis.

- Chair notes RAN1#120bis eom0, RAN1#120bis.

- Chair notes RAN1#121 eom0, RAN1#121.

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, V19.0.0.