3GPP TSG-RAN WG4 Meeting # 116                                                              R4-2509323

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on potentially new requirements for SBFD operation including BS conformance testing

Source: 	CATT

Agenda item:	7.21.3.1

Document for:	Discussion

- Background

WF [1] of RAN4#115 concluded that further studies are needed for potentially new RF requirements and conformance testing, in this contribution, we provide our further analysis of them.

- New requirements for SBFD operation

In WF [1], some in-channel new requirements need further analysis.

2.1 In-channel adjacent sub-band leakage requirement

##### Issue 4-2-1: Assumption for in-channel adjacent subband leakage requirement

- No agreement in this meeting and the way forward from RAN4#114bis can still be followed.

##### Issue 4-2-2: Absolute limit for in-channel adjacent subband leakage

Agreement

- For absolute limit for in-channel adjacent subband leakage:

  - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed.

  - For FR1 BS,

    - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed.

    - For FR1 WA and MR SBFD-capable BS,

      - Option 1: [3]dB relaxation is provided.

      - Option 2: no relaxation from corresponding existing requirement.

For {DU/UD} and {DUD} configurations, the value difference of the nonlinear products falling into the UL subband is relatively small, so the interference signal can be analyzed according to the same ACLR. The ACLR value can be decided based on feasibility analysis and simulations in SI stage. According to the required ACLR in SI stage for FR1 WA and MR SBFD-capable BSs, the absolute limits for in-channel adjacent subband leakage specified in the table 6.6.3.2-2 in TS 38.104 for FR1 BS can be reused.

Proposal 1: The absolute limits for in-channel adjacent subband leakage of FR1 WA and MR SBFD-capable BSs can reuse the values specified for FR1 NR BS.

2.2 In-channel adjacent subband blocking requirement

Issue 4-4-1: In-channel adjacent subband blocking requirement

Way forward:

- New requirement for in-channel adjacent subband blocking:

  - The requirement can be derived by considering co-site inter-sector case and/or inter-site case

    - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification.

  - The range of [-35dBm to -27dBm] can be used as the starting point for further study.

Issue 4-4-2: Assumption of gNB-to-gNB CLI handling

Way forward:

- FFS the assumption of gNB-to-gNB CLI handling for two cases:

  - Inter-site case

    - Note: for the blocking requirement, RX digital beamforming cannot be considered.

  - Co-site inter-sector case:

    - Note: Inter-sector is near field interaction.

The in-channel adjacent subband blocking requirement needs to consider co-site inter-sector case and inter-site case.

The in-channel adjacent sub-band blocking requirement should be defined based on simulation result, -32dBm can be used for MR BS and -34dBm can be used for LA BS.

Proposal 2: For in-channel adjacent sub-band blocking requirement, -32dBm can be used for MR BS and

-34dBm can be used for LA BS.

When consider interference from inter-sectors BSs, the isolation between two sectors need to be decided, but companies had different recommendations on the value of the sector isolation, and it may be difficult to reach a consensus. We can assume an isolation value between two sectors, and then analyse the in-channel adjacent sub-band blocking requirement based on the isolation assumption. The isolation assumption can be added as a standard note.

Proposal 3: 	If co-site inter-sector case is considered, the isolation assumption between two sectors BSs should be added as a standard note.

- Conformance testing for potentially new RF requirements

3.1 Additional manufacture declarations and updates

Following requirements are newly introduced, and need to be added to both TS 38.141-1 and TS 38.141-2.

•Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

Proposal 4: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2.

•	Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

3.2 Additional test models for SBFD capable BS

Transmitter output power in UL subband/DL subband(s)

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 1: Transient period between SBFD and non-SBFD

For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD, as shown in figure 1.

Proposal 5: For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

For SBFD capable BS, the desired test model and frequency location of wanted signal for in-channel adjacent subband leakage requirement testing are the same as those used for BS ACLR absolute basic limit test, but with different test frequency ranges. These contents need to be added to TS 38.141-1 and TS 38.141- 2

Proposal 6: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 8: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

For SBFD capable BS, the desired Rx input signal and its frequency location for in-channel adjacent subband blocking testing are the same as those used for in-band blocking requirement. When test this requirement, the transmitters must be turned on and the test frequency range is different from in-band blocking. These contents need to be added to TS 38.141-1 and TS 38.141- 2

Proposal 9: For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

Proposal 10: For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

Proposal 11: For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

- Summary

In this contribution, we have the following proposals.

Proposal 1: The absolute limits for in-channel adjacent subband leakage of FR1 WA and MR SBFD-capable BSs can reuse the values specified for FR1 NR BS.

Proposal 2: For in-channel adjacent sub-band blocking requirement, -32dBm can be used for MR BS and -34dBm can be used for LA BS.

Proposal 3: 	If co-site inter-sector case is considered, the isolation assumption between two sectors BSs should be added as a standard note.

Proposal 4: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2.

•	Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

Proposal 5: For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

Proposal 6: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 8: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 9: For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

Proposal 10: For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

Proposal 11: For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

Reference

[1] R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, Samsung, RAN4#115