3GPP TSG-RAN WG4 Meeting # 116                                                              R4-2509327

Bengaluru, India, August 25th – 29th, 2025

Title: 	                 Discussion on A-IoT BS requirements

Source: 	CATT

Agenda item:	7.22.3.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for A-IoT BS Tx and Rx requirements according to the WF [1].

- A-IoT BS RF requirements analysis

2.1 Tx requirements

2.3.1 Transmit signal quality

In WF[1], transmit signal quality need further analysis.

### Issue 1-4-2: Modulation quality

- Agreement:

  - Consider following options

    - Option 1: Take the table below as a starting point.

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tari= 103M*15 (us) M = 2, 6, 12 and 24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | [0.5~0.15] (A–B) | V/m orA/m |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
|  | RF Pulsewidth | PW |  | 0.5Tari | [0.65/0.525] Tari | µs |

      - Note 1: The ripple is calculated based on per-chip, comparing peak value of each chip to its own average.

      - Note 2: SIP is excluded for ripple calculation.

    - Option 2: No need to define RF envelop parameters.

R2D signal is ook-4 modulation based on OFDM, and we can define envelope requirements similar to RFID, including modulation depth, RF envelope ripple, RF envelope rise time, RF envelope fall time and RF pulse. It is necessary to consider the differences between R2D signal and RFID signal, including modulation method, encoding method and waveforms, etc.

Proposal 1: RF envelope requirements should be defined for R2D signal.

2.3.2 Unwanted emissions

2.3.2.1 ACLR

Based on the simulation results of scenario 2-1, the ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB. For 1PRB and multiple PRB transmission BW, if the frequency offset is located in the NR spectrum, the frequency offset can be defined. In this case, the frequency offset has no impact on the A-IoT channel bandwidth. For 1PRB and multiple PRB transmission BW, the frequency offset can be configured as 100kHz, with ACLR1 is 40dB and ACLR2 is 50dB, as shown in figure 1 and table 2.

Figure 1 ACLR for NB-IoT standalone

Table 2: R2D frequency and ACLR

| R2D CBW | offset | ACLR |
| --- | --- | --- |
| 200 kHz | 100kHz | 40/50dB |
| 400 kHz | 100kHz | 40/50dB |
| 600 kHz | 100kHz | 40/50dB |
| 800 kHz | 100kHz | 40/50dB |

Proposal 2: For both 1PRB and multiple PRB transmission BW, the frequency offset can be configured as 100kHz, with 40dB for ACLR1 and 50dB for ACLR2.

2.3.2.2 OBUE

The channel bandwidth of standalone NB-IoT BS is 200kHz, and it has guardbands at the low and up ends inside the channel, transmission bandwidth configuration of it is 180kHz, so OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS with both 1PRB and multiple PRB configurations.

Proposal 3: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS with both 1PRB and multiple PRB configurations.

2.4 Rx requirements

2.4.1 ACS

Table 7.5.1-6c: Adjacent channel selectivity for NB-IoT standalone Medium Range BS

| NB-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB (Note) | -47 | ±100 | 180 kHz NB-IoT signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.1-5c. |  |  |  |  |

Based on the simulation results of scenario 2-1, i.e. D1T1-B, NR UEs only distributed outdoor, R2D is located in DL spectrum and CW/D2R are located in UL spectrum, the ACS for NB-IoT standalone Medium Range BS  can be reused for A-IoT BS. The interfering signal centre frequency offset to the lower/upper BS RF Bandwidth edge is ±100kHz, and there is no additional frequency offset of ±100kHz. Therefore, additional frequency offset of ±100kHz is also not required for A-IoT BS.

Proposal 4: The ACS for NB-IoT standalone MR BS can be reused for A-IoT BS.

Proposal 5: For ACS requirement, additional frequency offset of ±100kHz is not required for A-IoT BS.

2.4.2 In-band blocking

In scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, but CW nodes are distributed indoor and the input CW interference signal level at A-IoT BS receiver needs to be considered, so the narrow-band blocking requirement can be defined, and the narrowband blocking requirement for NB-IoT standalone MR BS can be reused for A-IoT BS.

Proposal 6: The narrowband blocking requirement for NB-IoT standalone MR BS can be reused for A-IoT BS.

2.4.3 Receiver intermodulation

The scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended. If A-IoT BS and co-located CW node can’t run at the same time, in scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, and the CW interference signal has been considered by narrowband blocking, so receiver intermodulation and narrowband intermodulation requirements are not necessary. Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the interference signal can be considered by blocking requirement.

Observation 1: Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the interference signal can be considered by blocking requirement.

Proposal 7: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 8: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

- Summary

This contribution provides our further analysis for A-IoT BS requirements. We have the following proposals and observations.

Proposal 1: RF envelope requirements should be defined for R2D signal.

Proposal 2: For both 1PRB and multiple PRB transmission BW, the frequency offset can be configured as 100kHz, with 40dB for ACLR1 and 50dB for ACLR2.

Proposal 3: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS with both 1PRB and multiple PRB configurations.

Proposal 4: The ACS for NB-IoT standalone MR BS can be reused for A-IoT BS.

Proposal 5: For ACS requirement, additional frequency offset of ±100kHz is not required for A-IoT BS.

Proposal 6: The narrowband blocking requirement for NB-IoT standalone MR BS can be reused for A-IoT BS.

Observation 1: Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the interference signal can be considered by blocking requirement.

Proposal 7: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 8: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

Reference

[1] R4-2508101, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#115

[2] RP-243326, “Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, RAN#106