3GPP TSG-RAN WG4 Meeting # 116                                                              R4-2509331

# Bengaluru, India, August 25th – 29th, 2025

Title: 	                 draft TP for TS 38.194 to introduce transmitter intermodulation

Source: 	CATT

Agenda item:	7.22.3.1

Document for:	Approval

- Introduction

This contribution provides a draft TP to TS 38.194 for A-IoT BS Tx requirements. The agreements of Tx requirements in the WF [2] are included in the TP.

Reference

[1] TS 38.194, V0.1.0

[2] R4-2508101, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#115

TP for TS 38.194

## 6.7	Transmitter intermodulation

### 6.7.1	General

The transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the antenna. The requirement shall apply during the transmitter ON period and the transmitter transient period.

For A-IoT BS type 1-C, the transmitter intermodulation level is the power of the intermodulation products when an interfering signal is injected into the antenna connector.

For A-IoT BS type 1-C, the co-location transmitter intermodulation requirement is not needed.