3GPP TSG-RAN WG4 Meeting #116                                                                                                      R4-2509335

Bengaluru, India 25th –29th August, 2025

Source: 	CATT

Title: 	Further discussion on coexistence evaluations for Ku-band for NR NTN

Agenda item:	7.7.3

Document for:	Approval

- Introduction

In RAN4# 114bis meeting, a Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence was agreed.

In RAN4#115 meeting, the running document on Ku band co-existence simulation assumptions (running document) is noted.

In this contribution, we provide our views on some coexistence evaluation for Ku-band for NR NTN.

- Discussion

Equivalent satellite antenna aperture and gain for SAN GEO

As per the WF [1], the agreement concerning equivalent satellite antenna aperture and gain for SAN GEO for Ku-band NTN is shown as below:

Following Table 2 illustrates agreed LEO and GEO Satellite parameters: Table 2 Satellite Parameters Satellite orbit; GEO; LEO-600. Satellite altitude; 35786 km; 600 km. Satellite antenna pattern; Section 6.4.1 in [2]; Section 6.4.1 in [2]. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (Note 1); Ku-band (i.e. 11 GHz for DL); 4 m; 0.91 m. Satellite EIRP density; 43 dBW/MHz; 7 dBW/MHz. Satellite Tx max Gain; 43 dBi; 38.5 dBi. 3dB beamwidth; 0.400 deg; 1.768 deg. Satellite beam diameter (Note 2); 250 km; 19 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (Note1); Ku-band (i.e. 14 GHz for UL); 4 m; 0.714 m. G/T; 16.5 dB K-1; 10.9 dB K-1. Satellite RX max Gain; 44 dBi; 38.5 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB..

The antenna gain is dependent on 3dB beamwidth, and 3dB beamwidth is dependent on frequency(λ) and antenna diameter (D) as following equation.

2θ-3dB=2×Arcsin1.616×λπD                                                                                                                       (Eq.1)

For satellite antenna parameter, we can use 58.5 dBi for 20GHz and 0.1768degree 3dB beamwidth as reference, the antenna gain for other frequency and antenna diameter can be calculated by following Eq.2.

Gf1=Gf0-20log10⁡2θ-3dBf12θ-3dBf0                                                                                                                        (Eq.2)

Where,

Gf0=58.5

2θ-3dBf0=0.1768 degree, which is calculated by Eq.1 with frequency is 20GHz and antenna diameter is 5m.

2θ-3dBf1 can be calculated by Eq.1 for other frequency and antenna diameter.

Using above Eq.2 and Eq.1, the antenna gain and antenna diameter for parabolic Antenna are shown in the following table.

| Frequency range | Frequency (GHz) | Antenna diameter （m) | 3dB beamwidth (degree) | Gain (dBi) | Gain(dBi) from TR 38.821 |
| --- | --- | --- | --- | --- | --- |
| Ka-band | 20 | 5 | 0.1768 | 58.5 | 58.5 |
|  | 30 | 3.33 | 0.177 | 58.5 | 58.5 |
|  | 20 | 0.5 | 1.7684 | 38.5 | 38.5 |
|  | 30 | 0.333 | 1.7702 | 38.5 | 38.5 |
|  | 20 | 2 | 0.4421 | 50.5 | 50.5 |
|  | 30 | 1.33 | 0.4432 | 50.5 | 50.5 |
|  | 20 | 0.2 | 4.4219 | 30.5 | 30.5 |
|  | 30 | 0.13 | 4.5354 | 30.3 | 30.5 |
| 2GHz | 2 | 22 | 0.4019 | 51.4 | 51 |
|  | 2 | 2 | 4.4219 | 30.5 | 30 |
|  | 2 | 12 | 0.7368 | 46.1 | 45.5 |
|  | 2 | 1 | 8.8505 | 24.5 | 24 |
| Ku-band | 11 | 9.1 | 0.1767 | 58.5 |  |
|  | 14 | 7.14 | 0.1769 | 58.5 |  |
|  | 11 | 0.91 | 1.7666 | 38.5 |  |
|  | 14 | 0.714 | 1.7691 | 38.5 |  |
|  | 11 | 4 | 0.4019 | 51.4 |  |
|  | 14 | 3.14 | 0.4023 | 51.4 |  |

As above table, for the 4m parabolic Antenna with 0.400 degree 3dB beamwidth, its antenna gain is 51.4dBi instead of 43dBi. For parabolic antenna, if the same antenna is used for DL and UL transmission, the equivalent satellite antenna aperture for DL and UL transmission should be different. Equivalent satellite antenna aperture for UL should be 3.14m.

The G/T in dB can be calculated using following equation:

G/T = Ga – NF – 10*LOG (To)                                                                                        (Eq.1)

Where:

-	Antenna Gain: Ga in dBi

-	Ambient Temperature: T0 (usually 290 K)

-	Noise Figure: NF in dB.

RAN4 agreed that NF for SAN for Ku-band is 3dB. So for 51.4dBi Rx gain, the G/T for Ku-band GEO can be calculated as 23.8 dB K-1.

As above analysis, for 4m parabolic Antenna, antenna gain is 43dBi, comparing to 51.4dBi, antenna efficiency is very low.

Based on above discussion, the satellite parameter in [1,2] should be updated as in following table.

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 51.4 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 3.14 m | 0.714 m |
| G/T |  |  | 23.8 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 51.4 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

Based on above analysis, we propose the

Proposal 1: The following satellite Parameters for Ku-band co-existence is more reasonable.

Table 2 Satellite Parameters

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 51.4 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 3.14 m | 0.714 m |
| G/T |  |  | 23.8 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 51.4 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

NTN UE parameters for VSAT(parabolic)

In R4-2508774[2] in RAN4#115 on Ku band co-existence simulation assumptions (running document), some parameters for VSAT(parabolic) in Table 6a.2.2.2-2 are not aligned with those in Table 6a.2.2.2-1.

The Table 6a.2.2.2-1 in R4-2508774 is copied as below:

Table 6a.2.2.2-1: NTN UE antenna characteristics for system level simulations

| 1 | VSAT Antenna Characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| 1.1 | Antenna pattern | TR 38.921 | TR 38.921 | Circular Aperture TR 38.863 |
| 1.2 | Element gain (dBi) (Note 1) | 3.5 | 3.5 | N/A |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis | N/A |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB | N/A |
| 1.5 | Antenna polarization (Note 5) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements | 78x78 elements | 60 cm diameter aperture |
| 1.7 | Number of supported polarizations, P | 1 | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda | N/A |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 | N/A |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | 6.9 | 9.1 | N/A |
| 1.11 | Tx Transmit power | 10 Watts for GEO and for LEO | 50 Watt for GEO | 10 Watts for GEO and for LEO |
| 1.12 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees | 0~360 degrees |
| 1.13 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees | 0~60 degrees |
| 1.14 | Normal direction | Toward X+ axis | Toward X+ axis | Toward X+ axis |
| 1.15 | Max EIRP Value | 76.6 dBm | 88.2 dBm | 76.6 dBm |
| 1.16 | Noise Figure | 6 dB | 6.0 dB for GEO | 2.5 dB for both LEO and GEO |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization |  |  |  |  |

The Table 6a.2.2.2-2 in R4-2508774is copied as below:

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | VSAT (Parabolic) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 100 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | NTN VSAT Type 4 and 5: 2.5 dB (GEO) and 6 dB (both GEO and LEO) |
| Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO |
| Tx antenna gain | 41.3 dBi |
| Antenna elevation angle | 35˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

Since the Table 6a.2.2.2-2 is for VSAT (Parabolic), Comparing Table 6a.2.2.2-2 and “Circular Aperture TR 38.863 ”column of Table 6a.2.2.2-1, we can find that there are following misalignments.

1)  The 100 cm equivalent aperture diameter in Table 6a.2.2.2-2 is not aligned with 60 cm equivalent aperture diameter in Table 6a.2.2.2-1.

2) The “50 W for GSO and 10 W (40 dBm) for NGSO” Tx transmit power in Table 6a.2.2.2-2 is not aligned with “10 Watts for GEO and for LEO” Tx transmit power in Table 6a.2.2.2-1.

3) The “NTN VSAT Type 4 and 5: 2.5 dB (GEO) and 6 dB (both GEO and LEO)” noise figure in Table 6a.2.2.2-2 is not aligned with “2.5 dB for both LEO and GEO”  noise figure in Table 6a.2.2.2-1.

4) The “41.3 dBi” Tx antenna gain in Table 6a.2.2.2-2 is not aligned with “36.6dBi” Tx antenna gain in Table 6a.2.2.2-1, where 36.6dBi = 76.6 dBm – 10*log10(10*1000)=36.6dBi.  And for 60 cm equivalent aperture diameter, the 36.6dBi is applicable as following table.

| Frequency range | Frequency (GHz) | Antenna diameter （m) | 3dB beamwidth (degree) | Gain (dBi) |
| --- | --- | --- | --- | --- |
| Ka-band | 30 | 0.6 | 0.9824 | 43.2 |
|  | 20 | 0.6 | 1.4737 | 39.7 |
| Ku-band | 14 | 0.6 | 2.1053 | 36.6 |
|  | 11 | 0.6 | 2.6795 | 34.5 |
| Ku-band | 14 | 1 | 1.2631 | 41 |
|  | 11 | 1 | 1.6076 | 38.9 |

So, based on above analysis, to avoid misunderstanding for calibration and evaluation, it is necessary to update the parameters in Table 6a.2.2.2-2 as following table to align those in “Circular Aperture TR 38.863 ” column of Table 6a.2.2.2-1.

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | VSAT (Parabolic) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 2.5 dB for both LEO and GEO |
| Tx transmit power | 10 Watts for GEO and for LEO |
| Tx antenna gain | 36.6 dBi |
| Antenna elevation angle | 35˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

Proposal 2: Update the parameters in Table 6a.2.2.2-2 to align those in “Circular Aperture TR 38.863 ” column of Table 6a.2.2.2-1 in co-existence simulation assumptions running document.

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | VSAT (Parabolic) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 2.5 dB for both LEO and GEO |
| Tx transmit power | 10 Watts for GEO and for LEO |
| Tx antenna gain | 36.6 dBi |
| Antenna elevation angle | 35˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

- Conclusion

This contribution provides analysis on coexistence assumptions for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: The following satellite Parameters for Ku-band co-existence is more reasonable.

Table 2 Satellite Parameters

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 51.4 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 3.14 m | 0.714 m |
| G/T |  |  | 23.8 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 51.4 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

Proposal 2: Update the parameters in Table 6a.2.2.2-2 to align those in “Circular Aperture TR 38.863 ” column of Table 6a.2.2.2-1 in co-existence simulation assumptions running document.

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | VSAT (Parabolic) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 2.5 dB for both LEO and GEO |
| Tx transmit power | 10 Watts for GEO and for LEO |
| Tx antenna gain | 36.6 dBi |
| Antenna elevation angle | 35˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

- References

- R4-2504723, Way Forward for  [114bis][313] NR_NTN_Ku_Band_Coexistence, Moderator: Intelsat, RAN4#114bis

- R4-2508774, Ku band co-existence simulation assumptions (running document), Ericsson, RAN4#115

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114

- R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document), Ericssion, RAN4#114

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#113

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis

- R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112bis

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#106

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104