3GPP TSG-RAN WG4 Meeting #116                                                                                                      R4-2509338

Bengaluru, India, 25th –29th August, 2025

Source: 	CATT

Title: 	Further discussion on SAN RF core requirements for Ku-band for NR NTN

Agenda item:	7.7.6

Document for:	Approval

- Introduction

In RAN4#115 meeting, A WF [1] on Way Forward for NR_NTN_Ku_Band_UE_SAN_RF was agreed, and remaining open issues in WF require further discussion.

In this contribution, we provide our views on SAN RF core requirements for Ku-band for NR NTN.

- Discussion

The tolerance for the radiated transmit power and SAN output power

As per WF [1], the remaining issue concerning tolerance for EIRP and TRP is copied as below:

#### Issue 2-2-1: The tolerance for the radiated transmit power and SAN output power Wayforward: - For the tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands, down select to the following two options: - Option 1: [+-2.5] dB EIRP accuracy, [+-2.2] dB TRP accuracy - Option 2: [+-3.4] dB EIRP accuracy, [+-3] dB TRP accuracy (re-using FR2-NTN)

Since the frequency of FR1-NTN Ku-and is same as that of FR2-NTN Ku-band, the EIRP accuracy and TRP accuracy for FR1-NTN Ku-band should be same as those for FR2-NTN Ku-band. Therefore, it is reasonable to define same tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands.

We support the option 1 because it is beneficial to NTN network performance.

Proposal 1: Adopt Option 1: [+-2.5] dB EIRP accuracy, [+-2.2] dB TRP accuracy for both the FR1-NTN and FR2-NTN Ku bands.

Spurious frequency range for the Ku band SAN

As per WF [1], the remaining issue concerning tolerance for spurious frequency range for Ku band SAN is copied as below:

#### Issue 2-2-3: Spurious frequency range for the Ku band SAN - Proposals: - Proposal 1 (ZTE): It is proposed to define the upper limit of spurious frequency range as 26 GHz for both FR1-NTN Ku bands and FR2-NTN Ku bands. Wayforward: - Consider proposal 1 as the starting point, companies can further check in the next RAN4 meeting.

In TS 38.108, spurious frequency range for SAN type 1-O was defined as the 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band, and spurious frequency range for SAN type 2-O was defined as 30 MHz – 2nd harmonic of the upper frequency edge of the DL operating band. It is acceptable to define the upper limit of spurious frequency range as 26GHz, which is close to the 2nd harmonic of the upper frequency edge of the DL operating band. We propose to define upper limit of spurious frequency range as 2nd harmonic of the upper frequency edge of the DL operating band instead of a fixed value (26Hz).

Proposal 2: Define upper limit of spurious frequency range as 2nd harmonic of the upper frequency edge of the DL operating band for both FR1-NTN Ku bands and FR2-NTN Ku bands.

OTA reference sensitivity level for the Ku band SAN

As per WF [1], the remaining issue concerning OTA reference sensitivity level is copied as below:

#### Issue 2-2-4: OTA reference sensitivity level for the Ku band SAN Wayforward: - Consider calculate the SAN EIS requirement for 50MHz channel bandwidth based on the UL SNR target -1 dB (MCS for QPSK 1/3) - At least 2dB implementation margin and will be taken into account. - SAN parameters for GEO - Option 1: NF = 3, Rx gain = [36-44] - Option 2: G/T = 7.5~16.5 - Option 3: G/T = 12.5~21.5 - Other options are not precluded - SAN parameters for LEO - Option 1: NF = 3, Rx gain = [30.5-38.5] - Option 2: G/T = 1.9~10.9 - Other options are not precluded

The following CBW and SCS for band n248 and n247 in R4-2505737[2] was agreed and copied as below:

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |  |

According to above agreed CBW and SCS for band n248 and n247, above CBW and SCS should be defined for the SAN GEO and LEO class reference sensitivity levels for band n248 and n247.

So, the EISREFSENS for following CBW, SCS and FRC in the below table needs to be defined.

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) |  |
| 10 | 30 | G-FR1-NTN-A1-2 (Note 1) |  |
| 25, 35, 50 | 15 | G-FR1-NTN-A1-4 (Note 1) |  |
| 20, 25, 35, 50, 70, 100 | 30 | G-FR1-NTN-A1-5 (Note 1) |  |
| Note 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

The Transmission Bandwidth for above FRCs defined in Table A.1-1 of TS 38.108 is calculated in following Table 1

Table 1: transmission bandwidth for FRCs for FR1-NTN Ku-band

| Reference channel | G-FR1-NTN-A1-1 | G-FR1-NTN-A1-2 | G-FR1-NTN-A1-4 | G-FR1-NTN-A1-5 |
| --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 15 | 30 |
| Allocated resource blocks | 25 | 11 | 106 | 51 |
| Transmission Bandwidth | 4.5MHz | 3.96MHz | 19.08MHz | 18.36MHz |

We agreed to define OTA reference sensitivity for FR1-NTN Ku-band using approach for existing SAN type 2-O. OTA reference sensitivity level for existing SAN type 2-O is defined based on declared EISREFSENS_50M. For FR1-NTN Ku-band, using EISREFSENS_50M is sufficient since it is a single declared basis level. We can use EISREFSENS_50M for FR1-NTN and FR2-NTN. In this case, we don’t need to declare another basis level.

EISREFSENS_50M is defined based on G-FR2-NTN-A1-1. The transmission for G-FR2-NTN-A1-1 defined in Table A.1-2 of TS 38.108 is calculated in following Table 2.

Table 2: transmission bandwidth for FRC

| Reference channel | G-FR2-NTN-A1-1 |
| --- | --- |
| Subcarrier spacing (kHz) | 60 |
| Allocated resource blocks | 66 |
| Transmission Bandwidth | 47.5MHz |

Using the transmission bandwidth for G-FR2-NTN-A1-1 in Table2 and transmission Bandwidth for above FRCs in Table 1, the OTA reference sensitivity for band n248 and n247 should be defined as below:

Table 10.3.2-1a: SAN GEO and LEO class reference sensitivity levels for band n248 and n247

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | [EISREFSENS_50M -10.2+ ΔFR1_REFSENS] |
| 10 | 30 | G-FR1-NTN-A1-2 (Note 1) | [EISREFSENS_50M -10.8+ ΔFR1_REFSENS] |
| 25, 35, 50 | 15 | G-FR1-NTN-A1-4 (Note 1) | [EISREFSENS_50M -4+ ΔFR1_REFSENS] |
| 20, 25, 35, 50, 70, 100 | 30 | G-FR1-NTN-A1-5 (Note 1) | [EISREFSENS_50M -4.1+ ΔFR1_REFSENS] |
| Note 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

So we propose,

Proposal 3: Define SAN GEO and LEO class OTA reference sensitivity levels for FR1-NTN Ku-band as below:

Table 10.3.2-1a: SAN GEO and LEO class reference sensitivity levels for band n248 and n247

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | [EISREFSENS_50M -10.2+ ΔFR1_REFSENS] |
| 10 | 30 | G-FR1-NTN-A1-2 (Note 1) | [EISREFSENS_50M -10.8+ ΔFR1_REFSENS] |
| 25, 35, 50 | 15 | G-FR1-NTN-A1-4 (Note 1) | [EISREFSENS_50M -4+ ΔFR1_REFSENS] |
| 20, 25, 35, 50, 70, 100 | 30 | G-FR1-NTN-A1-5 (Note 1) | [EISREFSENS_50M -4.1+ ΔFR1_REFSENS] |
| Note 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

For FR2-NTN Ku-band, it is necessary to define OTA reference sensitivity based on declared EISREFSENS_50M.

For SAN antenna gain, we think that the Option 1 is reasonable, since it was used in Ku-band co-exist simulation. So we propose to use NF=3dB, Rx gain=[36-44] dBi for GEO, NF=3dB, Rx gain =[30.5-38.5] dBi for LEO to calculate EISREFSENS_50M for FR2-NTN Ku-band.

Proposal 4: Use Option 1 NF=3dB, Rx gain=[36-44] dBi for GEO, NF=3dB, Rx gain =[30.5-38.5] dBi for LEO to calculate EISREFSENS_50M for FR2-NTN Ku-band.

As above proposal, the parameters for FR2-NTN Ku-band related to OTA reference sensitivity level are shown as below:

PkT = -174 dBm/Hz.

IM = 2 dB.

G = [36-44] dBi for GEO, [30.5-38.5] dBi for LEO.

BW for EISREFSENS_50M for Ku-band SAN is 66RB, i.e. 66*12*60*1000Hz.

-1dB SNR can be reused for EIS for Ku-band SAN.

NF for GEO and LEO SAN: 3 dB.

Because the EISREFSENS_50M is the worst EIS in OTA REFSENS RoAoA, so 3dB off peak margin need to be considered. Referring to TR 38.817-2, EISREFSENS_50M can be calculated by the following,

EISREFSENS_50M = PkT +10*log10(BW) +IM+SNR+NF –G+3dB                                                                  (Equation 1)

Using equation 1 with agreed parameter,

For GEO class SAN,

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3-36+3=-126.2312 dBm

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3-44+3=-134.2312dBm

For LEO class SAN,

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3-30.5+3=-120.7312 dBm

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3-38.5+3=-128.7312 dBm

So, for FR2-NTN Ku-band,

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-126] to [-135] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor.

So, we propose,

Proposal 5: The declared ranges for EISREFSENS_50M for FR2-NTN Ku-band are defined as follows:

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-126] to [-135] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor.

OTA in-channel selectivity for the Ku band SAN

According to TS 38.108, the 12dB ICS (3dB IoT+9dB C/I) was defined for existing SAN type 1-O GEO class ICS requirement, 24dB ICS(15dB IoT+9dB C/I) was defined for existing SAN type 1-O SAN LEO class ICS requirement, and 14dB ICS(5dB IoT +9dB C/I) was defined for existing SAN type 2-O GEO and LEO.

Since RAN4 agreed not to specify OTA SAN dynamic range for NTN Ku bands in RAN4#114bis meeting, so it shows that the IoT for Ku-band is very smaller, the IoT for existing SAN type 2-O(Ka-band) is 5dB and very smaller, so we think that 14dB ICS(5dB IoT +9dB C/I) defined for existing SAN type 2-O GEO and LEO can be reused for Ku-band GEO and LEO.

Proposal 6: 14dB ICS(5dB IoT +9dB C/I) defined for existing SAN type 2-O GEO and LEO can be reused for Ku-band GEO and LEO.

OTA out-of-band blocking for Ku band SAN

As per WF [1], the remaining issue concerning requirements for OTA out-of-band blocking for Ku-band SAN is copied as below:

#### Issue 2-2-5: OTA out-of-band blocking for the Ku band SAN Wayforward: - Option 1: - Consider ΔfOOB = 100 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands - Consider 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz is agreeable for both FR1-NTN Ku bands and FR2-NTN Ku bands - Option 2 - Reuse the parameters for the Ka band

The ΔfOOB for SAN type 1-O in Table 10.6.2.1-2 of TS 38.108 is copied as below:

Table 10.6.2.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-O | FUL,high – FUL,low < 100 MHz | 20 |

The ΔfOOB for SAN type 2-O in Table 10.6.3.1-2 of TS 38.108 is copied as below:

Table 10.6.3.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL,high – FUL,low ≤ 4000 MHz | 1500 |

For Ku-band, the maximum FUL,high – FUL,low is 14500MHz-14000MHz=500MHz. Since maximum FUL,high – FUL,low is 500MHz for Ku-band, so the ΔfOOB = 100 MHz in option 1 is more reasonable than ΔfOOB = 1500 MHz in option 2. So it is reasonable to use option 1: ΔfOOB = 100 MHz for  FUL,high – FUL,low ≤ 900 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 7: Use Option 1: ΔfOOB = 100 MHz for  FUL,high – FUL,low ≤ 900 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

In order to define same ΔfOOB for both FR1-NTN Ku-band and FR2-NTN Ku-band and to avoid affecting the existing ΔfOOB, for Option 1, it is reasonable to add ΔfOOB = 100 MHz for 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz to Table 10.6.2.1-2 to support FR1-NTN Ku-and and add Table 10.6.3.1-2a to support FR2-NTN Ku-band to avoid affecting the existing ΔfOOB for existing SAN type 2-O.

Table 10.6.2.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-O | FUL,high – FUL,low < 100 MHz | 20 |
|  | 100MHz≤FUL,high – FUL,low ≤900 MHz | 100 |

Table 10.6.3.1-2a: ΔfOOB offset for satellite operating bands for band n509 and n508

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL,high – FUL,low ≤ 900 MHz | 100 |

Proposal 8: Add ΔfOOB = 100 MHz for 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz to Table 10.6.2.1-2 to support FR1-NTN Ku-and and add Table 10.6.3.1-2a to support FR2-NTN Ku-band as below:

Table 10.6.2.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-O | FUL,high – FUL,low < 100 MHz | 20 |
|  | 100MHz≤FUL,high – FUL,low ≤900 MHz | 100 |

Table 10.6.3.1-2a: ΔfOOB offset for satellite operating bands for band n509 and n508

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL,high – FUL,low ≤ 900 MHz | 100 |

The OTA out-of-band blocking for SAN type 1-O in TS 38.108 is copied as below:

For SAN type 1-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the SAN operating band. The ΔfOOB for SAN type 1-O is defined in table 10.6.2.1-2. Table 10.6.2.1-1: OTA out-of-band blocking performance requirement Wanted signal mean power (dBm); Interfering signal RMS field-strength (V/m); Type of interfering Signal. EISminSENS + 6 dB (NOTE 1); 0.0029 (NOTE 2); CW carrier. NOTE 1: EISminSENS depends on the channel bandwidth as specified in clause 10.2. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m. NOTE 3: For SAN supporting standalone NB-IoT operation, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of 0.0103 V/m for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1 MHz step size..

As above, the 12750 MHz is defined for upper limit of frequency range of interfering signal for SAN type 1-O, for FR1-NTN Ku-band. Since the FUL,high for FR1-NTN Ku-band is 14500MHz, which is greater than  12750 MHz, so the  12750 MHz as upper limit of frequency range of interfering signal is not applicable for FR1-NTN Ku-band, referring to the upper limit of frequency range of interfering signal for SAN type 2-O, we can define upper limit of frequency range of interfering signal for FR1-NTN Ku-band as 2nd harmonic of the upper frequency edge of the operating band.

Proposal 9: Define upper limit of frequency range of interfering signal for OTA out-of-band blocking for FR1-NTN Ku-band as 2nd harmonic of the upper frequency edge of the operating band.

- Conclusion

This contribution provides analysis on SAN RF core requirements for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Adopt Option 1: [+-2.5] dB EIRP accuracy, [+-2.2] dB TRP accuracy for both the FR1-NTN and FR2-NTN Ku bands.

Proposal 2: Define upper limit of spurious frequency range as 2nd harmonic of the upper frequency edge of the DL operating band for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 3: Define SAN GEO and LEO class OTA reference sensitivity levels for FR1-NTN Ku-band as below:

Table 10.3.2-1a: SAN GEO and LEO class reference sensitivity levels for band n248 and n247

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | [EISREFSENS_50M -10.2+ ΔFR1_REFSENS] |
| 10 | 30 | G-FR1-NTN-A1-2 (Note 1) | [EISREFSENS_50M -10.8+ ΔFR1_REFSENS] |
| 25, 35, 50 | 15 | G-FR1-NTN-A1-4 (Note 1) | [EISREFSENS_50M -4+ ΔFR1_REFSENS] |
| 20, 25, 35, 50, 70, 100 | 30 | G-FR1-NTN-A1-5 (Note 1) | [EISREFSENS_50M -4.1+ ΔFR1_REFSENS] |
| Note 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

Proposal 4: Use Option 1 NF=3dB, Rx gain=[36-44] dBi for GEO, NF=3dB, Rx gain =[30.5-38.5] dBi for LEO to calculate EISREFSENS_50M for FR2-NTN Ku-band.

Proposal 5: The declared ranges for EISREFSENS_50M for FR2-NTN Ku-band are defined as follows:

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-126] to [-135] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor.

Proposal 6: 14dB ICS(5dB IoT +9dB C/I) defined for existing SAN type 2-O GEO and LEO can be reused for Ku-band GEO and LEO.

Proposal 7: Use Option 1: ΔfOOB = 100 MHz for  FUL,high – FUL,low ≤ 900 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 8: Add ΔfOOB = 100 MHz for 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz to Table 10.6.2.1-2 to support FR1-NTN Ku-and and add Table 10.6.3.1-2a to support FR2-NTN Ku-band as below:

Table 10.6.2.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-O | FUL,high – FUL,low < 100 MHz | 20 |
|  | 100MHz≤FUL,high – FUL,low ≤900 MHz | 100 |

Table 10.6.3.1-2a: ΔfOOB offset for satellite operating bands for band n509 and n508

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL,high – FUL,low ≤ 900 MHz | 100 |

Proposal 9: Define upper limit of frequency range of interfering signal for FR1-NTN Ku-band as 2nd harmonic of the upper frequency edge of the operating band.

- References

- R4-2508778, Meeting minutes for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#115

- R4-2505737, Running CR for 38.101-5 Ku bands, Eutelsat Group, RAN4#115

- R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#114bis

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#114

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107