3GPP TSG-RAN WG4 Meeting # 116	R4-2509345

# Bengaluru, India,  25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.106 | CR | 0056 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 36.106, on framework simplification for co-location/co-existence requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR aims to simplify framework for co-location/co-existence requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add simplified co-existence spurious requirement in Table 9.2.2.1-1. 2) Add simplified co-location spurious requirement in Table 9.2.3.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Co-location/co-existence requirement would bring a lot of work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.1, 9.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.143 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.2.2	Co-existence with other systems in the same geographical area

These requirements may be applied for the protection of system operating in frequency ranges other than the E-UTRA Repeater operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the E-UTRA Repeater, or they may be set by local or regional regulation as a mandatory requirement for an E-UTRA operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in Clause 4.2.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, UTRA, E-UTRA, etc.) as listed below.

9.2.2.1	Minimum requirement

Unless otherwise stated this requirement applies to the uplink and downlink of the repeater, at maximum gain.

The power of any spurious emission shall not exceed the limits of Table 9.2.2.1-1 for an E-UTRA Repeater where requirements for co-existence with the system listed in the first column apply.

Table 9.2.2.1-1: Spurious emissions limits for E-UTRA-FDD repeater in geographic coverage area of systems operating in other frequency bands

| System type operating in the same geographical area |  | Frequency range for co-existence requirement |  | Maximum Level |  | Measurement Bandwidth |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 |  | 921 - 960 MHz |  | -57 dBm |  | 100 kHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 8. |  |
|  |  | 876 - 915 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4 |  |
| DCS1800 |  | 1805 - 1880 MHz |  | -47 dBm |  | 100 kHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 3. |  |
|  |  | 1710 - 1785 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| PCS1900 |  | 1930 - 1990 MHz |  | -47 dBm |  | 100 kHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. |  |
|  |  | 1850 - 1910 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| GSM850 or CDMA850 |  | 869 - 894 MHz |  | -57 dBm |  | 100 kHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26. This requirement applies to E-UTRA Repeater operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 824 - 849 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA Repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band I or E-UTRA Band 1 |  | 2110 - 2170 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 1 |  |
|  |  | 1920 - 1980 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band II or E-UTRA Band 2 |  | 1930 - 1990 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. |  |
|  |  | 1850 - 1910 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4 |  |
| UTRA FDD Band III or E-UTRA Band 3 |  | 1805 - 1880 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9. |  |
|  |  | 1710 - 1785 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 9 in the frequency Range from 1749,9 MHz to 1784,9 MHz, since it is already covered by the requirement in clause 9.1.4. |  |
| UTRA FDD Band IV or E-UTRA Band 4 |  | 2110 - 2155 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10. |  |
|  |  | 1710 - 1755 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band V or E-UTRA Band 5 |  | 869 – 894 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26. This requirement applies for the frequency range 879-894 MHz to E-UTRA FDD repeater operating in Band 27. |  |
|  |  | 824 – 849 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 |  | 860 – 890 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 5, 6, 18, 19, 20, 26 or 27. |  |
|  |  | 815 – 830 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 18, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 6, 19, 20, 26 or 27. |  |
|  |  | 830 – 845 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 619, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 18, 20, 26 or 27. |  |
| UTRA FDD Band VII or E-UTRA Band 7 |  | 2620 - 2690 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 7. |  |
|  |  | 2500 - 2570 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band VIII or E-UTRA Band 8 |  | 925 – 960 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 8. |  |
|  |  | 880 – 915 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band IX or E-UTRA Band 9 |  | 1844.9 - 1879.9 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9. |  |
|  |  | 1749.9 - 1784.9 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band X or E-UTRA Band 10 |  | 2110 - 2170 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10 |  |
|  |  | 1710 - 1770 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 10, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 in the frequency Range from 1710 MHz to 1755 MHz, since it is already covered by the requirement in clause 9.1.4. |  |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 |  | 1475.9 - 1510.9 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 11, 21, or 32 |  |
|  |  | 1427.9 - 1447.9 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 11, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD repeater operating in band 32, this requirement applies for operation within 1475.9MHz and 1495.9MHz. |  |
|  |  | 1447.9 - 1462.9 MHz |  | -49 dBm |  | 1MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 21, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD repeater operating in band 32, this requirement applies for operation within 1475.9MHz and 1495.9MHz. |  |
| UTRA FDD Band XII or E-UTRA Band 12 |  | 728 - 746 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 12, 28 or 29. |  |
|  |  | 698 - 716 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 12 or 28, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band. |  |
| UTRA FDD Band XIII or E-UTRA Band 13 |  | 746 - 756 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 13. |  |
|  |  | 777 - 787 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 13, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band XIV or E-UTRA Band 14 |  | 758 - 768 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 14. |  |
|  |  | 788 - 798 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 14, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| E-UTRA Band 17 |  | 734 – 746 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 17. |  |
|  |  | 704 - 716 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 17, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band XX or E-UTRA Band 20 |  | 791 – 821 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 20. |  |
|  |  | 832 - 862 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 20, since it is already covered by the requirement in sub-clause 9.1.4. |  |
| UTRA FDD Band XXII or E-UTRA Band 22 |  | 3510-3590 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |  |
|  |  | 3410 – 3490 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 22 since it is already covered by the requirement in sub-clause 9.1.4. |  |
| E-UTRA Band 23 |  | 2180 – 2200 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 23. |  |
|  |  | 2000 - 2020 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 23 since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25, where the limits are defined separately. |  |
|  |  | 2000 – 2010 MHz |  | -30 dBm |  | 1 MHz |  | This requirement only applies to to the uplink E-UTRA FDD Repeater operating in band 2 or band 25. This requirement applies starting 5 MHz above the band 25 downlink operating band. |  |
|  |  | 2010 – 2020 MHz |  | -49 dBm |  | 1 MHz |  |  |  |
| E-UTRA Band 24 |  | 1525 – 1559 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA Repeaters operating in band 24. |  |
|  |  | 1626.5 – 1660.5 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA Repeater operating in band 24, since it is already covered by the requirement in subclause 9.1.4. |  |
| UTRA FDD Band XXV or E-UTRA Band 25 |  | 1930 – 1995 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. |  |
|  |  | 1850 – 1915 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 25 since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD Repeater operating in band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 9.1.4 |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  | 859 - 894 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 5 or 26. This requirement applies to E-UTRA FDD repeater operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 814 - 849 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 26, since it is already covered by the requirement in subclause 9.1.4. For E-UTRA FDD repater operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in sub-clause 9.1.4. For E-UTRA FDD repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| E-UTRA Band 27 |  | 852 - 869 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in Band 5, 26 or 27. |  |
|  |  | 807 - 824 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in Band 27, since it is already covered by the requirement in subclause 9.1.4. For E-UTRA FDD repeater operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in sub-clause 9.1.4. This requirement also applies to E-UTRA FDD repeater operating in Band 28, starting 4 MHz above the Band 28 downlink operating band . |  |
| E-UTRA Band 28 |  | 758 - 803 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 28. . This requirement also applies to E-UTRA FDD repeater operating in Band 27, starting 4 MHz below the Band 27 uplink operating band . |  |
|  |  | 703 - 748 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 28, since it is already covered by the requirement in subclause 9.1.4. |  |
| E-UTRA Band 29 |  | 717 – 728 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in Band 29. |  |
| E-UTRA Band 30 |  | 2350 - 2360 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 30. |  |
|  |  | 2305 - 2315 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 30, since it is already covered by the requirement in subclause 9.1.4. |  |
| E-UTRA Band 31 |  | 462.5 – 467.5 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72. |  |
|  |  | 452.5 – 457.5 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72, since it is already covered by the requirement in subclause 9.1.4. |  |
| UTRA FDD Band XXXII or E-UTRA Band 32 |  | 1452 – 1496 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 11, 21 or 32. |  |
| UTRA TDD in Band a) or E-UTRA Band 33 |  | 1900 - 1920 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |
|  |  |  |  | -53 dBm |  | 100 kHz |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |
| UTRA TDD in Band a) or E-UTRA Band 34 |  | 2010 - 2025 MHz |  | -52 dBm |  | 1 MHz |  |  |  |
| UTRA TDD in Band b) or E-UTRA Band 35 |  | 1850 – 1910 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 3 or band 9. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |  |
|  |  |  |  | -53 dBm |  | 100 kHz |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |  |
| UTRA TDD in Band b) or E-UTRA Band 36 |  | 1930 - 1990 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |  |
|  |  |  |  | -53 dBm |  | 100 kHz |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1. |  |
| UTRA TDD in Band c) or E-UTRA Band 37 |  | 1910 - 1930 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |  |
|  |  |  |  | -53 dBm |  | 100 kHz |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |
| UTRA TDD in Band d) or E-UTRA Band 38 |  | 2570 – 2620 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7. |  |
|  |  |  |  | -53 dBm |  | 100 kHz |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 7. |  |
| UTRA TDD Band f) or E-UTRA Band 39 |  | 1880 – 1920 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |
|  |  |  |  | -53 dBm |  | 100 kHz |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |
| UTRA TDD Band e) or E-UTRA Band 40 |  | 2300 – 2400 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 30. |  |
| E-UTRA Band 41 |  | 2496 – 2690 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 7. |  |
| E-UTRA Band 42 |  | 3400 – 3600 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |  |
| E-UTRA Band 43 |  | 3600 – 3800 MHz |  | -52 dBm |  | 1 MHz |  |  |  |
| E-UTRA Band 44 |  | 703 - 803 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to E-UTRA FDD repeater operating in Band 28. |  |
| E-UTRA Band 72 |  | 461 - 466 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72. |  |
|  |  | 451 - 456 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72, since it is already covered by the requirement in subclause 9.1.4. |  |
| Other co-existence system type not listed above |  | Frequency range of co-existence downlink operating band |  | -52dBm |  | 1 MHz |  |  |  |
|  |  | Frequency range of co-existence uplink operating band |  | -49dBm |  | 1MHz |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-existence requirements in Table 9.2.2.1-1 do not apply for the 10 MHz frequency range immediately outside the repeaters operating band frequency range of an operating band (see Table 5.5-1). This is also the case when the repeaters operating band frequency range is adjacent to the band for the co-existence requirement in the Table 9.2.2.1-1. Emission limits for this excluded frequency range may also be covered by local or regional requirements.

NOTE 2:	The Table 9.2.2.1-1 assumes that two operating bands, where the frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	The requirements of -53dBm/100kHz in Table 9.2.2.1-1 for the up link direction of the Repeater reflect what can be achieved with present state of the art technology and are based on a coupling loss of 73 dB between a Repeater and a UTRA TDD BS receiver.

NOTE 4:	The requirements of -53dBm/100kHz in Table 9.2.2.1-1 shall be reconsidered when the state of the art technology progresses.

The following requirement may be applied for the protection of PHS in geographic areas in which both PHS and E-UTRA-FDD repeaters are deployed. This requirement is also applicable at specified frequencies falling between 10 MHz below the lowest frequency of the repeaters operating band and 10 MHz above the highest frequency of the repeaters operating band.

Unless otherwise stated this requirement applies to the uplink and downlink of the repeater, at maximum gain.

The power of any spurious emission shall not exceed:

Table 9.2.2.1-2: Spurious emissions limits for E-UTRA-FDD repeater in geographic coverage area of PHS

| Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -41 dBm | 300 kHz | Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz |

The following requirement shall be applied to E-UTRA-FDD repeaters operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at specified frequencies falling between 10 MHz below the lowest frequency of the repeaters operating band and 10 MHz above the highest frequency of the repeaters operating band

Unless otherwise stated this requirement applies to the uplink and downlink of the repeater, at maximum gain.

The power of any spurious emission shall not exceed:

Table 9.2.2.1-3: Spurious emissions limits for E-UTRA-FDD repeater for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -46 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -46 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -46 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -46 dBm | 6.25 kHz |  |

This requirement shall be applied to repeaters operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at specified frequencies falling between 12.5 MHz below the first carrier frequency used and 12.5 MHz above the last carrier frequency used.

The power of any spurious emission shall not exceed:

Table 9.2.2.1-3: Spurious emissions limits for the up-link and down-link of UTRA Repeater for protection of 800 MHz public safety operations

| Operating Band | Band | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -13 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

9.2.3	Co-location with base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850 UTRA FDD, UTRA TDD and/or E-UTRA BS are co-located with an E-UTRA FDD Repeater.

Unless otherwise stated the requirements assume a 30 dB coupling loss between transmitter and receiver.

NOTE:	For co-location with UTRA, the requirements are based on co-location with Wide Area UTRA FDD or TDD base stations.

9.2.3.1	Minimum Requirements

Unless otherwise stated this requirement applies to the uplink and downlink of the repeater, at maximum gain.

The power of any spurious emission shall not exceed the limits of Table 9.2.3.1-1 for an E-UTRA FDD Repeater where requirements for co-location with a Base Station listed in the first column apply.

Table 9.2.3.1-1: Spurious emissions limits for E-UTRA-FDD Repeater co-located with Base Stations

| Type of co-located Base Station |  |  |  | Frequency range for co-location requirement |  |  |  | Maximum Level |  | Measurement Bandwidth |  |  |  |  | Note |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 |  |  |  | 876 - 915 MHz |  |  |  | -98 dBm |  | 100 kHz |  |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. |  |  |  |  |
| DCS1800 |  |  |  | 1710 - 1785 MHz |  |  |  | -98 dBm |  | 100 kHz |  |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. |  |  |  |  |
| PCS1900 |  |  |  | 1850 - 1910 MHz |  |  |  | -98 dBm |  |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| GSM850 or CDMA850 |  |  |  | 824 - 849 MHz |  |  |  | -98 dBm |  |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. For E-UTRA Repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |  |  |
| UTRA FDD Band I or E-UTRA Band 1 |  |  |  | 1920 - 1980 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band II or E-UTRA Band 2 |  |  |  | 1850 - 1910 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band III or E-UTRA Band 3 |  |  |  | 1710 - 1785 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 9 in the frequency Range from 1749,9 MHz to 1784,9 MHz, since it is already covered by the requirement in clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band IV or E-UTRA Band 4 |  |  |  | 1710 - 1755 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band V or E-UTRA Band 5 |  |  |  | 824 - 849 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band VI or XIX or E-UTRA Band 6, 18 or 19 |  |  |  | 815 - 830 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 18, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 6, 19, 20 26 or 27. |  |  |  |
|  |  |  |  | 830 - 845 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 6 or 19, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 18, 20, 26 or 27. |  |  |  |
| UTRA FDD Band VII or E-UTRA Band 7 |  |  |  | 2500 - 2570 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band VIII or E-UTRA Band 8 |  |  |  | 880 - 915 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band IX or E-UTRA Band 9 |  |  |  | 1749.9 - 1784.9 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band X or E-UTRA Band 10 |  |  |  | 1710 - 1770 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 10, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 in the frequency Range from 1710 MHz to 1755 MHz, since it is already covered by the requirement in clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 |  |  |  | 1427.9 - 1447.9 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 11, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement applies only for operation between 1475.9 MHz and 1495.9 MHz for E-UTRA FDD repeater operating in band 32. |  |  |  |
|  |  |  |  | 1447.9 - 1462.9 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 21, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement applies only for operation between 1475.9 MHz and 1495.9 MHz for E-UTRA FDD repeater operating in band 32. |  |  |  |
| UTRA FDD Band XII or E-UTRA Band 12 |  |  |  | 698 - 716 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 12 or 28, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XIII or E-UTRA Band 13 |  |  |  | 777 - 787 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 13, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XIV or E-UTRA Band 14 |  |  |  | 788 - 798 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 14, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 17 |  |  |  | 704 - 716 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 17, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XX or E-UTRA Band 20 |  |  |  | 832 - 862 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 20, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XXII or E-UTRA Band 22 |  |  |  | 3410 - 3490 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 22, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 23 |  |  |  | 2000 - 2020 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 23, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 24 |  |  |  | 1626.5 – 1660.5 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 24, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XXV or E-UTRA Band 25 |  |  |  | 1850 - 1915 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 25, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. For the uplink of E-UTRA FDD Repeater operating in band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  |  |  | 814 – 849 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 26. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. For E-UTRA FDD Repeater operating in band 5, it applies from 814 MHz to 824MHz, while the rest is covered in sub-clause 9.1.4, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 27 |  |  |  | 807 – 824 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 27. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. For E-UTRA FDD Repeater operating in band 26, this requirement applies from 807 MHz to 814MHz, while the rest is covered in sub-clause 9.1.4, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 28 |  |  |  | 703 – 748 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 28. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 30 |  |  |  | 2305 – 2315 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 30. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 31 |  |  |  | 452.5 – 457.5 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 31 or 72. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| UTRA TDD in Band a) or E-UTRA Band 33 |  |  |  | 1900 - 1920 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
| UTRA TDD in Band a) or E-UTRA Band 34 |  |  |  | 2010 - 2025 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
|  |  |  |  |  |  |  |  |  | -83 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
| UTRA TDD in Band b) or E-UTRA Band 35 |  |  |  | 1850 – 1910 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 3 or band 9. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |  |  |  |
| UTRA TDD in Band b) or E-UTRA Band 36 |  |  |  | 1930 – 1990 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This is not applicable to the downlink of E-UTRA-FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
| UTRA TDD in Band c) or E-UTRA Band 37 |  |  |  | 1910 - 1930 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
| UTRA TDD in Band d) or E-UTRA Band 38 |  |  |  | 2570 – 2620 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 7. |  |  |  |
| UTRA TDD Band f) or E-UTRA Band 39 |  |  |  | 1880 – 1920 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
| UTRA TDD Band e) or E-UTRA Band 40 |  |  |  | 2300 – 2400 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 30. |  |  |  |
| E-UTRA Band 41 |  |  |  | 2496 – 2690 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 7. |  |  |  |
| E-UTRA Band 42 |  |  |  | 3400 – 3600 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |  |  |  |
| E-UTRA Band 43 |  |  |  | 3600 – 3800 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  |  |  |  |  |
| E-UTRA Band 44 |  |  |  | 703 – 803 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 28. |  |  |  |
| E-UTRA Band 72 |  |  |  | 451 – 456 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 31 or 72. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| Other co-located system type not listed above |  |  |  | Frequency range of co-located uplink operating band |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  |  |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in Table 9.2.3.1-1 do not apply for the 10 MHz frequency range immediately outside the repeaters operating band frequency range of an operating band (see Table 5.5-1). This is also the case when the repeaters operating band frequency range is adjacent to the frequency range of the co-location requirement in the Table 9.2.3.1-1. The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB Repeater-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [5].

NOTE 2:	The Table 9.2.3.1-1 assumes that two operating bands, where the corresponding eNode B transmit and receive frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	The requirements of -53dBm/100kHz in Table 9.2.3.1-1 for the up link direction of the Repeater reflect what can be achieved with present state of the art technology and are based on a coupling loss of 73 dB between a Repeater and a UTRA TDD BS receiver.

NOTE 4:	The requirements of -83dBm/100kHz in Table 9.2.3.1-1 for the up link direction of the Repeater reflect what can be achieved with present state of the art technology and are based on a coupling loss of 43 dB between a Repeater and a UTRA TDD BS receiver.

NOTE 5:	The requirements of -53dBm/100kHz and -83dBm/100kHz in Table 9.2.3.1-1shall be reconsidered when the state of the art technology progresses.

<End of Change 1>