3GPP TSG-RAN WG4 Meeting # 116	R4-2509349

# Bengaluru, India,  25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | 0120 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI18)CR for TS38.108, Correction on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band[NTNNBIoT_inbandNTNNR] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) in TS 38.108 should be -92.9 instead of -92.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) from -92.3 to -92.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.2.2	Minimum requirements for SAN type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 and 7.2.2-2 for SAN type 1-H in all operating band in FR1-NTN.

Table 7.2.2-1: SAN GEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-NTN-A1-7 | -101.2 |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -99.3 |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -99.3 (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -99.4 |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -96.5 |
| 20 | 15 | G-FR1-NTN-A1-4 | -92.9 |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -92.9 (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 | -93.2 |
| 20 | 60 | G-FR1-NTN-A1-6 | -93.3 |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. Note 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. Note 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. Note 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

Table 7.2.2-2: SAN LEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-NTN-A1-7 | -104.3 |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -102.4 |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -102.4 (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -102.5 |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -99.6 |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -96.0 |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -96.0 (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -96.3 |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -96.4 |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. Note 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. Note 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. Note 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

<End of Change 1>