3GPP TSG-RAN WG4 Meeting #116                                                                                                          R4-2509356

Bengaluru, India, 25th –29th August, 2025

Source: 	CATT

Title: 	Discussion on angle and coordinate system for Expected EIRP mask for upper 6GHz

Agenda item:	7.9.4.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on angle and coordinate system for Expected EIRP mask for upper 6GHz.

- Discussion

The  Θmin, Θmax, Φmax, Φmin for test beam directions set

There are two types of coordinate systems in TS 38.141-2. The first coordinate system is defined in sub-clause 4.14 of TS 38.141-2 for OTA peak directions set maximum steering direction(s) (D.10).

Observation 1: The reference coordinate system defined in sub-clause 4.14 of TS 38.141-2 is for OTA peak directions set maximum steering direction(s) (D.10).

Figure 4.14-1 in sub-clause 4.14 for OTA peak directions set maximum steering direction(s) (D.10) is copied as below:

Figure 4.14-1: Reference coordinate system

The text in sub-clause 4.14 “The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure.” indicates that reference coordinate system is local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 2: The reference coordinate system in sub-clause 4.14 of TS 38.141-2 is local coordinate system associated to an identifiable physical feature on the base station enclosure.

From our understanding, any electrical tilts including electrical adjustable tilt, electrical down tilt for beam peak direction are included in OTA peak directions set. Since any electrical tilts are related to an identifiable physical feature on the base station enclosure(or antenna), but the mechanical tilt is related to antenna installation, the antenna installation is related to global coordinate system. So mechanical tilt is related to global coordinate system instead of local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 3: Any electrical tilts including electrical adjustable tilt, electrical down tilt for beam peak direction are included in OTA peak directions set.

Observation 4: Mechanical tilt is related to global coordinate system instead of local coordinate system associated to an identifiable physical feature on the base station enclosure.

Since the OTA peak directions set maximum steering direction(s) (D.10) is declared based on local coordinate system associated to an identifiable physical feature on the base station enclosure, so the OTA peak directions set maximum steering direction(s) (D.10) is not varied with any mechanical tilt.

Observation 5: The OTA peak directions set maximum steering direction(s) (D.10) is not varied with any mechanical tilt, since they are declared based on local coordinate system associated to an identifiable physical feature on the base station enclosure.

The second coordinate system is defined in sub-clause 6.9.5.1 of TS 38.141-2 for Expected EIRP (EEIRP) requirement. The Figure 6.9.5.1-1 in sub-clause 6.9.5.1 for Expected EIRP (EEIRP) requirement is copied as below:

Figure 6.9.5.1-1: Definitions of θHLi and θHHi angles.

From our understanding, the above coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is global coordinate system, since it is defined based on horizon. So Expected EIRP (EEIRP) requirement is defined based on global coordinate system, although, coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is not aligned with general spherical coordinate system.

Observation 6: Expected EIRP (EEIRP) requirement is defined based on global coordinate system, which is coordinate system in sub-clause 6.9.5.1 of TS 38.141-2.

As above observation 4, mechanical tilt is related to global coordinate system. The coordinate system for mechanical tilts declaration (D.64) for EEIRP is unclear.

Observation 7: The coordinate system for mechanical tilts declaration (D.64) for EEIRP is unclear.

Based on our common understanding, mechanical tilt is defined with respect to the horizon with elevation angles above the horizon negative and below the horizon positive. But above global coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is defined with respect to the horizon with elevation angles above the horizon positive and below the horizon negative. So, if using above global coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 for mechanical tilt, common 10 degree mechanical tilt is inverted and corresponding -10 degree. So the global coordinate system in sub-clause 6.9.5.1 of TS 38.141-2 is not applicable for mechanical tilts declaration (D.64). It is reasonable to define that Mechanical tilts are defined with respect to the horizon with elevation angles above the horizon negative and below the horizon positive, and add it to Table 6.9.5.1-1 for EEIRP limits.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. |  |  |

Proposal 1: Add the coordinate system for mechanical tilts to Table 6.9.5.1-1 for EEIRP limits as below:.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. |  |  |

The Table 4.9.3.2-1 and Figure 4.9.3.2-1 of TS 48.141-2 for test beam directions set for EEIRP are copied as below:

Table 4.9.3.2-1: Test beam direction set for protection of FSS UL Beam index; Elevation angle; Azimuth angle; Weighting factor. 1; Θmin+2.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 2; Θmin+2.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 3; Θmin+2.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 4; Θmin+7.5(Θmax-Θmin)/25; Φmin+2.5(Φmax-Φmin)/25; 1/21. 5; Θmin+7.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 6; Θmin+7.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 7; Θmin+7.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 8; Θmin+7.5(Θmax-Θmin)/25; Φmin+22.5(Φmax-Φmin)/25; 1/21. 9; Θmin+12.5(Θmax-Θmin)/25; Φmin+2.5(Φmax-Φmin)/25; 1/21. 10; Θmin+12.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 11; Θmin+12.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 12; Θmin+12.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 13; Θmin+12.5(Θmax-Θmin)/25; Φmin+22.5(Φmax-Φmin)/25; 1/21. 14; Θmin+17.5(Θmax-Θmin)/25; Φmin+2.5(Φmax-Φmin)/25; 1/21. 15; Θmin+17.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 16; Θmin+17.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 17; Θmin+17.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. 18; Θmin+17.5(Θmax-Θmin)/25; Φmin+22.5(Φmax-Φmin)/25; 1/21. 19; Θmin+22.5(Θmax-Θmin)/25; Φmin+7.5(Φmax-Φmin)/25; 1/21. 20; Θmin+22.5(Θmax-Θmin)/25; Φmin+12.5(Φmax-Φmin)/25; 1/21. 21; Θmin+22.5(Θmax-Θmin)/25; Φmin+17.5(Φmax-Φmin)/25; 1/21. NOTE 1: Θmin, Θmax, Φmax, Φmin is defined in the declaration D.10 with the associated declaration D.64.. Figure 4.9.3.2-1 Test beam set for conformance testing

The declaration identifier related to OTA peak directions set (D.9), OTA peak directions set maximum steering direction(s) (D.10), The mechanical down-tilt (D.64), and NOTE 21 for The mechanical down-tilt (D.64) in Table 4.6-1 of TS 38.141-2 are copied as below:

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). For the EEIRP requirement in subclause 6.9, the OTA peak direction set maximum steering direction(s) describes the BS steering range(s) for a declared mechanical tilt range (D.64). | x | x | x |
| D.64 | The mechanical down-tilt | For EEIRP the declaration of mechanical tilt together with D.10.(NOTE 21) | x | x | n/a |
| NOTE 21: The minimum mechanical down tilt and the maximum electrical steering range shall be declared. If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances: One instance is minimum mechanical down tilt and the reduced electrical steering range, and the other instance is maximum electrical steering range and increased mechanical down tilt. |  |  |  |  |  |

As above the observations, the OTA peak directions set maximum steering direction(s) (D.10) is declared based on local coordinate system, EEIRP requirement is defined based on global coordinate system, and mechanical down-tilt (D.64) is related to global system.

From above Table 4.9.3.2-1, the “NOTE 1: Θmin, Θmax, Φmax, Φmin is defined in the declaration D.10 with the associated declaration D.64.” Table 4.9.3.2-1 indicates that Θmin, Θmax, Φmax, Φmin used for test beam directions set is OTA peak directions set maximum steering direction(s) (D.10) associated mechanical down-tilt (D.64). Since the Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) (D.10) is angle in local coordinate system, and mechanical down-tilt (D.64) is angle in global coordinate system, it is unclear to whether Θmin, Θmax, Φmax, Φmin associated mechanical down-tilt used for test beam directions set refers to the angle in local coordinate system or the angle in global coordinate system.

Observation 8: It is unclear to whether Θmin, Θmax, Φmax, Φmin associated mechanical down-tilt used for test beam directions set refers to the angle in local coordinate system or the angle in global coordinate system.

If Θmin, Θmax, Φmax, Φmin associated mechanical down-tilt used for test beam directions refers to the angle in local coordinate system, the text “with the associated declaration D.64.” in “NOTE 1: Θmin, Θmax, Φmax, Φmin is defined in the declaration D.10 with the associated declaration D.64.” in Table 4.9.3.2-1 is not needed, At same time,  “For the EEIRP requirement in subclause 6.9, the OTA peak direction set maximum steering direction(s) describes the BS steering range(s) for a declared mechanical tilt range (D.64).” in declaration D.10 in Table 4.6-1 is also needed, since the Θmin, Θmax, Φmax, Φmin in OTA peak directions set maximum steering direction(s) (D.10) is declared based on local coordinate system, which is not varied with any mechanical tilt.

If Θmin, Θmax, Φmax, Φmin associated mechanical down-tilt used for test beam directions refers to the angle in global coordinate system, this will be confusion, We know the OTA peak directions set maximum steering direction(s) (D.10) is for radiated transmit power (EIRP) without considering the mechanical tilt angle. For band n104, both radiated transmit power (EIRP) and Excepted EIRP(EEIRP) are required. It is confusion that same D.10 declaration is used for both radiated transmit power (EIRP) based on local coordinate system and Excepted EIRP(EEIRP) based on global coordinate system.

Based on above analysis, it is reasonable that Θmin, Θmax, Φmax, Φmin used for test beam directions should refer to the angle in local coordinate system, and they are original OTA peak directions set maximum steering direction(s) (D.10) with considering any mechanical tilt angle.

Observation 9: Θmin, Θmax, Φmax, Φmin used for test beam directions should refer to the angle in local coordinate system, and they are original OTA peak directions set maximum steering direction(s) (D.10) with considering any mechanical tilt angle.

Based on above analysis, mechanical tilt declaration (D.64) don’t need to be together with OTA peak directions set maximum steering direction(s) (D.10). In this case, we can test EEIRP for every mechanical tilt in declared mechanical tilt range (D.64), since EEIRP is defined to apply to all supported mechanical tilts.

Proposal 2: Mechanical tilt declaration (D.64) doesn’t need to be together with OTA peak directions set maximum steering direction(s) (D.10).

Measurement and calculation of EEIRP

From above, there are four types of angle directions used for EEIRP testing as following:

- The angle θn,φm  for Expected EIRP defined based on global coordinate system with respect to the horizon in Figure 6.9.5.1-1 in sub-clause 6.9.5.1. The angle θn,φm is defined with respect to the horizon with angles above the horizon positive and below the horizon negative.

- The test beam direction (s) in test beam directions set, which is OTA peak directions set maximum steering direction(s) (D.10) declared based on local coordinate system associated to an identifiable physical feature on the base station enclosure in sub-clause 4.14 of TS 38.141-2.

- Mechanical tilt angle (D.64), as above proposal 1, Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive.

- The direction angle in coordinate system used by the test system.

From our understanding, for testing EEIRP, basic step is to measure EIRPkθn,φm and then use EIRPkθn,φm to calculate EEIRP, where θn,φm is angle defined in Figure 6.9.5.1-1 in sub-clause 6.9.5.1 instead of angle in coordinate system used by the test system. We don’t need to coordinate translation for EEIRP calculation.

We can use the following steps to align θn,φm  for Expected EIRP, test beam direction (s) in test beam directions set, and mechanical tilt angle (D.64) in different coordinate system to the direction angle in coordinate system used by the test system to avoid coordinate translation for EEIRP calculation.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS for test beam direction(s) in test beam direction set in sub-clause 4.9.3.2with the coordinate system used by the test system. 2a. Align the coordinate system orientation for EEIRP defined in Figure 6.9.5.1-1 with the coordinate system used by the test system. 2b. Align the coordinate system orientation for supported mechanical tilts (D.64) with the coordinate system used by the test system.

Proposal 3: Use the following steps to align θn,φm  for Expected EIRP, test beam direction (s) in test beam directions set, and mechanical tilt angle (D.64) in different coordinate system to the direction angle in coordinate system used by the test system to avoid coordinate translation for EEIRP calculation.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS for test beam direction(s) in test beam direction set in sub-clause 4.9.3.2with the coordinate system used by the test system.

2a. Align the coordinate system orientation for EEIRP defined in Figure 6.9.5.1-1 with the coordinate system used by the test system.

2b. Align the coordinate system orientation for supported mechanical tilts (D.64) with the coordinate system used by the test system.

After aligning the coordinate system for mechanical tilts with the coordinate system used by the test system, we can use the following steps to test EEIRP for all mechanical tilts in declared mechanical tilt angle (D.64).

4. Set the BS to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2.

4a. Set the BS with first mechanical tilt within declared mechanical tilt range (D.64) at positioner.

5. Orient the positioner (and the BS) to the positioner angle for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in Annex N.3.

6. Configure test beam direction(s) in test beam direction set in sub-clause 4.9.3.2 and test equipment for measuring test beam direction k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

Follow either the order of steps 8a/9a or 8b/9b

8a. Repeat step 6 and 7 for all K test beam directions for each positioner angle(θn,φm).

8b. Repeat step 5 and 7 for all M and N positioner angles for each test beam direction k.

9a. Repeat step 5, and 8a for all M and N positioner angles.

9b. Repeat step 6 and 8b for all K test beam directions.

10. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for bins i=1..7 as described in  Annex N.

11. Repeat step 4a to 10 for all mechanical tilts within declared mechanical tilt range (D.64).

Proposal 4: Use the following steps to test EEIRP for all mechanical tilts in declared mechanical tilt angle (D.64).

4. Set the BS to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2.

4a. Set the BS with first mechanical tilt within declared mechanical tilt range (D.64) at positioner.

5. Orient the positioner (and the BS) to the positioner angle for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in Annex N.3.

6. Configure test beam direction(s) in test beam direction set in sub-clause 4.9.3.2 and test equipment for measuring test beam direction k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

Follow either the order of steps 8a/9a or 8b/9b

8a. Repeat step 6 and 7 for all K test beam directions for each positioner angle(θn,φm).

8b. Repeat step 5 and 7 for all M and N positioner angles for each test beam direction k.

9a. Repeat step 5, and 8a for all M and N positioner angles.

9b. Repeat step 6 and 8b for all K test beam directions.

10. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for bins i=1..7 as described in  Annex N.

11. Repeat step 4a to 10 for all mechanical tilts within declared mechanical tilt range (D.64).

Based on above proposal, A CR for TS 38.141-2 on conformance testing requirement for EEIRP for upper 6GHz is provided for approval.

- Conclusion

This contribution provides analysis on angle and coordinate system for Excepted EIRP mask for upper 6GHz. The following observation and proposal is concluded as follows:

Observation 1: The reference coordinate system defined in sub-clause 4.14 of TS 38.141-2 is for OTA peak directions set maximum steering direction(s) (D.10).

Observation 2: The reference coordinate system in sub-clause 4.14 of TS 38.141-2 is local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 3: Any electrical tilts including electrical adjustable tilt, electrical down tilt for beam peak direction are included in OTA peak directions set.

Observation 4: Mechanical tilt is related to global coordinate system instead of local coordinate system associated to an identifiable physical feature on the base station enclosure.

Observation 6: Expected EIRP (EEIRP) requirement is defined based on global coordinate system, which is coordinate system in sub-clause 6.9.5.1 of TS 38.141-2.

Proposal 1: Add the coordinate system for mechanical tilts to Table 6.9.5.1-1 for EEIRP limits as below:.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. Mechanical tilts are defined with respect to the horizon with elevation angles of boresight direction of BS enclosure above the horizon negative and below the horizon positive. |  |  |

Observation 8: It is unclear to whether Θmin, Θmax, Φmax, Φmin associated mechanical down-tilt used for test beam directions set refers to the angle in local coordinate system or the angle in global coordinate system.

Observation 9: Θmin, Θmax, Φmax, Φmin used for test beam directions should refer to the angle in local coordinate system, and they are original OTA peak directions set maximum steering direction(s) (D.10) with considering any mechanical tilt angle.

Proposal 2: Mechanical tilt declaration (D.64) doesn’t need to be together with OTA peak directions set maximum steering direction(s) (D.10).

Proposal 3: Use the following steps to align θn,φm  for Expected EIRP, test beam direction (s) in test beam directions set, and mechanical tilt angle (D.64) in different coordinate system to the direction angle in coordinate system used by the test system to avoid coordinate translation for EEIRP calculation.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS for test beam direction(s) in test beam direction set in sub-clause 4.9.3.2with the coordinate system used by the test system.

2a. Align the coordinate system orientation for EEIRP defined in Figure 6.9.5.1-1 with the coordinate system used by the test system.

2b. Align the coordinate system orientation for supported mechanical tilts (D.64) with the coordinate system used by the test system.

Proposal 4: Use the following steps to test EEIRP for all mechanical tilts in declared mechanical tilt angle (D.64).

4. Set the BS to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2.

4a. Set the BS with first mechanical tilt within declared mechanical tilt range (D.64) at positioner.

5. Orient the positioner (and the BS) to the positioner angle for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in Annex N.3.

6. Configure test beam direction(s) in test beam direction set in sub-clause 4.9.3.2 and test equipment for measuring test beam direction k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

Follow either the order of steps 8a/9a or 8b/9b

8a. Repeat step 6 and 7 for all K test beam directions for each positioner angle(θn,φm).

8b. Repeat step 5 and 7 for all M and N positioner angles for each test beam direction k.

9a. Repeat step 5, and 8a for all M and N positioner angles.

9b. Repeat step 6 and 8b for all K test beam directions.

10. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for bins i=1..7 as described in  Annex N.

11. Repeat step 4a to 10 for all mechanical tilts within declared mechanical tilt range (D.64).

Based on above proposal, A CR for TS 38.141-2 on conformance testing requirement for EEIRP for upper 6GHz is provided for approval.

- References

- R4-2509357, (NR_BS_RF_req_evo-Core)CR for TS 38.141-2, On conformance testing requirement for EEIRP for upper 6GHz, CATT, RAN4#116