3GPP TSG-RAN WG4 Meeting #116	                                                                                            R4-2509358

Bengaluru, IN, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on Ambient IoT device 1 RF requirements

Agenda item:	7.22.3.2

Document for:	Approval

- Introduction

In RAN4 #115 meeting, a WF [1] on RF requirements for A-IoT was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

This contribution provides our analysis for device 1 RF requirements based on the discussion in previous meeting.

- Discussion

The open issues on RF requirements for A-IoT were discussed during the previous meetings, as shown below:

Issue 3-1-1: Transmit output power Agreement in RAN4#115: - Performance metric: EIRP - FFS on how to determine device’s peak antenna gain direction - FFS on whether to measure multiple directions - Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna. - FFS for BPSK - The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself. Issue 3-2-1: SFO requirement Agreement in RAN4#115: - No need to directly test SFO if SFO can be indirectly verified via other test. - FFS whether to explicitly define SFO requirements or not. Issue 3-3-1: SEM requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - The applicable frequency range for SEM: from the CBW edge to [OOB boundary] - Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value. Issue 3-3-1: spurious emission requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - OOB boundary - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW - Other options are not precluded - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value. Issue 3-4-1: Reference sensitivity Agreement in RAN4#115: - Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna. - Consider to define orientation requirements with following options: - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage - Option 2: spatial averaging EIS - Option 3: EIS in the peak direction - Testing metric: 10% miss detection rate, FFS on whether to consider false alarm - FFS for the FRC, consider following as starting point: SCS; 15; 15; 15; 15. PRB; 1; 2; 3; 4. TBS; 20; 20; 20; 20. CRC; 6; 6; 6; 6. M; 6; 12; 24; 24. Issue 3-5-1: Maximum input power Agreement in RAN4#115: - Define maximum input level at peak antenna direction, using [-4dBm] as starting point.

The Device Tx output power requirement has been discussed several times, but no agreements reached. Based on the agreements from RAN4 #115 meeting, the performance metric of Device Tx output power will be EIRP based. Therefore, the peak antenna gain direction of device should be discussed. We believed that the peak antenna gain direction of Device could be declaration based.

Proposal 1: For Device Tx output power requirements, the peak antenna gain direction could be declaration based.

For Device Tx output power requirements, the backscattering loss is an important factor. According to the RFID research, the backscattering loss will be affected by the CW input power. Therefore, the Device Tx output power may need to be defined as a range in order to comprehensively describe the changes in Device Tx output power caused by changes in CW input power and backscattering loss.

Proposal 2: For Device Tx output power requirements, a range should be defined based on the different CW input power.

Referring to the Tx output power discussion above, since the peak antenna gain direction for Tx output power could be defined as declaration based, the peak antenna gain direction could also be defined as declaration based for SEM requirements.

Meanwhile, since the flat requirements for SEM will be considered as starting point, the value should be discussed as well. Considering that the tentative values of SEM captured in WF are [9~14 dBc], we believed that the stricter value is appropriate because -9 dB is closer to the theoretical value, which may be difficult to achieve.

Proposal 3: For Device SEM requirements, the peak antenna gain direction could be defined as declaration based, and the SEM value should be defined as stricter than -9 dBc.

For Device spurious emissions requirements, the OOB boundary should be discussed. Since the spurious requirement is regulatory requirement, the relevant requirements could be defined in same manner with NR i.e. follow the ITU-R SM.329. Therefore, the option 2 proposed from last meeting i.e. max of 500kHz and 10 times NB where NB equals to D2R CBW could be considered.

Proposal 4: For Device spurious emissions requirements, the OOB boundary could be defined as max of 500 kHz and 10 times NB, where NB equals to D2R CBW.

- Conclusion

This contribution provides our initial views on A-IoT Device 1 requirements. The following proposals are concluded as follows:

Proposal 1: For Device Tx output power requirements, the peak antenna gain direction could be declaration based.

Proposal 2: For Device Tx output power requirements, a range should be defined based on the different CW input power.

Proposal 3: For Device SEM requirements, the peak antenna gain direction could be defined as declaration based, and the SEM value should be defined as stricter than -9 dBc.

Proposal 4: For Device spurious emissions requirements, the OOB boundary could be defined as max of 500 kHz and 10 times NB, where NB equals to D2R CBW.

- References

[1] R4-2508116, “WF on requirements for system parameters and A-IOT device”, RAN4#115, CMCC