3GPP TSG-RAN WG4 Meeting #116                                                                                            R4-2509360

Bengaluru, IN, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on feature list for NR_NTN_Ph3

Agenda item:	7.27.3.2

Document for:	Approval

- Introduction

In 3GPP release 18, RAN4 has completed the NTN feature list. In release 19, some new features were introduced, so the feature list needs to be updated.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE feature list.

- Discussion

The agreement on NR NTN PC3 UE output power enhancement was achieved during the previous meetings, as shown below:

## Topic 1: Output power enhancement for NR NTN PC3 Agreements: - Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting - FFS for details, capabilities will be needed for feature list - When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations] - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost - Boost is 0.5 dB - FFS on duplex modes - For information only: HD-FDD/FD-FDD support is indicated per band - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting - Rel-18 and Rel-19 power boosting schemes can be simultaneously supported - FFS for details - PC3 ACLR requirements apply for power boosted transmissions Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5

The power boosting scheme was introduced in Rel-18, and the corresponding UE feature was also introduced in TR 38.822, as shown below:

### 7.3.15 NR_cov_enh2 Table 7.3.15-1: RF and RRM Feature List for NR_cov_enh2 Features; Index; Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 41.NR_cov_enh2; 41-1; Support of ΔPPowerClass reporting mechanism; Support of ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting which is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance, as specified in TS 38.101-1 and TS 38.101-3.; deltaPowerClassReporting-r18; Phy-ParametersFR1; No; FR1 only; Component 1 candidate values: Type 1: The UE can only report ∆PPowerClass for non-CA operation Type 2: The UE can report ∆PPowerClass for non-CA operation, and the UE can also report ∆PPowerClass/ ΔPPowerClass,CA/∆PPowerClass,EN-DC/∆PPowerClass,NR-DC for CA operation.; Optional with capability signalling. 41.NR_cov_enh2; 41-2; Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions without modified spectrum flatness requirement; 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK without modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1.The power boosting is only enabled when signalled via RRC powerBoostPi2BPSK-r18 for BPSK and powerBoostQPSK-r18 for QPSK; 1-6, 1-7; powerBoosting-pi2BPSK-QPSK-r18; FeatureSetUplink-v1800; NO; FR1 only; The feature can be supported in below scenarios: RAN4 intends for the UE to be able support any or all scenarios below: Case 1: FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. Case 2: FR1 DL CA with a single uplink CC configured in a band where power boosting capability is indicated. The power boosting feature can be configured in this FR1 NR band. Case 3: FR1 inter-band UL CA/DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured only in one of the bands where capability is indicated. Case 4: FR1+FR2 UL CA, FR1+FR2 DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured in the FR1 NR band.; Optional with capability signalling. 41.NR_cov_enh2; 41-3; Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions with modified spectrum flatness requirement shaping; 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK with modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1. The power boosting is only enabled when signalled via RRC powerBoostPi2BPSK-r18 for BPSK and powerBoostQPSK-r18 for QPSK; 1-6, 1-7; powerBoosting-pi2BPSK-QPSK-Modified-r18; FeatureSetUplink-v1800; NO; FR1 only; The feature can be supported in below scenarios: RAN4 intends for the UE to be able support any or all scenarios below: Case 1: FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. Case 2: FR1 DL CA with a single uplink CC configured in a band where power boosting capability is indicated. The power boosting feature can be configured in this FR1 NR band. Case 3: FR1 inter-band UL CA/DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured only in one of the bands where capability is indicated. Case 4: FR1+FR2 UL CA, FR1+FR2 DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured in the FR1 NR band.; Optional with capability signalling.

For Rel-19 NR_NTN_Ph3 WI, according to the discussion at the last meeting, a new UE feature will be defined for NR NTN power boosting, with some details still need to be clarified. Referring to the WF from last meeting, the impact on UE feature list of NTN power boosting feature will be:

- Per band

- DFT-s-OFDM and QPSK and pi/2 BPSK

- Boost 0.5 dB

- Rel-18 and Rel-19 power boosting schemes can be simultaneously supported

Therefore, the for NR NTN power boosting, the feature list could be introduced as follow:

XX.	NR_NTN_Ph3

Table 2.1-1: RF and RRM Feature List for NR_NTN_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [XX.NR_NTN_Ph3] | [XX-1] | Additional power boosting for DFT-s-OFDM pi/2 BPSK and QPSK with IL reduction | 1. Support of NTN (e) RedCap UE additional power boosting for DFT-s-OFDM pi/2 BPSK and QPSK by reducing insertion loss, when applicable as defined in 6.2 of TS 38.101-5. | 1-6, 1-7, 28-3 | [NTN-additionalPowerBoosting-pi2BPSK-QPSK -r19] | [FeatureSetUplink-v1900] | NO | FR1 only | The feature can be supported in below scenarios: RAN4 intends for the NTN (e) RedCap UE to be able support the scenario below: FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. | Optional with capability signalling |

Proposal 1: Input the Rel-19 NTN (e) RedCap UE feature “power boosting” to Rel-19 feature list.

- Conclusion

This contribution provides our views on NR NTN (e) RedCap UE feature list. The following proposals are concluded as follows:

Proposal 1: Input the Rel-19 NTN (e) RedCap UE feature “power boosting” to Rel-19 feature list.

- References

- R4-2508764, Way Forward for [115][309] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4# 115.