3GPP TSG-RAN WG4 Meeting #116	                                                                                            R4-2509362

Bengaluru, IN, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on UE Rx requirement for NTN support HPUE

Agenda item:	7.6.3.2.2

Document for:	Approval

- Introduction

In RAN4 #115 meeting, a WF [1] on UE Rx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE Rx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Rx requirements for NTN support HPUE was achieved during the previous meetings, as shown below:

# Topic #2: NR-NTN HPUE Rx requirements Issue 2-1: RSD for PC2 Agreement: - To confirm the n255 RSD for PC2 as follows: n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). RSD@1Tx; 0.8; 0.8; 0.9; 0.9. RSD@2Tx; 1.6; 1.6; 1.7; 1.7. Issue 2-2: RSD for PC1.5/PC1 non-handheld devices Agreement: - Consider [0] dB RSD for PC1.5/PC1 non-handheld devices for the current NTN bands with the following. - To document 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD in the TR. - Revisit this value with more inputs in next meeting.

Since FR1 NR NTN UE and FR1 NR TN UE are implemented in a similar approach, the study of TN HPUE operating in single band FDD mode could be reused for NR NTN HPUE. Referring to the RSD study TR 38.861 and TR 38.896, the 2Tx architecture causes more severe interference compared to the 1Tx architecture, but the larger Tx-Rx separation prevents potential interference from the Tx chain.

For PC1 and PC 1.5 NTN HPUE, the Tx architectures will be non-handheld with 1Tx. For non-handheld 1Tx PC 1 and PC 1.5 NTN HPUE, since there is only 1 Tx chain, we believed that the RSD is not required. Meanwhile, the WF from RAN4 #115 meeting already agreed that the RSD for PC1 and PC 1.5 non-handheld devices will be [0] dB, we believed that the square brackets could be removed.

Proposal 1: For NR NTN 1Tx PC 1 and PC 1.5 non-handheld HPUE, the square brackets for the RSD value could be removed, i.e. 0 dB.

- Conclusion

This contribution provides our initial views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: For NR NTN 1Tx PC 1 and PC 1.5 non-handheld HPUE, the square brackets for the RSD value could be removed, i.e. 0 dB.

- References

- R4-2508008, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #115