3GPP TSG RAN WG4 Meeting #116		R4-2509376

Bangalore, India, Aug 25th – 29th , 2025

Title: 	TP to TR 38.794 on PC2 CA_n79C with UL MIMO

Agenda Item:	6.8.1

Source: 	CATT

Document for:	Approval

# 1 Introduction

In Rel-19, PC2 CA_n79C is introduced directly by approving a CR to TS 38.101-1 without any input to TR 38.794 [1], because there is no technical issue. However, as the technical report on HPUE for NR intra-band CA with high power on FDD or TDD band, it is better to capture the corresponding text proposal for completeness of the technical report indicating all such band combinations introduced in this release. Hence, in this contribution, we provide the text proposal on PC2 CA_n79C to TR 38.794 in a similar way as TR 38.897 [2].

# 2 Text proposal

< Start of text proposal>

## 6.1	CA_n79C with UL MIMO

### 6.1.1	Configurations

The configuration is specified as Table 5.5A.1-1 of TS 38.101-1.

### 6.1.2	UE maximum output power

For intra-band UL contiguous CA and UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors and all UL CCs. The period of measurement shall be at least one sub frame (1 ms), as specified in Table 6.1.2-1.

Table 6.1.2-1: UE Power Class for intra-band UL contiguous CA with UL MIMO in closed loop spatial multiplexing scheme

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n79C |  |  | 26 | +2/-3 |  |  |  |  |
| NOTE: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |  |  |

### 6.1.3	UE additional maximum output power reduction

There is no A-MPR issue for this CA configuration.

< End of text proposal>

# 3 Reference

[1] R4-2415196, “draft CR to TS 38.101-1 Introduction of PC2 CA_n79C with UL MIMO”, ZTE

[2] 3GPP TR 38.897