3GPP TSG RAN WG4 Meeting #116		R4-2509381

Bangalore, India, Aug 25th – 29th , 2025

Title: 	Finalized consideration on other RF requirements for LP-WUS

Agenda Item:	7.24.3.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

With approaching to the end of the work item, other RF requirements are almost finalized, except a few details for further study [1]. One of the open issues is the details for maximum input level:

Issue 3-2-3: Maximum input level requirements for FR1 LP-WUR Agreements: - In spec, for requirements using -25dBm as single value. - The max. input power level is specified for all channel bandwidths. FFS the side condition - in the presence of legacy RBs with the same PSD as the LPWUS RBs. - And no NR RBs in the CBW

In this contribution, we present our views on required details for defining maximum input level as a core requirement for LP-WUR.

# 2 Discussion

First of all, “all channel bandwidth” in the agreement above clearly means “all NR channel bandwidth”, and the last two sub-bullets seem contradicting. Moreover, the definition of maximum input level cannot directly reuse that for NR.

Since LP-WUS always occupies 11 PRBs, there should be “NR fillers” surrounding the LP-WUS with the same PSD and OCNG can be used for that purpose as defined in A.5.1.1/A.5.2.1 in TS 38.101-1.

Proposal 1: Reuse OCNG defined at A.5.1.1/A.5.2.1 in TS 38.101-1 as the “NR filler” for unused REs within the NR carrier where the LP-WUS is embedded, and its PSD should be the same as that of LP-WUS.

Moreover, the definition of maximum input level should be revised based on that for NR:

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4-1.

Of course, 95% principle does not apply here. A similar way to REFSENS where 1% mis-detection rate, i.e., 99% detection rate, is used instead. One proposal for maximum input level definition can be:

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified detection rate of LP-WUS shall meet or exceed the minimum requirements for the specified reference measurement channel. The detection rate shall be ≥ 99 % under the reference measurement channels as specified in Annex A.x.x (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for NR REs  as described in Annex A.5.1.1/A.5.2.1 with the same PSD as LP-WUS) with parameters specified in Table 7.4M-1.

Table 7.4M-1: Maximum input level

| Rx Parameter | Units | NR Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| LP-WUS Power in Transmission Bandwidth1 | dBm | -252 |  |  |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 2: Reference measurement channel is A.x.x. |  |  |  |  |

Proposal 2: RAN4 to define maximum input level for LP-WUR as the following texts:

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified detection rate of LP-WUS shall meet or exceed the minimum requirements for the specified reference measurement channel. The detection rate shall be ≥ 99 % under the reference measurement channels as specified in Annex A.x.x (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for NR REs  as described in Annex A.5.1.1/A.5.2.1 with the same PSD as LP-WUS) with parameters specified in Table 7.4M-1.

Table 7.4M-1: Maximum input level

| LP-WUS Rx Parameter | Units | NR Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| LP-WUS Power in Transmission Bandwidth1 | dBm | -252 |  |  |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 2: Reference measurement channel is A.x.x. |  |  |  |  |

The corresponding draftCR can be found in our companion paper [2].

# 3 Conclusion

In this contribution, we have the following proposals for maximum input level for LP-WUR:

Proposal 1: Reuse OCNG defined at A.5.1.1/A.5.2.1 in TS 38.101-1 as the “NR filler” for unused REs within the NR carrier where the LP-WUS is embedded, and its PSD should be the same as that of LP-WUS.

Proposal 2: RAN4 to define maximum input level for LP-WUR as the following texts:

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified detection rate of LP-WUS shall meet or exceed the minimum requirements for the specified reference measurement channel. The detection rate shall be ≥ 99 % under the reference measurement channels as specified in Annex A.x.x (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for NR REs  as described in Annex A.5.1.1/A.5.2.1 with the same PSD as LP-WUS) with parameters specified in Table 7.4M-1.

Table 7.4M-1: Maximum input level

| LP-WUS Rx Parameter | Units | NR Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| LP-WUS Power in Transmission Bandwidth1 | dBm | -252 |  |  |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 2: Reference measurement channel is A.x.x. |  |  |  |  |

# Reference

[1] R4-2507936, “WF for [115][136] NR_LPWUS_UERF”, vivo

[2] R4-2511268, “DraftCR to TS 38.101-1 on maximum input level for LP-WUR”, CATT