3GPP TSG RAN WG4 Meeting #116		R4-2509385

Bangalore, India, August 25th – 29th , 2025

Title: 	Further discussion on updating duty cycle evaluation with the introduction of SBFD

Agenda Item:	7.21.3.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, one issue was raised on whether or not or how to update HPUE duty cycle evaluation with the introduction of SBFD, and RAN4 reached the following common understanding [1]:

##### Issue 1-7-1: Update for HPUE duty cycle evaluation Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

Moreover, In RAN4#115, the issue was discussed however no consensus was reached [2]:

### Sub-topic 2-2: Update for HPUE duty cycle evaluation ##### Issue 2-2-1: The necessity to update for HPUE duty cycle evaluation - No agreement in this meeting and the way forward from RAN4#114bis can still be followed.

Since RAN4#116 would be the last meeting for the core requirements of this work item, we propose to have another try to conclude this issue by adding a clarification under sub-clause 6.1, which is captured in our companion paper [4].

# 2 Discussion

As seen in our paper [3], the root cause for this issue is that a symbol can be configured as either only DL transmission or only UL transmission in a TDD band without introduction of SBFD, however, it is not the case when introducing SBFD, where an SBFD symbol consists of both UL and DL transmission in TDD UL-DL pattern, hence there could be different understandings on the current wording “uplink symbols transmitted” used for uplink duty cycle evaluation:

- Understanding #1: “uplink symbols transmitted” means “uplink only symbols transmitted”, then this requires update of RAN4 specs.

- Understanding #2: “uplink symbols transmitted” means “uplink transmission”, not referring to the symbol type in TDD UL-DL pattern, then there is no need to make update of RAN4 specs.

As confirmed internally, working groups other than RAN4 has the former understanding, therefore, some clarification could be helpful in RAN4 specifications.

Two alternatives were proposed in [3] i.e., generalization or patching. Both ways have their own pros and cons as shown in the table below:

|  | Pros | Cons |
| --- | --- | --- |
| Generalization | - Future proof | - More than 35 phrases spread in TS 38.101-1 |
| Patching | - One sentence is needed | - Less user-friendliness |

Though both of them are not perfect, the patching way requires less changes which is an important aspect at this stage. Therefore, we propose to agree on the patching way.

Proposal 1: RAN4 to use the method of patching to update uplink duty cycle evaluation.

As one example, the clarification sentence can be added in sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in SBFD and non-SBFD symbols shall be counted as uplink symbols when evaluating the uplink duty cycle”, which is shown in our companion draftCR [4].

Proposal 2: RAN4 to add a clarification sentence under sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in SBFD and non-SBFD symbols shall be counted as uplink symbols when evaluating the uplink duty cycle”.

# 3 Conclusion

In this contribution, we have the following proposal for updating duty cycle evaluation with the introduction of SBFD:

Proposal 1: RAN4 to use the method of patching to update uplink duty cycle evaluation.

Proposal 2: RAN4 to add a clarification sentence under sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in SBFD and non-SBFD symbols shall be counted as uplink symbols when evaluating the uplink duty cycle”.

# Reference

[1] R4-2504752, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung

[2] R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, Samsung

[3] R4-2506047, “Discussion on updating duty cycle evaluation with the introduction of SBFD”, CATT

[4] R4-2509386, “draftCR to TS 38.101-1 on updating duty cycle evaluation with the introduction of SBFD”, CATT