3GPP TSG RAN WG4 Meeting #116		R4-2509387

Bangalore, India, Aug 25th – 29th , 2025

Title: 	Finalized consideration on power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, one open issue with down-selected options on whether the extended IBE is applied in the guard band of the original UE channel bandwidth was identified [1]:

#### Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW Issue1 Issue 2; Option1. Allocate the extended RBs from the GB of the original UE; Option2. Allocate the extended RBs from the edge of the UE CBW. Option1. Individual evaluation of the extended RBs & Horizontal line; Case1; Case2. Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line; Case3; Case4. Note: It is assumed that extended RBs are RB aligned with the UE and BS RBs for the listed cases.. - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW

Furthermore, in RAN4#115 has made an agreement on Issue 1-2-1 that the extended IBE value is derived from the outmost allocated RB of UE CBW which in principle is Case 1 while leaving a diagram to be developed, and a starting point to be further discussed on derivation of new inner RB region in extended channel bandwidth was also agreed [2]:

#### Issue 1-1-1: Extension values/ratios - Agreement in main session: - At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases - It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides. - It is FFS on details of the single side case. #### Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below: - RBStart,Low_new = max(0, RBStart,Low – RBshift_low); - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB); - RBStart,Low_new ≤ RBStart ≤ RBStart,High_new, and LCRB ≤ ceil((NRB+ RBshift_low + RBshift_high)/2); - RBshift_low, RBshift_high ≤ NRB/2 - Other approach is not precluded - The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios #### Issue 1-2-1: Derivation of the extended IBE - Agreement in main session: - Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed) - The details and the related test cases can be discussed and decided later. - If significant issue is identified, other options can be further considered.

In this contribution, we provide our views on these open issues.

# 2 Discussion

First of all, for in-band emission definition in RAN4 specs, it is not clear enough for “non-allocated RB” [3], e.g., does RBs in guard band belong to non-allocated RB?

6.4.2.3 In-band emissions The in-band emission is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain; however, the minimum requirement applies when the in-band emission measurement is averaged over 10 sub-frames. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one or more symbols, accordingly.

However, it is very clear in RAN5 specs where non-allocated RB power is measured on non-allocated RBs within fmin and fmax, where fmin and fmax associate with the lower edge and upper edge of the transmission bandwidth configuration which exclude explicitly the guard band [4]:

E.4.3 In-band emissions measurement The in-band emissions are a measure of the interference falling into the non-allocated resources blocks. Explanatory Note: The inband emission measurement is only meaningful with allocated RB(s) next to non-allocated RB. The allocated RB(s) are necessary but not under test. The non-allocated RBs are under test. The RB allocation for this test is as follows: The allocated RB(s) are at one end of the channel BW, leaving the other end unallocated. The number of allocated RB(s) is smaller than half of the number of RBs, available in the channel BW. This means that the vicinity of the carrier in the centre is unallocated. There are 3 types of inband emissions: 1. General 2. IQ image 3. Carrier leakage Carrier leakage are inband emissions next to the carrier. IQ image are inband emissions symmetrically (with respect to the carrier) on the other side of the allocated RBs. General are applied to all unallocated RBs. For each evaluated RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. In specific the following combinations: - Power (General) - Power (General + Carrier leakage) - Power (General + IQ Image) 1 and 2 is expressed in terms of power in one non allocated RB under test, normalized to the average power of an allocated RB (unit dB). 3 is expressed in terms of power in one non allocated RB, normalized to the power of all allocated RBs. (unit dBc). This is the reason for two formulas Emissions relative. Create one set of Y(t,f) per slot according to the timing “” For the non-allocated RBs below the in-band emissions are calculated as follows , where the upper formula represents the in band emissions below the allocated frequency block and the lower one the in band emissions above the allocated frequency block. is a set of DFT-s-OFDM symbols with the considered modulation scheme being active within the measurement period, is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. for the first upper or for the first lower adjacent RB), and are the lower and upper edge of the UL transmission BW configuration, and are the lower and upper edge of the allocated BW, is the SCS, and is the frequency domain signal evaluated for in-band emissions as defined in the clause E.3.3

Therefore, it is better to make a simple clarification in legacy IBE definition, e.g., “The in-band emission is defined as the average emission across 12 sub-carriers within UL transmission bandwidth configuration and as a function of the RB offset from the edge of the allocated UL transmission bandwidth”

Proposal 1: RAN4 to clarify the legacy IBE definition within UL transmission bandwidth configuration to be aligned with RAN5 specs.

As agreed in RAN4#115, a fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW, and sub-clause 6.4.2.3 is a good place to capture this agreement.

Proposal 2: RAN4 to capture the fixed IBE for extended CBW under sub-clause 6.4.2.3.

The proposed changes can be found in our companion paper [5].

Regarding the extension itself, several details should be settled because of the following facts:

- Guard band of the original UE channel bandwidth is not an integer multiple of PRBs corresponding the concerned SCS;

- Extended PRBs should be aligned with the common PRB grid;

- The allowed extension ratio ½ and ¼ may lead to fractional number of PRBs.

Observation 1: Several details should be settled due to the facts that original guard band is not an integer multiple of PRBs, and extended PRBs should be aligned with the common PRB grid, and the allowed extension ratio may lead to fractional number of PRBs.

Hence, where is the starting position of the extension should be decided at first. Obviously, it does not make any sense that the extended starts from the outer edge of UE channel bandwidth, i.e., the edge of original guard band. Instead, the extended RBs should start from the edge of UE transmission bandwidth configuration.

Proposal 3: Extended RBs should start from the edge of UE transmission bandwidth configuration, not the edge of UE channel bandwidth.

It is reasonable to presume that the number of extended RBs, labelled as NRB-EXT-L for the lower end, NRB-EXT-H for the higher end, should be the floor of extension ration times NRB, i.e.,  NRB-EXT-L = floor(RL * NRB), NRB-EXT-H = floor(RH * NRB). It implies that transmission bandwidth configuration of extended UE channel bandwidth NRB-EXT is the sum of NRB-EXT-L and NRB-EXT-H and NRB, i.e., NRB-EXT= NRB-EXT-L+ NRB-EXT-H + NRB.

Proposal 4: The number of extended RBs at lower end and higher end is calculated as the floor of lower-end extension ratio and higher-end extension ration respectively, making transmission bandwidth configuration of extended UE channel bandwidth equal to NRB-EXT= floor(RL * NRB) + floor(RH * NRB) + NRB.

However, the extended UE channel bandwidth does not have such issues, therefore, the lower extended bandwidth starts from the lower edge of original UE channel bandwidth and the higher extended bandwidth starts from the higher edge of original UE channel bandwidth.

Proposal 5: The extended bandwidth should start from the edge of original UE channel bandwidth.

Of course, the size should also directly be the product of extension ratio and UE channel bandwidth, that is to say, the size of the lower extended bandwidth is BEXT-L = RL * CBW, and that of the higher extended bandwidth is BEXT-L = RH * CBW, hence the extended UE channel bandwidth should be the sum of lower extended bandwidth and higher extended bandwidth and original UE channel bandwidth, i.e., BEXT = (RL + RH+1) * CBW.

Proposal 6: The extended UE channel bandwidth BEXT is equal to (RL + RH+1) * CBW.

Regarding applicability of the fixed IBE value, it can be from the edge of original UE transmission bandwidth configuration to the last non-fractional RB within the extended UE channel bandwidth, since it is per-RB basis.

Proposal 7: The fixed IBE value should apply from the edge of original UE transmission bandwidth configuration to the last non-fractional RB within the extended UE channel bandwidth.

It is straightforward that the size of the virtual concept “extended guard band” can be derived as RL * CBW + GB – (floor(RL * NRB) + floor(RH * NRB) + NRB)*12*SCS for lower extended guard band, and RH * CBW + GB – (floor(RL * NRB) + floor(RH * NRB) + NRB)*12*SCS for higher extended guard band.

Proposal 8: The size of the virtual concept “extended guard band” can be derived as RL * CBW + GB – (floor(RL * NRB) + floor(RH * NRB) + NRB)*12*SCS for lower extended guard band, and RH * CBW + GB – (floor(RL * NRB) + floor(RH * NRB) + NRB)*12*SCS for higher extended guard band.

Fig. 1 illustrates proposals above.

Fig. 1, Extension of UE channel bandwidth for improved MPR

Regarding the new inner region, we can derive it via three steps:

- Step 1: use the coordination system of the extended NRB for extended inner region, which can reuse the current definition directly but with extension:

- Step 2: move to the original coordination system of the original NRB:

- Step 3: Inter-section of the original outer region and extended inner region

Under these three steps, the final new inner region is defined as:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

Proposal 9: The new inner region is defined in RAN4 specs as:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

# 3 Conclusion

In this contribution, we have the following observation and proposals for power domain enhancements for single carrier:

Observation 1: Several details should be settled due to the facts that original guard band is not an integer multiple of PRBs, and extended PRBs should be aligned with the common PRB grid, and the allowed extension ratio may lead to fractional number of PRBs.

Proposal 1: RAN4 to clarify the legacy IBE definition within UL transmission bandwidth configuration to be aligned with RAN5 specs.

Proposal 2: RAN4 to capture the fixed IBE for extended CBW under sub-clause 6.4.2.3.

Proposal 3: Extended RBs should start from the edge of UE transmission bandwidth configuration, not the edge of UE channel bandwidth.

Proposal 4: The number of extended RBs at lower end and higher end is calculated as the floor of lower-end extension ratio and higher-end extension ration respectively, making transmission bandwidth configuration of extended UE channel bandwidth equal to NRB-EXT= floor(RL * NRB) + floor(RH * NRB) + NRB.

Proposal 5: The extended bandwidth should start from the edge of original UE channel bandwidth.

Proposal 6: The extended UE channel bandwidth BEXT is equal to (RL + RH+1) * CBW.

Proposal 7: The fixed IBE value should apply from the edge of original UE transmission bandwidth configuration to the last non-fractional RB within the extended UE channel bandwidth.

Proposal 8: The size of the virtual concept “extended guard band” can be derived as RL * CBW + GB – (floor(RL * NRB) + floor(RH * NRB) + NRB)*12*SCS for lower extended guard band, and RH * CBW + GB – (floor(RL * NRB) + floor(RH * NRB) + NRB)*12*SCS for higher extended guard band.

Proposal 9: The new inner region is defined in RAN4 specs as:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

# Reference

[1] R4-2505099, “WF on power domain enhancements”, Huawei, HiSilicon

[2] R4-2507930, “WF on NR_ENDC_RF_Ph4_part1”, Moderator(Huawei, HiSilicon)

[3] 3GPP TS 38.101-1 v19.2.0

[4] 3GPP TS 38521-1 v19.0.0

[5] R4-2509388, “draftCR to TS 38.101-1 on IBE for UE capable of Rel-19 power domain enhancements”, CATT