3GPP TSG-RAN WG4 #116	R4-2509394

Bengaluru, India, August 25-29, 2025

Agenda item:	7.20.5		RRM performance requirements

Source:	Qualcomm Incorporated

Title:	RRM performance requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #115 the work on RRM performance requirements for MIMO Phase 5 has been started. In this paper we provide our view on RRM performance requirements identified in [1] in RAN4 #115.

# Discussion

## UE-initiated/Event-driven Beam Management

#### Issue 2-1-2: Test scope

<Way forward >

- Further discuss based on the table below:

|  | QC | Nokia | Samsung | Apple | CMCC | Huawei | Ericsson |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Y |  | Event-2 1 TC for Event-2 No for Event-7 |  | Y | Event-2 FFS on others | Event-2&7 |
| Whether to cover Mode A and Mode B? |  | Mode A FFS on Mode B | Y |  |  |  | Mode A at least |
| Event-trigger for Intra-cell BM for SSB | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y |  |  | FFS | ? |
| Event-trigger for Inter-cell BM for CSI-RS |  | FFS |  |  |  |  |  |
| Whether to cover no time window and eventInstanceCount-r19 > 1? |  |  | Y |  | Y |  | Y |
| FR2? |  |  |  | Y |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI |  |  |  |  |  | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |

Regarding the test scope, the following table has been collected in RAN4 #115.

Our updated view is as follows:

Proposal 1: RAN4 should define test cases for event-2. Whether to define test cases for event-7 and event-1 can be further discussed.

Proposal 2: RAN4 should not define test cases for CSI-RS based inter-cell beam management.

Proposal 3: No performance requirements should be defined for eventInstanceCout-r19 > 1. Performance requirements should only be defined for the case when no time window is defined.

## CJTC Reporting

The first open issue was whether RAN4 should define reporting mapping tables for delay, frequency and phase offset reporting.

#### Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

<Way forward >

- Option 1: (Samsung, Apple, CMCC)

  - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

We think that RAN4 should define such tables for all three offset reporting schemes.

Proposal 4: RAN4 should define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F and CJTC-P.

The next open issue was whether RAN4 should define accuracy requirements for delay, frequency and phase offset reporting.

#### Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

<Way forward >

- Option 1: (Samsung)

  - RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F.

- Option 2: (Apple)

  - RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

- Option 3: (Ericsson)

  - Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for:

    - Different number of samples

    - High and low SNR

    - Normal and extreme conditions

In our opinion RAN4 need1 to define accuracy requirements for time offset and frequency offset reporting. The number of samples to be used will be defined in the core requirements. Since it is likely that no demod requirements will be defined, we should have at least accuracy requirements in RRM for time and frequency offset reporting. For phase offset reporting, no discussions at all took place in Rel-19. Hence, we think that neither core nor performance requirements should be defined in Rel-19.

Proposal 5: RAN4 should define accuracy requirements for time offset and frequency offset reporting. The number of samples will be defined in the core requirements.

Proposal 6: RAN4 should not define core and performance requirements in Rel-19 for phase offset reporting.

## Asymmetric DL sTRP/UL mTRP

#### Issue 2-3-1: Test scope

<Way forward >

- Further discuss based on the table below:

| High level: | QC | Nokia | Apple | Samsung | CMCC | Huawei |
| --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y |  | Y |
| Whether to define TCs to verify TCI state switching requirements? | Y |  |  | N |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | both | both | both |  | both | Scenarios#1 |

Regarding the test scope for asymmetric DL sTRP / UL mTRP, the following table has been collected in RAN4 #115.

In our opinion, RAN4 should define test cases for FR1 and FR2. Also, both scenarios #1 and #2 should be covered by test cases in FR1. For FR2, however, we are fine to define only test cases for scenario #2. All these cases are in the scope of the work item. If no performance requirements are defined, the task of the work item is not fulfilled in our opinion.

Proposal 7: RAN4 should define test cases for FR1 and FR2.

Proposal 8: Also, both scenarios #1 and #2 should be covered by test cases for FR1. For FR2, only test cases for scenario #2 should be defined.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #115. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 should define test cases for event-2. Whether to define test cases for event-7 and event-1 can be further discussed.

Proposal 2: RAN4 should not define test cases for CSI-RS based inter-cell beam management.

Proposal 3: No performance requirements should be defined for eventInstanceCout-r19 > 1. Performance requirements should only be defined for the case when no time window is defined.

Proposal 4: RAN4 should define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F and CJTC-P.

Proposal 5: RAN4 should define accuracy requirements for time offset and frequency offset reporting. The number of samples will be defined in the core requirements.

Proposal 6: RAN4 should not define core and performance requirements in Rel-19 for phase offset reporting.

Proposal 7: RAN4 should define test cases for FR1 and FR2.

Proposal 8: Also, both scenarios #1 and #2 should be covered by test cases for FR1. For FR2, only test cases for scenario #2 should be defined.

# References

- R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #115, May 2025