3GPP TSG-RAN WG4 Meeting #116	R4-2509395

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	7.12.3

Document for:	Discussion

Introduction

The study item on Spatial Channel Model (SCM) for demodulation performance requirements (FS_NR_demod_SCM) resulted in the agreed WF [1] at RAN4#115.

This contribution addresses the open issues captured in the last WF [1] and the discussions surrounding it. Specific emphasis is placed on the following topics:

- TR 38.753 CDLC

  - Alignment

  - Evolution

- Comparison of methodologies

  - Performance Comparison of Methodologies

  - Channel Properties Comparison

- Further topics for discussion

  - Optional requirements

  - Proposals for study conclusion

In this meeting we see it as the most important task to facilitate the writing and alignment of the TR, to capture the results delivered and insights gained during this SI, and to reach conclusions that reflect the inferences gained in the room, both qualitatively and quantitatively.

A complete list of performance comparison simulations is provided in our companion tdoc [2], which follows agreed common simulation assumptions [3].

TR38.753 CDLC

Alignment of TR38.753 CDLC

Following the efforts of RAN4#115 [4] a simulation alignment summary spreadsheet was created [5].	
While we expect the alignment to still be improved with the efforts in this meeting, we see the final results in RAN4#115 to be already well aligned across a significant cluster of contributors [6]:

Cases: 1 PDSCH 4x4:4 rnd 30% TPUT, 2 PDSCH 4x4:4 rnd 70% TPUT; 3 PMI 8x4:4 T1f 70% TPUT, 4 PMI 8x4:4 T1f 90% TPUT. Notes: “rnd” = randomly chosen TypeI codebook precoder; “T1f” = precoding following TypeI codebook PMI reported by DUT.

Figure 1: Scatter plot of delivered PDSCH SNR operating point results of 4-layer cases for the down-selected 38.753 CDL channel model candidate (previously known as “truncated and quantized CDL - A4”).

Hence, we observe that 753 CDL is ready to provide a sound basis for requirement definition

A significant cluster of implementations of the RAN4#115 version of 38.753 CDLC, are well aligned and thus show the channel model proving a sound basis for performance requirements.

We refer to our prior contribution [7], section 3.2, for a comparison with alignment thresholds used in 5G NR Rel-15 discussion to claim alignment for the legacy channel models.

Thus, we propose

RAN4 shall declare that TR 38.753 CDLC is alignable and used implementations from a significant cluster of contributors are aligned.

Notes:

- Further alignment results shared by additional contributors, ahead of RAN4#116, have already increased the size of the aligned contributors cluster, when compared to 1.

- It is not evident to distinguish model implementation reasons from receiver related reasons, when alignment is not achieved.

  - A channel model implementation that is aligned in several configurations (e.g., 4T4R and 8T4R) can be assumed to be aligned in straightforward evolutions of the aligned configurations (e.g., 8T8R); as long as the evolution does not require changes of fundamental aspects of the model or implementation.

  - It may be more likely that receiver/channel estimation/SDM implementations are to blame (e.g., applying 1st CW derived parameters to demodulate the 2nd CW, which is not at same operating point).

Evolution of TR38.753 CDLC

Last meeting’s WF [1] does not leave any open issues on the concept and details of TR38.753 CDLC. The channel description is thus stable and complete.

The RAN4#115 description of TR 38.753 CDLC is both stable and complete.

Potential SU evolution

There are two aspects of the SU CDLC model that may benefit from future evolution.

First, the group has often discussed and named parts of the TR 38.753 CDL framework as the “randomness reduction framework following TR 38.827”. However, the history of TR 38.827 did not designate the fixed ray coupling and fixed initial phases as “randomness reduction”. While there are no traces in the published contributions on what these modifications are intended to achieve, offline discussions with involved parties seem to indicate that these modifications (w.r.t. TR 38.901) were included for “convergence and alignment speedup”. As such we propose

RAN4 to change the term “randomness reduction” framework to “convergence and alignment speedup framework”.

The second avenue for CDL framework evolution and clarification concerns the agreed “angular parameter table” truncation procedure.

For one, we observed that the current cluster truncation procedure has an impact on the final angular spread (AS) of the truncated model. It would be possible to perfectly match the AS_desired of the CDL model, by iterating and converging truncation, delay spread (DS), and AS scaling, but an additional AS scaling step at the end of the current algorithm already reduces any error to “below noise” levels.

RAN4 to recommend an additional AS scaling step (to AS_desired) at the end of the cluster truncation procedure, for creation of tables to be used in future normative requirements.

Additionally, Nokia and another company have traced back an observed truncation procedure implementation misalignment, to the lack of clarify in the AAV&AE field pattern gain to cluster scaling step. It should be clarified whether the angle dependent gains of those two gain factors are dependent on mean cluster angles, or by averaging gains from ray angles (which are symmetric around the mean).

RAN4 to clarify that in the cluster truncation procedure, angular dependent gains stemming from the AAV and AE field patterns are to be calculated based on cluster mean angles.

MU evolution of CDL (“informative”)

In the previous meetings, we have described the necessity of supporting MU-MIMO use cases. In this section, we want to describe MU-MIMO use cases and channel scenarios.

Spatial consistency of CDL models

In the channel scenario for MU-MIMO case, more than one UE is involved in the same cell or in two neighbor cells for the network. Therefore, the physical channel environment should be considered as special consistent.

It means that if two UEs are served as MU-MIMO cases, both UEs should see the same clusters from their own physical locations in the wireless environment. The key aspect in the channel models is that the cluster generated for both the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should be the same clusters. Then, it is considered reasonable to have some variation on cluster power, angle, and delay between those two CDL models.

In MU-MIMO scenario related to two UEs, the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should contain the same clusters. Then, it is considered reasonable to have some variation on cluster power, angle, and delay between those two CDL models.

For the equation of generating the channel coefficients of fast fading model in TR 38.901 for MU-MIMO case can consider generating one channel model first, then generate the second channel model based on the modification of the first one by adjust the λ(f):

Similarly, for using the CDL tables, i.e., CDL-A/B/C/D/E in TR 38.901, to generate two CDL models for both UE1 and UE2, the second CDL model should be adjusted based on the first CDL model by power, delay and angles. However, due to lack of the description in the standard, then it is implementation agnostic.

The spatial consistency should be considered for MU-MIMO scenarios during the simulation. The channel model generated for both UE1 and UE2 should be based on the same clusters. One model can be generated first and the other model can be adjusted based on the first model.

CDL simplification and large bandwidth support

Based on the table of in CDL-A/B/C/D/E models in TR 38.901, there are some clusters sharing the same AoA angles.

The calculated Pearson correlation coefficient between clusters 2 and 3 is 1.000, and it is also 1.000 between clusters 3 and 4, between clusters 5 and 6, and between clusters 6 and 7. This indicate that the wide sense stationary (WSS) assumption does not hold in the frequency/delay domain between those clusters. Such channel characteristic can be observed during channel measurement in the in-field by using the sounding signal with very wide bandwidth (200MHz in FR1, 500MHz in FR2). The reason can be that the signal pulse length or signal wavelength become comparable to the size of the objects. The slightly change of the frequency cause big impact on the reflections and the diffusions from the objects.

Figure 2. based on [8].

However, in the simplified SU-MIMO channel models, the clusters sharing the same AoAs are removed. For the MU-MIMO use case, the bandwidth at gNB can be much larger than bandwidth of UE. From the gNB point of view, the modelling of the channel should consider using a model support much larger bandwidth than that of SU-MIMO case, and the channel characteristic of non-WSS may present in the frequency/delay domain.

Therefore, the CDL-A/B/C/D/E models in TR 38.901 are recommended for MU-MIMO use case instead of RAN4#115 version of TR 38.753 CDLC channel models.

Removing the clusters with same angular information from TR 38.901 CDL models may hurt the non-WSS channel characteristic in the frequency/delay domain for wide system bandwidth use cases. A non-truncated CDL model may be more suitable for high bandwidth MU-MIMO scenarios.

Comparison of methodologies

Performance Comparison

In the last meeting no conclusion was reached concerning how the performance comparison of methodologies shall be carried out. Since this is the planned last meeting for the FS_NR_demod_SCM SI, we bring our analysis in this section based on the previously described “expectation vs. observation” framework.

Comparison method

In case a shared performance comparison framework remains a discussion point during RAN4#116, we re-summarize our preference for an “expectation vs. observation” framework here.

To compare channel models, have each contributor set performance or channel property expectations for each comparison case. Then, each contributor compares their observations from simulations against their expectations, to subjectively decide whether channel models fit expectations. Context on importance of expectations for RAN4 minimum performance requirements is also given.

For the same proposal in easier to read diagrams, please refer to [7] (section 4.4).

SU 2CW MIMO PDSCH

Configuration: TPUT curves for SU 8T8R 8layers (2CWs) at low speed. CDLC uses 365ns DS, while TDL uses 300ns.

Expectation:
In deployment and channel models following typical MIMO spatial effects, the two codewords are expected to perform differently, depending on how the chosen precoder allocates layers to spatial dimensions and hence from which directions/spatial dimension the signal is received. 	
This is also expected for random T1 precoding, where each CW has an overlapping, but not equal, pool of precoders to be randomly drawn from.

Comparison:

Figure 3: TPUT curves for SU 8T8R 8layers (2CWs) at low speed.

Observations:

- TDL low: Performance for both CWs is equal. This is attributed to DoAs not mattering for receive performance in TDL low.

- TDL nonLow: No TPUT is achieved. This is attributed to the adhoc spatial correlation matrix forcing the channel to become rank deficient/extremely ill conditioned. If SNR was extended or more realistic correlation was used, random precoding would result in equal CW performance, while fixed PMI would show expected performance gap.

- CDL: Performance gap between both CWs; magnitude depending on chosen precoder. SNR level in reasonable range.

SU 1CW MIMO PDSCH

Configuration: TPUT curves for SU 4T4R 4layers (1CW) at low speed. CDLC uses 365ns DS, while TDL uses 300ns.

Expectation: PMI choice/receive DoA dependent TPUT performance at reasonable SNR levels and reasonable channel condition.

Comparison:

Figure 4: TPUT curves for SU 4T4R 4layers (1CW) at low speed.

Observations:

- TDL low: Perfectly conditioned channel. Reasonable SNR range.

- TDL nonLow: TPUT limited to 50% due to ill conditioned channel. Unreasonable SNR range.

- CDL: Performance depending on PMI/DoA. Reasonable SNR range. Reasonable channel condition.

SU PMI 4 layer

Configuration: Follow PMI conditioned TPUT curves for SU 8T4R 4layers (1CW) at low speed. CDLC uses 365ns DS, while TDL uses 300ns. eTypeII and typeI precoding.

Expectation: Reliable performance scaling between eTypeII and typeI gains that change consistently and explainable with channel configurations.

Comparison:

Figure 5: Follow PMI conditioned TPUT curves for SU 8T4R 4layers (1CW) at low speed.

Observations:

- TDL low: T1f>eT2f, and T1r>eT2f.

- TDL nonLow: Random PMI same performance as follow PMI for XP med, different for med. When channel is strongly ill-conditioned (Med) eT2f>>T1f, when well-conditioned (XP Med) T1f>>eT2f.

  - The latter is non consistent behaviour, likely an artifact from adhoc antenna correlation model, and requires per use case tuning of configurations to achieve seemingly senseful PMI requirements.

- CDL: Significant performance gap between random and follow precoding. T1f>~eT2f, aligned with experience from deployment.

Note: Random precoding results (and many more) available in companion tdoc [2]. eT2f meaning TPUT with eTypeII CB based feedback applied directly (“follow”). Similar for other abbreviations.

SU MIMO PDSCH and receiver implementation

Configuration: TPUT curves for SU 4T4R 4layers (1CW) at medium speed. CDLC uses 365ns DS, while TDL uses 300ns. Different receiver implementations: Channel estimation taking average of measured REs vs. single centre RE value applied to all RE.

Expectation: Different qualities of implementations of the same SDM/MIMO receiver approach (both equalizer and channel estimator) result in performance differences, i.e., bugs in implementation are visible by performance.

Comparison:

| TDLC low | TDLC MedA | CDLC |
| --- | --- | --- |

Figure 6: 4T4R, 1CW (MCS17) full rank, random precoding, Differing SDM receiver implementation. 
Relative TPUT vs. pre-EQ SNR.

Observations:

- TDL low: Small performance gap.

- TDL nonLow: Small performance gap.

- CDL: Significant performance gap.

MU-light and advanced receivers

Configuration: TPUT curves for simple MU setup with 4T4R 2+2 layers at medium speed. CDLC uses 365ns DS, while TDL uses 300ns. SU channel created and MU interference approximated via different precoders (either chosen randomly or “random but orthogonal”). Using advance receivers.

Expectation: The performance of advanced IRC improvements to the same receiver should not have a big performance impact, as even interference unaware SDM receiver cancel interference that comes from different spatial supports (directions/angles).

Comparison:

| TDLC low | TDLC XP Med | CDLC |
| --- | --- | --- |

Figure 7: Differing advanced receivers (R-ML, E-IRC, IRC) in presence of co-scheduled UE.
4T4R, 1CW (MCS13+MCS4), rank 2+2, MCS4 interference, orthogonal and random MU precoding.

Observations:

- TDL low: Significant performance gap between IRC and eIRC. Very large gap between R-ML and eIRC.

- TDL nonLow: Significant performance gap between IRC and eIRC. Very large gap between R-ML and eIRC.

- CDL: Negligible performance gap between IRC and eIRC. Large gap between R-ML and eIRC at high TPUT.

Conclusion

Following the performance comparison in the section we draw the following conclusions, that we would propose to capture in the TR.

RAN4 to capture the following observations on the TDL low channel model:
- Overly optimistic channel matrix condition values. 
- Neither performance impact from PMI choice nor DoA nor related SDM receiver implementation.

RAN4 to capture the following observations on the TDL nonLow channel model:
- Contradicting and non-consistent performance scaling behaviour and results (e.g., 1CW PMI). 
- Unjustified channel condition degradation and SNR requirement increase. 
- Needs heuristic correlation tuning to show expected performance scaling, hence unreliable for new features.

RAN4 to capture the following observations on the CDL channel model:
- Highlights SDM receiver issues. 
- Reasonable SNR range. 
- Matches etypeII vs. typeI deployment observations. 
- Significantly different performance between random and follow precoding. 
- Reliable performance scaling trends and explainable channel matrix condition numbers. 
- Matches deployment observations on advanced receivers in MU adjacent setups.

Finally, capture the foundations for comparison in the TR:

RAN4 to capture the performance curves all contributors in the TR, for (at least) the following use cases/configurations: SU 2CW MIMO PDSCH, SU 1CW MIMO PDSCH, SU PMI 4 layers.

RAN4 to capture all Nokia contributed performance curves from this section in the TR.

Channel Properties Comparison

In the last meeting it was agreed to use channel properties for comparison of channel models, but not for alignment.	
We see it especially useful to look at the convergence properties and spatial (“angular”) properties of the candidates.

Convergence properties

For a channel model to be usable it needs to provide reproducible results that it converges to (from different starting seeds) within reasonable time. This has direct impact on the testing time of performance requirements.

Convergence speed / testing time

To gauge potential testing-time increase issues with the CDL channel models, we have analyzed a simulation that uses a single draw/seed of 10000 slots and then calculates the TPUT limiting itself to 10/100/250/…/10000 slots.

To enable fair comparison, 50%TPUT operating point for all channel models were intended to be chosen. However, in TDLC medA the channel is very rank deficient/ill conditioned, hence not reaching higher than 32% TPUT even with MCS reduced to 7:

CDLC @ 22dB (52% TPUT) / TDLClow @ 19dB (50% TPUT) / TDLCmedA @ 50dB/MCS7 (32% TPUT)

Figure 8: Simulation convergence trials: 8T8R 2CW (MCS17 both) full rank, SU-MMSE receiver, Random precoding vs. simulated slots (at 50% TPUT operating point target).

We also observed that differing starting seeds do not matter for requirement quality numbers of samples.

Conclusion:

From the above results we observe:

The convergence speed, and hence testing time, of TR 38.753 CDLC is on par with legacy TDL nonLow and well within the number of samples needed for requirement quality evaluations. Furthermore, CDL convergence speed is independent of starting seed.

RAN4 to capture above convergence properties results in the TR, along with the observation that TR 38.753 CDLC convergence speed, and hence testing time, is on par with legacy TDL nonLow.

Spatial properties

In the last WF [1] this was split in three separate categories:

- Power Angular/Phase Distribution in Tx and Rx Directions (including comments on stability and diversity)

- Time Coherence as channel correlation between symbols

- Frequency Coherence measured as channel correlation between subcarriers

Though we see all three dimensions as being the same spatial properties, evaluated in different dimensions. Hence, we present all outcomes in the same set of results in the following.

We are using a simple Bartlett style direction of arrival (DoA) estimation algorithm (approximately a receiver beam sweeping approach) to show the receive direction preferences of the channel, based on a DUT ULA assumption. Other, more complex and higher resolution, algorithms exist (e.g., the "MUSIC" or "ESPRIT" based ones), and various SDM receivers are able to exploit more directional information than the Bartlett visualization resolves. 	
The same is true for any direction of departure (DoD) estimations, which rely on a URA assumption of the transmitter, where applicable.

Results: (BT/Ericsson) measurements

Taken from [9].

Figure 9: From [9] “Figure 2. Three primary AoAs for SRS from UE in Locations A and C.” using MUSIC algorithm.

Looking at DoAs from field measurements, we observe a limited number of mid-term stable directions (environment properties). It is suspected that the DoAs relative strengths vary over time, albeit those measurements are not disclosed in the measurement contribution.

Results: Legacy channel TDL low

| Slot 1 | Slot 5 |
| --- | --- |

Figure 10: TDLC300-100 (low) Bartlett DoA analysis vs. relative “RE distance”:

Looking at the DoAs of a 3GPP (low correlation) TDL channel, the large-scale spatial preference of the per RE channels is seen to fully decorrelate after about 2.5ms (5 slots in our simulation); it could be argued that this already occurs after only 1ms

Results: Legacy channel TDL nonLow

| Slot 1 | Slot 5 |
| --- | --- |

Figure 11: TDLC300-100 (MedA) Bartlett DoA analysis vs. relative “RE distance”:

Looking at the DoAs of a 3GPP TDLC300-100 MedA channel, we can directly see the limitation to broadside spatial preference, which remains unchanged indefinitely.

Results: TR 38.753 CDLC

| Slot 20 | Slot 1002 |
| --- | --- |

Figure 12: TR 38.753 based CDLC Uma Bartlett DoA analysis vs. relative “RE distance” (x-axis is DoA):
4x1 Xpol ULA assumption.

Looking at the DoAs of the TR 38.753 based CDLC channel, we observe a limited number of mid-term stable directions (clusters), that slowly fade in and out over time (or rather with RE distance).

Results: TR 38.827 CDLC

| Slot 1 | Slot 100 |
| --- | --- |

Figure 13: TR 38.827 based CDLC Uma Bartlett DoA analysis vs. relative “RE distance” (x-axis is DoA):
4x1 Xpol ULA assumption.

Conclusion:

From the above results we observe:

The spatial properties of legacy channel models do not match the measured typical deployment MIMO characteristics (“limited number of mid-term stable directions”), while TR 38.753 CDLC matches very well.

RAN4 to capture above spatial properties results in the TR, along with the observation that the spatial properties of legacy channel models do not match the measured typical deployment MIMO characteristics, while TR 38.753 CDLC matches very well.

(PostEQ) SINR distributions

In the following, we denote the per layer post-EQ SINR as being the SINR (or quality) of each MIMO layer measured after the application of a baseband receiver/equalizer on the channel facing receive ports. Here the receiver application gives rise to the concept of MIMO layers and is usually denoted as spatial division multiplexing (SDM). 	
Note that the operating point of a system is physically given by a preEQ SNR value, which indicates the average received signal power at the channel facing receive ports (“antennas”).	
In common MIMO principles, the SINR of a layer determines how much TPUT the corresponding layer can carry. The resulting post-EQ per layer SINR are not expected to be equal, especially when optimizing for TPUT. Channel estimation and demodulation strategies need to adapt to the per layer quality to achieve industry expectations for sum TPUT.

Results: (BT) measurements

Taken from [10] and [11].

Figure 14: From [11] “Figure 2. Histograms of the [per layer] SINR assuming SVD precoding and combining for measurement locations with a strong line of sight (B), and for non-line of sight position (D).”

Looking at these postEQ processed SINR distributions from field measurements, we observe a that each spatial layer exhibits individual loss in a realistic deployment; this is in direct contrast to the extant TDL simulations.

Results: Simulations of channel model candidates

The post-EQ SINR distributions are derived with both random and fixed typeI precoding and assuming MMSE-IRC receivers. The system is further configured as 8T8R with 8layers/2CW, and pre-EQ SNR operating point is set at 25dB. Statistics are formed from 1272 REs and 1000 slots, 1dB wide binning is used.

| TDLC low | TDLC Med | 38.753 CDLC |
| --- | --- | --- |

Figure 15: Post-EQ SINR distributions for channel candidates under random (top row) and fixed (bottom row) precoding.

Conclusion:

From the above discussion we observe:

The PDSCH post-EQ SINR profiles, when using TDL channel models do not match measurements. SDM processing does not impact performance, when using TDL channel models. CDL both shows typical post-EQ SINR profiles and typical deployment spatial components.

RAN4 to capture above post-EQ SINR distribution results in the TR, along with the observation that TDL low channel models do not model mid-term stable per layer post-EQ SINR differences, and TDL nonLow do not model meaningful per layer post-EQ SINR differences at all.

Further topics for discussion

Optional requirements

Since this is a study item, no requirements will be introduced. However, a study can and shall make recommendations on how potential requirements could be introduced by a future work item, in ways that conform to any findings made during the study.

It has been discussed in this study that SCM introduction would likely require an update of the established 5G conformance testing setup, hardware, and/or software. Additionally, reservations have been raised whether specific algorithmic updates to DUTs may be necessary to adhere to new SCM based performance requirements (even though any algorithms that work in practical deployment will not have issues with a spatial channel model).

From our perspective it seems clear that any potential 5GA requirements should be encapsulated, using an optional UE capability (or BS manufacturer declaration) and applicability rules. For example:

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SCM compatible MIMO receiver | FR1 TDD | PDSCH | Clause X.X.X Test X-X |  |

This is similar to the current requirement status of the “Simplified SU-MIMO 8Rx receiver” UE capability and applicability rule.

Or put in the proposal form:

The study TR shall contain a recommendation that any SCM requirements in 5GNR are recommended to be optional via optional UE capability/BS declaration and corresponding applicability rules.

Proposal for study conclusion

Given the discussion in this contribution, along with the contributions throughout the SI, we have the following proposal for the conclusion of the study.

RAN4 to capture the following aspects about TR 38.753 CDLC as outcome of the FS_NR_demod_SCM work:
- CDL covers typical spatial channel effects expected in deployment, as well as measurements, and is aligned with RAN1 assumptions for feature development.
- CDL implementations used to contribute to the study are majority wise aligned.
- CDL is aligned with TE implementation requirements.
- CDL is preferred SCM for the large majority of contributors.
- CDL is ready for use to specify normative demodulation and CSI performance requirements for SU MIMO cases in FR1.
Additionally, for SCMs in general, specific use cases have been identified especially for setups with multiple codewords, advanced receivers, enhanced codebooks, and SDM reliant cases in general.

Conclusion

The following Observations and Proposals were made:

=== TR38.753 CDLC

- A significant cluster of implementations of the RAN4#115 version of 38.753 CDLC, are well aligned and thus show the channel model proving a sound basis for performance requirements.

- RAN4 shall declare that TR 38.753 CDLC is alignable and used implementations from a significant cluster of contributors are aligned.

The RAN4#115 description of TR 38.753 CDLC is both stable and complete.

RAN4 to change the term “randomness reduction” framework to “convergence and alignment speedup framework”.

RAN4 to recommend an additional AS scaling step (to AS_desired) at the end of the cluster truncation procedure, for creation of tables to be used in future normative requirements.

RAN4 to clarify that in the cluster truncation procedure, angular dependent gains stemming from the AAV and AE field patterns are to be calculated based on cluster mean angles.

In MU-MIMO scenario related to two UEs, the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should contain the same clusters. Then, it is considered reasonable to have some variation on cluster power, angle, and delay between those two CDL models.

The spatial consistency should be considered for MU-MIMO scenarios during the simulation. The channel model generated for both UE1 and UE2 should be based on the same clusters. One model can be generated first and the other model can be adjusted based on the first model.

Removing the clusters with same angular information from TR 38.901 CDL models may hurt the non-WSS channel characteristic in the frequency/delay domain for wide system bandwidth use cases. A non-truncated CDL model may be more suitable for high bandwidth MU-MIMO scenarios.

=== Comparison of methodologies

To compare channel models, have each contributor set performance or channel property expectations for each comparison case. Then, each contributor compares their observations from simulations against their expectations, to subjectively decide whether channel models fit expectations. Context on importance of expectations for RAN4 minimum performance requirements is also given.

RAN4 to capture the following observations on the TDL low channel model: 
- Overly optimistic channel matrix condition values.  
- Neither performance impact from PMI choice nor DoA nor related SDM receiver implementation.

RAN4 to capture the following observations on the TDL nonLow channel model: 
- Contradicting and non-consistent performance scaling behaviour and results (e.g., 1CW PMI).  
- Unjustified channel condition degradation and SNR requirement increase.  
- Needs heuristic correlation tuning to show expected performance scaling, hence unreliable for new features.

RAN4 to capture the following observations on the CDL channel model: 
- Highlights SDM receiver issues.  - Reasonable SNR range.  
- Matches etypeII vs. typeI deployment observations.  
- Significantly different performance between random and follow precoding.  
- Reliable performance scaling trends and explainable channel matrix condition numbers.  
- Matches deployment observations on advanced receivers in MU adjacent setups.

RAN4 to capture the performance curves all contributors in the TR, for (at least) the following use cases/configurations: SU 2CW MIMO PDSCH, SU 1CW MIMO PDSCH, SU PMI 4 layers.

RAN4 to capture all Nokia contributed performance curves from this section in the TR.

The convergence speed, and hence testing time, of TR 38.753 CDLC is on par with legacy TDL nonLow and well within the number of samples needed for requirement quality evaluations. Furthermore, CDL convergence speed is independent of starting seed.

RAN4 to capture above convergence properties results in the TR, along with the observation that TR 38.753 CDLC convergence speed, and hence testing time, is on par with legacy TDL nonLow.

The spatial properties of legacy channel models do not match the measured typical deployment MIMO characteristics (“limited number of mid-term stable directions”), while TR 38.753 CDLC matches very well.

RAN4 to capture above spatial properties results in the TR, along with the observation that the spatial properties of legacy channel models do not match the measured typical deployment MIMO characteristics, while TR 38.753 CDLC matches very well.

he PDSCH post-EQ SINR profiles, when using TDL channel models do not match measurements. SDM processing does not impact performance, when using TDL channel models. CDL both shows typical post-EQ SINR profiles and typical deployment spatial components.

RAN4 to capture above post-EQ SINR distribution results in the TR, along with the observation that TDL low channel models do not model mid-term stable per layer post-EQ SINR differences, and TDL nonLow do not model meaningful per layer post-EQ SINR differences at all.

=== Further topics for discussion

The study TR shall contain a recommendation that any SCM requirements in 5GNR are recommended to be optional via optional UE capability/BS declaration and corresponding applicability rules.

RAN4 to capture the following aspects about TR 38.753 CDLC as outcome of the FS_NR_demod_SCM work: 
- CDL covers typical spatial channel effects expected in deployment, as well as measurements, and is aligned with RAN1 assumptions for feature development. 
- CDL implementations used to contribute to the study are majority wise aligned. 
- CDL is aligned with TE implementation requirements. 
- CDL is preferred SCM for the large majority of contributors. 
- CDL is ready for use to specify normative demodulation and CSI performance requirements for SU MIMO cases in FR1. 
Additionally, for SCMs in general, specific use cases have been identified especially for setups with multiple codewords, advanced receivers, enhanced codebooks, and SDM reliant cases in general.

- References

- R4-2508776, Way Forward for [115][321] NR_SCM, Moderator (Nokia), RAN4#115.

- R4-2509396, Simulation results for Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#116.

- R4-2508622, Simulation assumptions for SCM, Huawei, HiSilicon, MediaTek, RAN4#115.

- R4-2508623, Ad-hoc meeting minutes for [115][321] NR_SCM, Moderator (Nokia), RAN4#115.

- R4-2508621, Simulation summary for alignment for NR SCM, Apple, Ericsson, RAN4#115.

- RP-250906, Demod topics for 5G-Advanced Rel-20, Nokia, RAN#108.

- R4-2506172, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#115.

- Y. Tan, C. -X. Wang, J. Ø. Nielsen, G. F. Pedersen and Q. Zhu, "A Novel B5G Frequency Nonstationary Wireless Channel Model," in IEEE Transactions on Antennas and Propagation, vol. 69, no. 8, pp. 4846-4860, Aug. 2021, doi: 10.1109/TA.

- R4-2419338, Further observations on the properties of MIMO channels in typical deployments, BT plc, Ericsson, RAN4#113.

- R4-2402277, Discussion on 8Rx general demodulation aspects and spatial channel models, Nokia, Nokia Shanghai Bell, BT, RAN4#110.

- R4-2411557, Scenarios and Requirements for Spatial Channel Modelling. BT plc, RAN4#112.

- R4-2411300, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#112.