3GPP TSG-RAN WG4 Meeting #116           	R4-2509406

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.8.7

Source:	Samsung

Title:	Discussion and simulation results of PUSCH demodulation requirement for ATG

Document for:	Discussion

- Introduction

In the last meeting, the test scope and test setup for performance part of ATG for Rel-19 was under discussion. The related agreement was captured into WF in [1] as following

- General Scope - Whether to reuse existing requirements for ATG UL MIMO - Only the applicable PUSCH test cases in Clause 8.2.1 with 2Tx 2 layer will be applied for ATG - Whether to introduce new test cases for ATG UL MIMO - Limited new test cases could be considered for ATG 2Tx PUSCH demodulation requirements - Whether to introduce new PUCCH and PRACH test cases for ATG UL MIMO - No PUCCH and PRACH requirement for UL MIMO with 2Tx - Antenna configuration for incremental test - Option 1:2T2R - Option 2: 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point - Option 3: 2T2R with low correlation - Propagation condition for incremental test - For incremental test, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS 38.101-4 can be taken as the reference - Capture the static channel matrix in spec - Bandwidth & SCS for incremental test - 5MHz for FDD 15KHz SCS and 10MHz for TDD 30KHz SCS - Mapping type for incremental test - Mapping type A - DMRS configuration for incremental test - Rel-15 DMRS type - Rank for incremental test - Rank 2 - MCS for incremental test - 16QAM table 1 MCS 12, 64QAM Table 1 MCS 29, and [256 QAM Table 2 MCS 20] for initial simulation - Only one MCS will be used - TMPI for incremental test - TMPI index 0 - Others - Existing configuration as Rel-15 BS PUSCH demodulation requirement with 2Tx

In this contribution, the view on remaining issue on test setup of BS demodulation requirement for ATG is provided, meanwhile, the initial simulation results on feasible MCS are provided for alignment purpose.

# 2	Test setup

In this section, the remaining issue on test setup for BS demodulation for ATG is discussed.

Number of Rx

For legacy requirement, different antenna configurations with 2Rx/4Rx/8Rx were considered. As agreed, limited new test cases could be considered for ATG 2Tx PUSCH demodulation.  For Rel-19 ATG scenario, the main purpose is to test UL MIMO with ATG channel condition, where AWGN channel and frequency offset was considered.  For different number of Rx, the baseband processing should be same as TN BS. Meanwhile, only 2Rx requirement for Rel-18 ATG with single Tx transmission was specified. To reduce the test effort, for the dedicated requirement with UL MIMO for ATG scenario, our preference is to only consider 2Rx for requirement with the following channel matrix

.

Proposal 1:  Only consider 2Rx for the incremental test of UL MIMO with 2Tx for ATG scenario

Test applicability rule

In the last meeting, RAN4 agreed that only the applicable PUSCH test cases in Clause 8.2.1 with 2Tx 2 layer will be applied for ATG. Similar as the test applicability rule for 1Tx, the following test applicability rule for 2Tx can be considered,

- Unless otherwise stated, PUSCH requirement tests in clause 8.2.1 with 2Tx 2-layer test cases shall apply only for BS declared to be supported (see D.123 in table 4.6-1)

Proposal 2:  The following test applicability rule for 2Tx can be considered for ATG scenario

- Unless otherwise stated, PUSCH requirement tests in clause 8.2.1 with 2Tx 2-layer test cases shall apply only for BS declared to be supported (see D.123 in table 4.6-1)

# 3	Simulation Results

In this section, the initial simulation results are provided based on agreed simulation assumption in the last meeting.

Table 1: Initial simulation results for PUSCH with 2Tx/2Rx,

| Case | Tx/Rx | SCS | BW | Channel | Frequency offset | Mapping Type | DMRS | MCS | SNR@70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2Tx/2Rx | 15KHz | 5MHz | AWGN | 200Hz | A | (2,11) | MCS 12 table 1 | 6.247 |
| 2 | 2Tx/2Rx | 15KHz | 5MHz | AWGN | 200Hz | A | (2,11) | MCS 20 table 1 | 12.839 |
| 3 | 2Tx/2Rx | 15KHz | 5MHz | AWGN | 200Hz | A | (2,11) | MCS 20 table 2 | 19.505 |
| 4 | 2Tx/2Rx | 30KHz | 10MHz | AWGN | 500Hz | A | (2,11) | MCS 12 table 1 | 6.351 |
| 5 | 2Tx/2Rx | 30KHz | 10MHz | AWGN | 500Hz | A | (2,11) | MCS 20 table 1 | 13.035 |
| 6 | 2Tx/2Rx | 30KHz | 10MHz | AWGN | 500Hz | A | (2,11) | MCS 20 table 2 | 21.172 |

Observation 1: The achievable SNR for 256QAM (MCS 20 table 2) is large than 20dB for 2Tx/2Rx configuration

Based on our initial results, the achievable SNR for 256QAM (MCS 20 table 2) is large than 20dB, it maybe not testable considering the current OTA test limitation with SNR is not large than 20dB. Therefore, we prefer to define PUSCH requirements with MCS 20 table 1 for 2Tx2Rx antenna configuration.

Proposal 3:  Only consider MCS 20 with table 1 for the incremental test of UL MIMO with 2Tx for ATG scenario

# 4	Conclusion

In this contribution, the view on remaining issue of BS demodulation requirement with UL MIMO with 2Tx for ATG was provided, meanwhile, the initial simulation results with different MCS were provided.

Proposal 1:  Only consider 2Rx for the incremental test of UL MIMO with 2Tx for ATG scenario

Proposal 2:  The following test applicability rule for 2Tx can be considered for ATG scenario

- Unless otherwise stated, PUSCH requirement tests in clause 8.2.1 with 2Tx 2-layer test cases shall apply only for BS declared to be supported (see D.123 in table 4.6-1)

Observation 1: The achievable SNR for 256QAM (MCS 20 table 2) is large than 20dB for 2Tx/2Rx configuration

Proposal 3:  Only consider MCS 20 with table 1 for the incremental test of UL MIMO with 2Tx for ATG scenario

# References

R4-2508724, Way Forward for [115][328] NR_ATG_enh_demod, CMCC