3GPP TSG-RAN WG4 Meeting #116            	R4-2509411

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.17.3

Source:	Samsung

Title:	View on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In the last RAN4#115 meeting, CSI reporting requirement framework for CSI prediction was under discussed. The related agreed agreements are captured in the WF [1] as following

- CSI reporting requirement framework for CSI prediction - Requirement considerations - The performance of AI based CSI prediction should not worse than R16 etype II - The exact test procedure and configurations are FFS - The impact of different timeline between AI and Rel16 eType II should be also carefully analyzed - Doppler handing - De-prioritize 100Hz Doppler in Rel-19 - N4>1 can be further discussed in the future if needed - SNR levels for training data and generalization - No need to capture anything explicit in RAN4 about training data in the spec - For simulation calibration, SNR range alignment for training can be discussed separately - UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS - LCM requirements - Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - CSI prediction - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring - Deactivation delay for CSI prediction - The following questions were noted for further discussion - Can NW differentiate between the predicted CSI reporting and the measured CSI reporting - In case of switch between AI, is there existing reporting to differentiate the reports before and after the deactivation - Generalization - The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case - Study and, if feasible, define requirements for each AI/ML functionality - Define one test per UE capability as a minimum - FFS > 1 test per UE capability - Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML enabled feature - Potential areas to consider for generalization testing - gNB array parameters - port layouts, array size, antenna virtualization - propagation conditions - Deployment scenarios - Carrier frequencies - Speeds - Indoor/outdoor - Bandwidths - SNR

In this contribution, the view on the remain issue of CSI reporting requirement and testing framework for CSI prediction is provided.

# 2	LCM related requirement

Regarding the LCM requirement, in the last meeting, serval options are considered to be further discussed.

- Option 1: Wait for further progress in RAN1/2 before continuing the discussion in RAN4 - Option 2: Define requirements for performance monitoring - For type 3 monitoring use intermediate KPI (SGCS) - Introduce requirements for activation/deactivation - Is there any difference compared to RRC configuration - Option 3: RAN4 to discuss the testability of monitoring reporting options, and RAN4 could inform RAN1 if RAN1 does not make decision - Option 4: RAN4 should start discussing where/how to capture LCM requirements - Use 38.133 spec

In the last RAN1 meeting, the following about LCM procedure was agreed

Agreement in RAN1#121 For CSI prediction using UE-side model, for UE assisted performance monitoring - Support to reuse CSI framework for the configuration for monitoring result report in L1 signalling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported. Agreement in RAN1#121 For CSI prediction using UE-side model, for UE assisted performance monitoring - If the interference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report Agreement in RAN1#121 For CSI prediction using UE-side model, for UE assisted performance monitoring - One SGCS is calculated based on predicted CSI for one interference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - Wideband frequency granularity - Only for one prediction instance configured by NW - Per layer, and the total number of layers is the same as the reported RI in the associated interference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19 - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codeprint is used to indicate the value of range of (, 0.3) - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) Agreement in RAN1#121 For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCS SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

Based on RAN1 agreement, with type 3 monitoring, UE will report the performance metric to network, based on the configured resources for monitor and report, as well the prediction instance. For performance monitoring metric report, both semi-persistent and aperiodic can be supported. For report value, the SGCS will be used and is quantized with 4-bit. For each report, two SGCS are included to reflect the Quantity reported, including the SGSC of predicted CSI and ground truth CSI, and the SGCS of non-predicted CSI and ground truth CSI. RAN1 has defined the quantized value with 4 bits for SGCS reporting, and also reuse CSI framework for the configuration for monitoring in L1 signaling, we think the performance monitoring accuracy requirements for CSI prediction. While how to test in RAN4 specification should be further discussed based on the reported SGCS, since RAN4 has never to the test the accuracy of the reported PMI.

Observation 1:  For performance monitoring metric report, SGCS will be used and is quantized with 4-bit. For each report, two SGCS are included to reflect the Quantity reported.

Proposal 1:  RAN4 should discuss the testability of performance monitoring accuracy with SGCS based on RAN1 definition for CSI prediction

Activation delay

Regarding the activation delay requirements, RAN4 has agreed it was created for periodic, semi-persistent and aperiodic CSI reporting for beam prediction. For CSI prediction, it is FFS, and the detail is subject to RAN1 decision. Based on RAN1 agreement, we think the activation delay requirements should be created for semi-persistent and aperiodic CSI reporting for CSI prediction

Proposal 2:  Activation delay requirements created for semi-persistent and aperiodic CSI reporting for CSI prediction

Deactivation delay for CSI prediction

Regarding the deactivation delay requirement for CSI prediction, the following questions are noted for further discussion

- Can NW differentiate between the predicted CSI reporting and the measured CSI reporting - In case of switch between AI, is there existing reporting to differentiate the reports before and after the deactivation

When gNB configure CSI report with AI/ML-based CSI prediction, UE needs to report the performance monitoring in addition to CSI report. Based on performance monitoring report, gNB will deactivate the AI/ML CSI prediction function if the reporting quantity is less than a threshold. Then, UE will switch to another AI/ML CSI prediction function. With switching AI/ML model, it is expected to the SGCS including in the performance monitoring will be improved before and after the deactivation operation.

Meanwhile, RAN1 has introduced new RRC parameter for report quantity, e.g., ‘SGCS-r19’. Since RAN1 has agreed to support reuse the CSI framework for the configuration for monitoring results report in L1 signaling. And the ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration.

Therefore, for question 1, in our understanding, NW can use the ID of interference report configured in the configuration for monitoring to differentiate the predicated CSI reporting and the measured CSI reporting

Therefore, for question 2, in cases of switching between AI, in our understanding, the existing performance monitoring report can be used to differentiate the reports before and after the deactivation.

Observation 3:  NW can use the ID of interference report configured in the configuration for monitoring to differentiate the predicated CSI reporting and the measured CSI reporting.

Observation 4:  The existing performance monitoring report can be used to differentiate the reports before and after the deactivation

# 3	Generalization

In the last meeting, the high level and general guidelines for generalization test are provided.

For generalization testing, the purpose is to guarantee the AI model training on specific scenario/condition can still work well in other scenario/condition.

Firstly, we would like to clarify whether the generalization testing will be tested in one case with mixed different configuration, TE will change the test configurations with different TTIs, or to be tested in multiple case for different configuration.

Proposal 3:  RAN4 should clarify how the generalization testing will be conducted, whether it will be tested in one case with multiple scenario combination, or it will be tested in multiple cases with different configurations

As agreed, RAN4 will define one test per UE capability as a minimum, and FFS >1 test per UE capability. Regarding the UE capability, RAN1 has still discussed the UE feature list for AI/ML functionally. From RAN4 demodulation test perspective, to minimize the test effort, only one test will be specified per UE capability. Different with legacy non-AI test cases, since it needs to guarantee the generalization of AI/ML model, we think it is meaningful to define multiple cases with different configuration. As clarified in our proposal 1, if the one test is conducted with multiple test configurations, then, one case should be enough. Otherwise, we think multiple cases with different configurations are needed.

Proposal 4:  If one test case is conducted with multiple test configurations, one case per UE capability should be enough. If one test case is conducted with individual test configuration, multiple cases with different configurations can be considered.

Regarding the potential areas to consider for generalization testing, several aspects are listed for further discussion.

Static and non-Static

Regarding the static and non-static scenario, the following was agreed in RAN4#110bis meeting

- Testing environment/framework - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenario/configurations as baseline - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR setting are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other condition

From RAN4 PMI test requirement perspective, it was usually to define with the specific channel models and different SNR levels.

For AI based CSI prediction, in terms of functionality test, we think it should be enough with static scenario/ configuration similar as legacy non-AI based CSI prediction. While considering the generalization test, the AI model training on specific scenario/condition should be guaranteed to work well in other scenario/condition. Therefore, it should be also beneficial to consider the non-static scenario/configuration to check the generalization of the model.

Regarding the non-static for CSI prediction, the key issue is how to introduce the non-static scenario/configuration during the demodulation test with combining multiple parameters. Another alternative solution for considering non-static conditions is to define requirements with multiple scenarios under different condition, where UE can train multiple models corresponding to specific conditions

Proposal 5:  If non-static scenario/configuration introduced, RAN4 should discuss how to perform the test with different parameters during one test setup.

Indoor/outdoor

As for indoor and outdoor, it will impact the pathloss and some change parameters. Since RAN4 has agreed to apply the TDL channel will be used for CSI prediction testing in the previous meeting, while from RAN4 demodulation requirement perspective, there is no dedicated modeling the TDL channel model for indoor scenario and outdoor scenario. Therefore, we think there is no need the generalization testing with related aspects for indoor/outdoor

Proposal 6:  There is no need the generalization testing related with indoor/outdoor scenario.

Channel Bandwidth

As for different bandwidth, from RAN4 UE demodulation and CSI reporting requirement aspects, only selecting the typical channel bandwidth for requirement definition. Since the CSI prediction is to track the channel variation in the time domain. The process should be same regardless different channel bandwidth. Therefore, there is no need the generalization testing related with channel bandwidth

Proposal 7:  There is no need the generalization testing related with channel bandwidth

Propagation conditions

For CSI prediction, the main purpose of CSI prediction is to track the channel variation and correlation in the time domain considering the doppler impact. In general, although the CDL based channel model was applied in RAN1 model performance evaluation, we think TDL should be enough to test the AI/ML functionality.  For CDL channel model, it can provide more information about the spatial domain, it should be beneficial to check the model generalization with considering different spatial correlation level.  Therefore, to ensure the model generalization, the CDL based channel model can be considered.

However, from RAN4 demodulation perspective, CDL-based channel models are not usually used. Typically, TDL based channel conditions were used for requirement definition in the demodulation tests.

In Rel-19, the SI on spatial channel model (SCM) for demodulation requirement was under discussion. The WID of this SI was captured as following

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e., up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

Based on the SID, both CDL and TDL based channel modeling was under studied. According to current progress, the limited scenarios are identified to be studied. Meanwhile, the testability and test repeatability for CDL channel model on demodulation and CSI testing should be further studied.

Observation 5:  Rel-19 SCM channel model study is expected to only focus on limited scenarios. The testability and test repeatability for CDL channel model on demodulation and testing should be further studied

Therefore, the performance impact on AI/ML based CSI prediction with CDL channel model should be further studied. It was relied on the stable and testable CDL channel model for RAN4 demodulation requirements

Meanwhile, only technical report was released for this SI in Rel-19, there is no specific CDL model introduced in the specification of Rel-19.

Based on above observations, in our understanding, it is better that TDL based channel model should be prioritized for Rel-19 AI based CSI prediction testing. Further study the performance impact on AI CSI prediction with spatial channel model (e.g., CDL based channel model, or TDL based extended channel model) in Rel-20 pending on the progress of SCM channel model study in Rel-19 and specific CDL channel model in Rel-20.

Proposal 8:  Prioritize to use TDL channel model for AI/ML based CSI prediction for generalization testing in Rel-19. Further study the performance impact on AI/ML based CSI prediction with spatial channel model (e.g., CDL based channel model, or TDL based extended channel model) in Rel-20 pending on the progress of SCM channel model study in Rel-19 and the specific channel model in Rel-20.

gNB array parameters

Regarding the gNB array parameters, such as port layouts, array size, and antenna virtualization, the following is agreement in RAN1 evaluation

Agreement in RAN1#118bis For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configuration are not precluded - Note: for other evaluation assumptions, reuse baseline of TR 38.843 - Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

As evaluated by companies, performance evaluation of UE-side CSI prediction with respect to data distribution mismatch that may arise from NW-side additional condition, considering TRP antenna setting with different TRXU mapping.  RAN1 has considered and studied some NW-side additional conditions indication. Therefore, we think RAN4 should generate the training data by using the NW-side condition specified for test. For different gNB array parameters, such as number of Ports, RAN4 may need to define the multiple cases

Proposal 9:  RAN4 should generate the training data with the same gNB array parameters (NW-side) specified for test. For different gNB array parameters, RAN4 may need to define the multiple cases.

UE speed and carrier frequency

Regarding the different UE speed and carrier frequency, during RAN1 evaluation, various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h) and carrier frequencies (e.g., 2GHz, 3.5GHz) were considered. In legacy RAN4 demodulation, the test cases were specified targeting different doppler spread, which has already covered different UE speeds and carrier frequencies implicitly.

For AI/ML based CSI prediction, according to the simulation assumption in [3], several values of doppler spread are agreed for evaluation, including 20Hz, 50Hz, 100Hz. As shown in our paper [4], the SGCS performance of AI/ML based CSI prediction is significantly better than the nearest historical CSI scheme for all the cases as following, while there is a generalization problem for AI/ML based CSI prediction. It requires further investigation

Table 1 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 2, Observation window (5/5ms) based on non-mixed dataset

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  |  |  |  | Non-AI Benchmark#1 (sample and hold) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20Hz |  | 50Hz |  | 100Hz |  |  |  |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 20Hz | 0.9999 | 0.9997 | 0.9669 | 0.9403 | 0.3242 | 0.1669 | 0.7922 | 0.6623 |
| 50Hz | 0.7117 | 0.5187 | 0.9298 | 0.8836 | 0.2989 | 0.1843 | 0.3332 | 0.2020 |
| 100Hz | 0.1727 | 0.1379 | 0.1798 | 0.1348 | 0.1929 | 0.1424 | 0.1878 | 0.1545 |

Observation 6:  AI/ML based CSI prediction shows better performance compared with the nearest historical CSI scheme, while generalization problem related different doppler needs further investigated

To handle the generalization, the mixed dataset is agreed in previous meeting. With including the data from different doppler values for AI/ML model training, as shown in table 2, although there is a performance degradation compared to the model training the specific doppler scenario, the good generalization can be achieved for different doppler scenarios.

Table 2 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 2, Observation window (5/5ms) based on mixed dataset

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  | Non-AI Benchmark#1 (sample and hold) |  |
| --- | --- | --- | --- | --- |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 20Hz | 0.99920948 | 0.99870243 | 0.7922 | 0.6623 |
| 50Hz | 0.92254627 | 0.87335623 | 0.3332 | 0.2020 |
| 100Hz | 0.17877519 | 0.12702188 | 0.1878 | 0.1545 |

Observation 7: With including the data from different doppler values for AI/ML model training, good generalization can be achieved for different doppler scenario, although there is a performance degradation

Based our observations and simulation results, it is necessary to define the requirements with considering different doppler scenario to guarantee the generalization of AI/ML based CSI prediction

Proposal 10:  RAN4 can define requirements with considering different scenarios to guarantee the generalization of AL/ML based CSI prediction.

SNR condition

Regarding the generalization with different SNR condition, in our understanding, the selected SNR for testing depends on the selected MCS. Based on agreement, UE performance at more than one MCS should be tested. Therefore, to sure the generalization of SNR, RAN4 can define the requirement with different MCS, targeting for different SNR range. Since the purpose of AI/ML based CSI prediction is to track the channel correlation property and prediction the channel information in the future slots instead of denoising, the specific SNR range associated with MCS for generalization test should be further discussed.

Beside MCS, another alternative solution is to check the SNR value associated with X% of maximum throughout, beside the 90%, for example, X can be considered 30% or 70%.

Proposal 11:  RAN4 can consider to define different MCS requirement targeting different SNR range. The specific SNR range associated with MCS for generalization test should be further discussed. Another alternative solution is to check the SNR value associated with X% of maximum throughput, beside the 90%, for example, X can be considered 30% or 70%.

# 4	Conclusion

In this contribution, the view on the remaining issue of CSI reporting requirement and testing framework for CSI prediction was provided with following observations and proposals

Observation 1:  For performance monitoring metric report, SGCS will be used and is quantized with 4-bit. For each report, two SGCS are included to reflect the Quantity reported.

Proposal 1:  RAN4 should discuss the testability of performance monitoring accuracy with SGCS based on RAN1 definition for CSI prediction

Proposal 2:  Activation delay requirements created for semi-persistent and aperiodic CSI reporting for CSI prediction

Observation 3:  NW can use the ID of interference report configured in the configuration for monitoring to differentiate the predicated CSI reporting and the measured CSI reporting.

Observation 4:  The existing performance monitoring report can be used to differentiate the reports before and after the deactivation

Proposal 3:  RAN4 should clarify how the generalization testing will be conducted, whether it will be tested in one case with multiple configurations combination, or it will be tested in multiple cases with individual configurations

Proposal 4:  If one test case is conducted with multiple test configurations, one case per UE capability should be enough. If one test case is conducted with individual test configuration, multiple cases with different configurations can be considered.

Proposal 5:  If non-static scenario/configuration introduced, RAN4 should discuss how to perform the test with different parameters during one test setup.

Proposal 6:  There is no need the generalization testing related with indoor/outdoor scenario.

Proposal 7:  There is no need the generalization testing related with channel bandwidth

Observation 5:  Rel-19 SCM channel model study is expected to only focus on limited scenarios. The testability and test repeatability for CDL channel model on demodulation and testing should be further studied

Proposal 8:  Prioritize to use TDL channel model for AI/ML based CSI prediction for generalization testing in Rel-19. Further study the performance impact on AI/ML based CSI prediction with spatial channel model (e.g., CDL based channel model, or TDL based extended channel model) in Rel-20 pending on the progress of SCM channel model study in Rel-19 and the specific channel model in Rel-20.

Proposal 9:  RAN4 should generate the training data with the same gNB array parameters (NW-side) specified for test. For different gNB array parameters, RAN4 may need to define the multiple cases.

Observation 6:  AI/ML based CSI prediction shows better performance compared with the nearest historical CSI scheme, while generalization problem related different doppler needs further investigated

Observation 7: With including the data from different doppler values for AI/ML model training, good generalization can be achieved for different doppler scenario, although there is a performance degradation

Proposal 10:  RAN4 can define requirements with considering different scenarios to guarantee the generalization of AL/ML based CSI prediction.

Proposal 11:  RAN4 can consider to define different MCS requirement targeting different SNR range. The specific SNR range associated with MCS for generalization test should be further discussed. Another alternative solution is to check the SNR value associated with X% of maximum throughput, beside the 90%, for example, X can be considered 30% or 70%.

# References

R4-2508080, WF on AI/ML for NR air interface1

R4-2508051, WF on AI/ML for NR Air Interface2

R4-2508102, Simulation assumption for CSI prediction

R4-2505973, View on demodulation requirement for AI based CSI prediction, Samsung