3GPP TSG-RAN WG4 Meeting #116	R4-2509418

# Bengaluru, India, 25th – 29th August 2025

Agenda item:			6.14.2.1

Source:	Nokia

Title:	On 4940-4990 MHz band regulations for adjacent spectrum protection

Document for:	Discussion

# Introduction

During RAN#108, new WID has been approved on introduction of NR TDD 4.9GHz Band for US Operation [1]. The normative work on this band will start after completion of the following study phase:

- Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band.

    - Identify adjacent spectrum protection requirements

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

    - Identify other technologies operating in the 4940-4990 MHz band

      - Identify potential coexistence mechanism(s)

        - Frequency division sharing

        - Geographic separation

        - TDD synchronization

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

This document addresses regulatory requirements related to adjacent spectrum protection.

# Discussion

Regulations Governing Licensing and Use of Frequencies in the 4940-4990 MHz Band for US operation are available in [2]. Below are specific details related to future RAN4 work on this band and adjacent spectrum protection (it should be noted there is no specific maximum conducted output power limit for 25 MHz channel bandwidth which is currently considered in [1]):

§ 90.1213 Band plan.

(a) The following table lists center frequencies for channels in the 4940-4990 MHz band. Channel numbers 1 through 5 and 14 through 18 are 1 MHz bandwidth channels, and channel numbers 6 through 13 are 5 MHz bandwidth channels.

| Center frequency (MHz) | Bandwidth (MHz) | Channel numbers |
| --- | --- | --- |
| 4940.5 | 1 | 1 |
| 4941.5 | 1 | 2 |
| 4942.5 | 1 | 3 |
| 4943.5 | 1 | 4 |
| 4944.5 | 1 | 5 |
| 4947.5 | 5 | 6 |
| 4952.5 | 5 | 7 |
| 4957.5 | 5 | 8 |
| 4962.5 | 5 | 9 |
| 4967.5 | 5 | 10 |
| 4972.5 | 5 | 11 |
| 4977.5 | 5 | 12 |
| 4982.5 | 5 | 13 |
| 4985.5 | 1 | 14 |
| 4986.5 | 1 | 15 |
| 4987.5 | 1 | 16 |
| 4988.5 | 1 | 17 |
| 4989.5 | 1 | 18 |

(b) The channels listed in the table in paragraph (a) of this section may be aggregated in any manner up to 50 MHz for wider bandwidth operation. Nonetheless, applicants should request no more bandwidth than necessary for a particular use.

§ 90.1215 Power limits.

Except as provided in paragraph (f) of this section, the transmitting power of stations operating in the 4940-4990 MHz band must not exceed the maximum limits in this section.

(a)

(1) For base, mobile, and temporary fixed operations, the maximum conducted output power must not exceed:

| Table 1 to Paragraph (a)(1) |  |  |
| --- | --- | --- |
| Channel bandwidth (MHz) | Low power maximum conducted output power (dBm) | High power maximum conducted output power (dBm) |
| 1 | 7 | 20 |
| 5 | 14 | 27 |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

(2) High power devices are also limited to a peak power spectral density of 21 dBm per one MHz. High power devices using channel bandwidths other than those listed above are permitted; however, they are limited to peak power spectral density of 21 dBm/MHz. If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the peak power spectral density should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi. However, high power point-to-point and point-to-multipoint operations (both fixed and temporary-fixed rapid deployment) may employ transmitting antennas with directional gain up to 26 dBi without any corresponding reduction in the maximum conducted output power or spectral density. Corresponding reduction in the maximum conducted output power and peak power spectral density should be the amount in decibels that the directional gain of the antenna exceeds 26 dBi.

(b) Low power devices are also limited to a peak power spectral density of 8 dBm per one MHz. Low power devices using channel bandwidths other than those listed above are permitted; however, they are limited to a peak power spectral density of 8 dBm/MHz. If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the peak power spectral density should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi.

(c) The maximum conducted output power is measured as a conducted emission over any interval of continuous transmission using instrumentation calibrated in terms of an RMS-equivalent voltage. If the device cannot be connected directly, alternative techniques acceptable to the Commission may be used. The measurement results shall be properly adjusted for any instrument limitations, such as detector response times, limited resolution bandwidth capability when compared to the emission bandwidth, sensitivity, etc., so as to obtain a true maximum conducted output power measurement conforming to the definitions in this paragraph for the emission in question.

(d) The peak power spectral density is measured as conducted emission by direct connection of a calibrated test instrument to the equipment under test. If the device cannot be connected directly, alternative techniques acceptable to the Commission may be used. Measurements are made over a bandwidth of one MHz or the 26 dB emission bandwidth of the device, whichever is less. A resolution bandwidth less than the measurement bandwidth can be used, provided that the measured power is integrated to show total power over the measurement bandwidth. If the resolution bandwidth is approximately equal to the measurement bandwidth, and much less than the emission bandwidth of the equipment under test, the measured results shall be corrected to account for any difference between the resolution bandwidth of the test instrument and its actual noise bandwidth.

(e) The ratio of the peak excursion of the modulation envelope (measured using a peak hold function) to the maximum conducted output power shall not exceed 13 dB across any 1 MHz bandwidth or the emission bandwidth whichever is less.

(f) The transmitting power of permanent fixed point-to-point and point-to-multipoint stations operating in the 4940-4990 MHz band must not exceed the maximum limits in this paragraph (f). Moreover, applicants should request no more power than necessary for a particular use.

(1) The maximum equivalent isotropically radiated power (EIRP), as referenced to an isotropic radiator, must not exceed 55 dBW (85 dBm).

(2) For path lengths shorter than 17 kilometers, the EIRP shall not exceed the value derived from the following equation: New EIRP limit = 55 dBW—40*log(17/B) dBW, where B = the actual path length in kilometers.

Furthermore, to guarantee adjacent spectrum protection, the following emissions masks (L for low power and M for high power transmitters) apply [3]:

(l) Emission Mask L. For low power transmitters (20 dBm or less) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows:

(1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.

(2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 219 log (% of (BW)/45) dB.

(3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 10 + 242 log (% of (BW)/50) dB.

(4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 20 + 31 log (% of (BW)/55) dB attenuation.

(5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 28 + 68 log (% of (BW)/100) dB attenuation.

(6) On any frequency removed from the assigned frequency above 150% of the authorized bandwidth: 40 dB.

(7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

Emission Mask M. For high power transmitters (greater that 20 dBm) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows:

(1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.

(2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB.

(3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB.

(4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB.

(5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB.

(6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation.

(7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

# Conclusion

This document addresses regulatory requirements related to introduction of 4.9 GHz band for US operation (including adjacent spectrum protection) which should be taken into account for future RAN4 work on this band.

# Reference

[1] RP-251887; New WID on Introduction of NR TDD 4.9GHz Band for US Operation; Fujitsu

[2] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y

[3] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I