3GPP TSG RAN WG4 #116															     R4-2509421

Bengaluru, India, August 25h – 29th 2025

Agenda Item: 		7.26.3

Source:			 	InterDigital Belgium LLC.

Title:                      NR_XR_Ph3 remaining open issues

Document for: 		Discussion and decision

# Background

In the last RAN4 meeting #115, the discussions on this topic progressed. As a result, a WF has been agreed in the meeting in [1].

# Discussion

In this contribution, we continue our analysis on gap skipping measurements occasions for NR XR case and the NW scheduling mechanism.

In RAN4 meeting #115, the agreed WF in [1] treats a variety of topics and a certain number of issues still need to be sorted out. We will address them in the following sections.

# types of measurements with gaps

In the last meeting, the following agreements were made:

## Issue 2-1: Type 2 gaps <Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms

The FFS bullet from the above agreement remains to be settled. If a rule is to be defined, then our proposal would be that a gap cancelation would apply for the earliest gap that respects the “X ms” distance from the DCI reception, but not to the second gap that is less than “X ms” from the first one. In our opinion, the current RAN1 agreed CR in [3] for TS38.213 interpretation leads to such a conclusion as can be seen below:

## *** Unchanged parts are omitted *** ## 10.6 Indication for cancelation of RRM measurement gaps/restrictions A measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]. A UE can be indicated by a value ‘1’ of a measurement gap cancellation field in a DCI format provided by a PDCCH reception to cancel an associated RRM measurement gap occasion. If a UE indicates “Option 1” for XYZ, the UE ignores a value ‘0’ of the measurement gap cancellation field in a DCI format. If a UE indicates “Option 2” for XYZ, a value ‘0’ of the measurement gap cancellation field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the associated RRM measurement gap occasion is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the measurement gap cancellation field and are associated with a same associated RRM measurement gap occasion, the UE applies the indication provided by the measurement gap cancellation field of a last DCI format, from the more than one DCI formats. ## *** Unchanged parts are omitted ***

Observation 1: The RAN1 agreed CR indicates that the DCI with the cancelation order applies to the earliest MG occasion that starts at least at 3ms or 5ms depending on UE capability.

If a rule is to be defined for this case, then our proposal is:

Proposal 1: If two non-colliding concurrent gaps are separated by less than “X ms”, a gap cancelation would apply for the earliest measurement gap occasion that respects the “X ms” distance from the DCI reception, but not to the second gap that is less than “X ms” from the first one.

# XR skipping requirements impact

The follow up was to have some equations for the delay calculation and three options have been retained:

Issue 3-1: Formulas for measurement delay extension without DRX <Way forward> - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) x Max(MGRP , SMTC period) ) x CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter

In the last meeting, the above 3 options were discussed and even though Option 1 has a more balanced approach when measuring different frequency carriers, and leading to a reasonable delay, it was claimed that it is difficult to implement. Same thing for Option 3, hard to implement.

On the other hand, Option 2 would lead to long delays. As we don’t have a very strong opinion, we can support Option 2 if an improvement is added to limit the delay issue. This means that a maximum measurements extension should be defined, so the system would limit naturally such issues.

Observation 2: Option 2 requires additional constraints to limit the possible long delays.

Proposal 2: Support Option 2 with some additional requirements that would limit the long delay issue:

- Option 2: Max( 200 ms, ( 8 + Nskip ) x Max(MGRP , SMTC period) ) x CSSFinter

Another issue that needs to be resolved is about the DRX cases for measurement delay extension:

## Issue 3-2: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle - Other options are not precluded

We don’t have a strong opinion, but it seems that Option 1 provides a more intuitive reading for the requirement and its purpose.

Observation 3: If Option 2 is agreed then the same approach shall be followed for the DRX case.

Proposal 3: Support Option 2:

  - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

# Conditions for measurements skipping

In the last meeting the following options were retained as a way forward:

<Way forward>: - Option 1: No conditions are defined - Option 2: Define the maximum number of gaps skipped during the measurement period. - Option 3: Define a maximum separation between two non-skipped gaps for measurement

If the supported option for delay calculation follows Option 2, we believe that a supplementary requirement would be beneficial to limit the delays and ensure measurements performance. In the last meeting several options were suggested as can be seen in [2] with quantitative impact.

Observation 4: A supplementary requirement would be beneficial to limit the measurements delay issues and ensure measurements performance.

Proposal 4: Further discuss the following proposal presented in the last meeting:

    - The maximum number of extensions are

|  | PSSS/SSS detection | SSB index detection | Measurement period |
| --- | --- | --- | --- |
| No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms | FR1: 12 FR2: 1.5×Mpss/sss_sync | FR1: 5 FR2: 1.5×MSSB_index | FR1: 12 FR2: 1.5×Mmeas_period |
| DRX cycle ≤ 320 ms | FR1: 8 FR2: Mpss/sss_sync | FR1: 3 FR2: MSSB_index | FR1: 8 FR2: Mmeas_period |
| DRX cycle > 320 ms | FR1: 5 FR2: 0.5×Mpss/sss_sync | FR1: 2 FR2: 0.5× MSSB_index | FR1: 5 FR2: 0.5× Mmeas_period |

    - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

    - Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

    - Upon exceeding Xmeas,max the UE shall restart the measurement.

# Conclusions

This contribution discussed our analysis related to measurements occasions skipping for NR XR case. Based on the discussion, the following observations and proposals are made:

Observation 1: The RAN1 agreed CR indicates that the DCI with the cancelation order applies to the earliest MG occasion that starts at least at 3ms or 5ms depending on UE capability.

If a rule is to be defined for this case, then our proposal is:

Proposal 1: If two non-colliding concurrent gaps are separated by less than “X ms”, a gap cancelation would apply for the earliest measurement gap occasion that respects the “X ms” distance from the DCI reception, but not to the second gap that is less than “X ms” from the first one.

Observation 2: Option 2 requires additional constraints to limit the possible long delays.

Proposal 2: Support Option 2 with some additional requirements that would limit the long delay issue:

- Option 2: Max( 200 ms, ( 8 + Nskip ) x Max(MGRP , SMTC period) ) x CSSFinter

Observation 3: If Option 2 is agreed then the same approach shall be followed for the DRX case.

Proposal 3: Support Option 2:

  - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

Observation 4: A supplementary requirement would be beneficial to limit the measurements delay issues and ensure measurements performance.

Proposal 4: Further discuss the following proposal presented in the last meeting:

    - The maximum number of extensions are

|  | PSSS/SSS detection | SSB index detection | Measurement period |
| --- | --- | --- | --- |
| No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms | FR1: 12 FR2: 1.5×Mpss/sss_sync | FR1: 5 FR2: 1.5×MSSB_index | FR1: 12 FR2: 1.5×Mmeas_period |
| DRX cycle ≤ 320 ms | FR1: 8 FR2: Mpss/sss_sync | FR1: 3 FR2: MSSB_index | FR1: 8 FR2: Mmeas_period |
| DRX cycle > 320 ms | FR1: 5 FR2: 0.5×Mpss/sss_sync | FR1: 2 FR2: 0.5× MSSB_index | FR1: 5 FR2: 0.5× Mmeas_period |

    - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

    - Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

    - Upon exceeding Xmeas,max the UE shall restart the measurement.

# References

- R4-2508285 “WF on RRM requirements for XR_Ph3, RAN4 Meeting #115, St-Julian’s, Malta, May 2025

- R4-2508284 “Ad-hoc minutes for NR_XR_PH3”, Nokia, RAN4 Meeting #115, St-Julian’s, Malta, May 2025

- R1-2505064, CR - Introduction of XR (eXtended Reality) for NR Phase 3, Rel-19, St Julisan’s, Malta, May 2025