3GPP TSG-RAN WG4 Meeting #116	R4-2509423

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.2

Source:	Apple

Title:	On general issues on AI/ML

Document for:	Discussion

# 1	Introduction

In this contribution, we further discuss the remaining issues related to:

- post-deployment pre/post activation functionality testing, and

- LCM requirements.

# 2	Post-deployment pre/post activation functionality testing

As AI/ML models become increasingly integrated into NR (New Radio) air interface functionalities, particularly on the UE side, ensuring their sustained performance post-deployment poses new challenges for 3GPP. After a model is deployed in the field especially when it has been updated or fine-tuned it may no longer be practical or even possible to subject the device to full RAN5 conformance testing again. This creates a need for alternative mechanisms to ensure that the updated model continues to perform reliably and does not degrade the UE's behavior or violate compliance standards.

The critical problem is how to validate AI/ML models after deployment in a way that provides confidence to vendors, operators, and test authorities without introducing unmanageable testing burdens, excessive signaling overhead, or compromising performance monitoring reliability.

During RAN4#114bis, there was not much discussion on the topic, apart from agreeing a naming update.

Agreement:

- Rename “option 1” as “Post-deployment pre-activation functionality/configuration update testing”

- Rename “option 2” as “Post-deployment post-activation functionality testing based on performance monitoring”

## Option 1: Post-Deployment Pre-Activation Functionality / Configuration Update Testing

Option 1 is a validation process performed before activating an AI/ML model update in deployed UEs. The intent is to ensure that any new or modified model still meets the performance expectations established during initial RAN5 conformance testing, taking into account the hardware characteristics of the target UE. This validation can occur in a controlled laboratory environment or within the model development facility, where realistic device behavior is emulated. By doing so, the testing environment can reproduce the hardware constraints, RF front-end behavior, and processing latency that the model will encounter in the field.

The approach depends on defining clear triggers for when pre-activation testing is required. It is important to distinguish between major and minor model changes. Major changes, such as a full retrain or architectural modification, are more likely to require this form of testing, whereas fine-tuning of weights with unchanged architecture might be exempt. However, the criteria for making this distinction remain an open standardization question. Determining whether such classification is standardized or left to the UE vendor has significant implications for enforceability and interoperability.

Another critical consideration in Option 1 is whether the testing process itself is executed on the UE hardware, on a lab replica, or via simulation. Laboratory testing can guarantee controlled conditions but may introduce long turnaround times and significant costs, especially if updates are frequent. On-device validation, by contrast, allows faster iteration but risks variability due to hardware differences and makes independent verification more difficult. These trade-offs are central to deciding the practicality and scalability of Option 1 in commercial deployments

In the post-deployment context, on-device validation refers to testing an updated AI/ML model directly inside the actual UE, either in the field or in a controlled test mode, before it is activated for live use. In this setup, the new model can operate in a “shadow” mode alongside the active model, receiving the same inputs but without influencing live network decisions. The UE collects performance metrics ,such as beam prediction accuracy or RSRP differences , using either pre-loaded test datasets or duplicated live measurements. These KPIs are then evaluated against pre-defined thresholds to decide whether the model can be activated. This method is fast, captures the true behavior of the UE’s RF and processing hardware, and is well-suited for minor updates or fine-tuning where quick validation is important. However, it is difficult for the network or certification bodies to independently verify results, and outcomes may vary between UE hardware versions. It also assumes the UE has enough compute and memory resources to run two models in parallel during validation.

Lab validation, by contrast, is performed in a controlled and reproducible environment , typically in a vendor’s AI/ML development facility or a certification lab, using either the target UE hardware or an accurate hardware emulator. The updated model is tested with standardized datasets and well-defined emulated conditions for channel fading, interference, Doppler, and measurement impairments, often matching the profiles used in RAN5 conformance testing. This allows direct comparison with baseline KPIs from the original conformance stage and ensures the results are verifiable by third parties. Lab validation avoids any risk of degrading live network performance during testing and is easier to standardize across vendors, since the environment and procedures are fixed.

While on-device validation is quicker and more representative of real-world device performance, it lacks the reproducibility and independent oversight of lab-based methods. Lab validation offers strong repeatability and regulatory trust but at the cost of slower turnaround, higher expense, and potentially missing device-specific quirks that only appear in the field.

## Option 2, Post-Deployment Post-Activation Functionality Testing Based on Performance Monitoring (LCM)

Option 2 moves the assurance process into the operational phase, monitoring the model’s performance in real-world conditions after activation. This approach leverages Life Cycle Management (LCM) procedures and relies on continuous or periodic collection of KPIs from the UE. The objective is to detect degradation early, identify root causes, and trigger recovery actions such as model switching, fallback to legacy algorithms, or retraining. The monitoring configuration, e.g., CSI-RS/SSB resource sets, measurement intervals, and thresholds, is signaled by the network and may align with RAN1-defined inference timing.

In beam management, RAN1 has already agreed on a Top-M vs. Top-K beam prediction accuracy metric for UE-assisted monitoring. In other use cases, like CSI prediction or positioning, intermediate KPIs or difference metrics are being considered. However, RAN4’s test metrics and RAN1’s monitoring metrics are not always aligned, which could complicate using monitoring results as a direct proxy for performance validation. Consistency across UE implementations is also a challenge, differences in analogue front-ends, filtering, or averaging methods could make KPI comparisons non-uniform unless RAN4 defines strict reporting requirements.

Option 2,  post-deployment monitoring via LCM, is attractive because it scales well to frequent model updates and evaluates performance in real network conditions without the delays of lab-based testing. However, running continuous monitoring in the field can create challenges: if KPI reporting thresholds are set too tightly, normal stochastic fluctuations in AI/ML outputs may trigger excessive reporting and even unnecessary fallback, increasing both signaling overhead and processing load on the UE and network.

To address this, different companies propose several mitigations: use relaxed thresholds during an initial “validation mode” and tighter thresholds in normal operation, apply multi-M evaluation (e.g., Top-1, Top-2, Top-4 beam accuracy) to enrich KPI data without extra signaling, and temporarily disable fallback in the early validation phase to avoid premature deactivation of a model that is still performing adequately. These measures aim to preserve Option 2’s scalability while reducing false triggers and minimizing overhead.

## Pros and Cons of Option 1 and Option 2

Option 1 offers the advantage of controlled, reproducible testing conditions and the ability to verify model behavior before it impacts live network performance. It avoids the risk of service degradation due to deploying an underperforming model and can be tied directly to conformance-like pass/fail criteria. However, it can be slow, costly, and operationally heavy, especially if frequent updates are expected. Standardizing major/minor change criteria is non-trivial, and on-device validation lacks the transparency and reproducibility of lab testing.

Option 2 enables real-time validation in the actual deployment environment, inherently capturing the effects of real-world conditions such as interference, mobility, and device variability. It scales well to frequent updates and can be integrated into existing LCM frameworks. At the same time, it depends heavily on robust metric design, interoperability of KPI reporting, and careful threshold tuning to avoid false triggers. Without alignment between performance testing and monitoring metrics, Option 2 risks ambiguity in determining whether a performance drop is due to the model update or external factors.

## Proposal Analysis: Hybrid Post-Deployment Validation

### 1. Problem Statement

Post-deployment AI/ML model updates present two competing needs:

- Assurance: Updated models must meet performance expectations comparable to those established in RAN5 conformance testing.

- Agility: The framework must support frequent updates without introducing excessive delay, cost, or complexity.

Option 1 (pre-activation testing) ensures assurance but can be slow, costly, and impractical for frequent updates. Option 2 (post-activation LCM monitoring) is scalable but risks false triggers, overhead, and delayed detection if not carefully designed.

Option 1 (pre-activation testing) ensures assurance but can be slow, costly, and impractical for frequent updates. Option 2 (post-activation LCM monitoring) is scalable but risks false triggers, overhead, and delayed detection if not carefully designed

### 2. Proposal Overview

The hybrid approach combines the strengths of both options:

- Stage 1 : Pre-Activation Testing (Option 1 Layer):
Applied only to major updates (full retrain, architecture change, high-impact fine-tuning, or critical KPI use cases). Testing occurs in either a lab or on-device with a standardized, verifiable procedure. This establishes a baseline KPI envelope for post-activation comparison.

- Stage 2 : Post-Activation Monitoring (Option 2 Layer):
Applied to all updates, including those that underwent Stage 1. Continuous KPI monitoring is configured through LCM, using metrics aligned with RAN1 definitions (e.g., Top-M vs Top-K beam prediction accuracy for beam mgmt).

In the hybrid approach, the post-deployment stage is the overall lifecycle stage after a model is activated in the live network , it covers everything that happens once the model leaves the lab or pre-activation test phase.

We provide an example below:

Example: Hybrid Post-Deployment Validation Process for Five Dedicated Models

### Step 1: Example of Five Dedicated Models

- Model A: Urban Macro, Low Doppler, CDL-D

- Model B: Urban Macro, High Doppler, CDL-D/E

- Model C: Urban Micro, Low Doppler, CDL-A

- Model D: Rural Macro, Low Doppler, CDL-F

- Model E: High-Speed Train, High Doppler, CDL-E

Each model is deployed in the UE with its own KPI envelope, derived from initial RAN5 conformance testing or controlled lab validation.

### Step 2: Major vs. Minor Updates for Dedicated Models

Major update (within-scenario change):

- Full retrain with new architecture or altered output format.

- Inclusion of previously unseen channel model or hardware front-end in that scenario.

- Large dataset expansion that significantly changes the statistical distribution.

- Algorithmic change to the inference pipeline (e.g., adding a new feature extraction block).

Minor update (within-scenario change):

- Fine-tuning with additional data from the same network/operator.

- Small parameter calibration or bug fix.

- Limited dataset refresh without altering distribution characteristics.

### Step 3: Example Update Classification

- Model A: Fine-tuned with new drive-test data in an existing network → Minor.

- Model B: Retrained with new CDL-E traces from a vendor RF front-end not in original dataset → Major.

- Model C: Adjusted normalization for updated power control in a known network → Minor.

- Model D: Expanded training set to include a new rural macro operator with different CDL delay spreads → Major.

- Model E: Architecture change from CNN to Transformer for high-Doppler handling → Major.

### Step 4: Hybrid Approach Execution

A. Pre-Activation Stage (Option 1 Layer)

- Major updates (B, D, E):

  - Lab validation preferred for architecture or RF profile changes (B, E).

  - On-device validation possible for dataset distribution changes without hardware changes (D, if test mode available).

  - Updated KPI envelopes generated after validation.

- Minor updates (A, C):

  - Skip lab stage and proceed directly to activation with immediate post-activation monitoring.

B. Post-Activation Stage (Option 2 Layer)

- All five models enter post-activation LCM monitoring after activation.

- Even major updates validated pre-activation still undergo post-activation monitoring to detect performance drift in real network conditions (channel variability, interference, hardware aging, data drift).

- KPI envelope from pre-activation becomes the baseline for live KPI comparisons, triggering fallback, retraining, or threshold adjustments when deviations occur.

Hybrid logic:

- Major update → Pre-activation validation → Post-activation monitoring (using KPI envelope from validation).

- Minor update → Skip pre-activation → Post-activation monitoring (using KPI envelope from previous deployment or initial conformance).

The technical rationale is clear: pre-activation testing is reserved for high-risk updates, reducing the volume of lab or vendor-facility testing while still protecting against degraded deployments.

Overall, this hybrid process reduces operational delays by avoiding mandatory lab testing for minor updates, maintains high assurance for major updates, captures real-world performance effects in the field, and minimizes signaling overhead. It also aligns with ongoing RAN1 work on metric definitions while preserving RAN4’s role in ensuring KPI interoperability.

Pre-activation validation acts as a gatekeeper. It stops obviously broken or high-risk models from ever reaching the live network. For major updates, such as architecture changes, new RF front-end profiles, or large distribution shifts , the potential for severe KPI degradation is high. If we skipped this step, those failures would surface only after activation, meaning the live network and end users would experience degraded performance until the monitoring system detected it and fallback was triggered. In a mobility or beam management context, that could mean seconds to minutes of mis-steering, dropped calls, or inefficient handovers before recovery.

Post-activation monitoring, in contrast, is about lifetime assurance under real network conditions. Even a model that passed pre-activation validation perfectly can degrade over time due to factors that were absent from the test dataset , e.g., new interference patterns, weather impacts, hardware aging, or seasonal usage shifts. Without ongoing monitoring, these long-term drifts would go unnoticed.

In short: pre-activation protects the network from immediate catastrophic failures at the moment of deployment, while post-activation protects against gradual or unforeseen degradation during the model’s operational life. Both are needed in the hybrid approach to cover different risks and time horizons.

Pre-activation protects the network from immediate catastrophic failures at the moment of deployment, while post-activation protects against gradual or unforeseen degradation during the model’s operational life. Both are needed in the hybrid approach to cover different risks and time horizons

However, in order for Option 2 to prove useful for post-deployment the UE should support shadow inference.

Shadow inference is when a model runs in parallel with the active, production model but its outputs are never used for live decision-making.

Here’s how it works in the RAN4 AI/ML context:

- The UE’s active model handles all real-time inference for beam prediction, mobility, etc.  its outputs drive network procedures.

- At the same time, one or more standby models (e.g., different adapters, updated versions) receive the exact same input data stream as the active model.

- These standby models produce predictions “in the shadows”   meaning the results are captured for KPI evaluation but never applied to the network.

- KPIs from these shadow models (e.g., beam ID accuracy, RSRP error) are compared to genie references or to the active model’s performance.

If shadow inference is not supported, Option 2 post-activation monitoring loses most of its value. This is because monitoring in Option 2 can only collect live KPI data from the model currently serving inference; any other models  stored on the UE remain untested during actual network operation. Without shadow inference, there is no mechanism to detect drift, degradation, or dataset mismatch in standby models until they are actively swapped in for inference  at which point a performance problem could directly affect service quality. In other words, Option 2’s “continuous safeguard” applies only to the active model, leaving all non-active models outside its protection. This effectively reduces the benefit of Option 2 to a single-model scenario, negating its intended role in multi-model LCM.

Shadow inference solves this gap by running one or more standby models in parallel with the active model, using the same input data stream. The outputs of these standby models are evaluated against genie values (when available) or compared to the active model’s KPIs, allowing their performance to be tracked without impacting live service. While shadow inference increases processing and memory load  especially in FR2 or high-throughput scenarios  it is the only way to achieve true post-activation monitoring for all stored models without service interruption. For efficiency, shadow inference can be limited to periodic sampling, reduced-resolution inputs, or selected KPIs, but some form of it is essential if Option 2 is to fulfill its purpose in a multi-model deployment.

Shadow inference is essential for Option 2 because post-activation monitoring only gathers KPIs from the model actively serving inference. Without shadow mode, standby models remain untested in live conditions, risking undetected drift or degradation until swapped

Based on our discussion we have the following proposals:

For post-deployment AI/ML operation, RAN4 shall adopt a hybrid life-cycle management (LCM) framework that combines pre-activation validation for major updates with post-activation monitoring for all updates. Major updates, defined as architecture changes, new RF front-end profiles, or significant dataset distribution shifts, shall undergo pre-activation validation in lab or on-device test mode to establish or update their KPI envelope. Minor updates may bypass pre-activation validation but must retain the previously established KPI envelope through online monitoring.

For multi-model deployments under Option 2 LCM, post-activation monitoring shall include shadow inference for all non-active models. These standby models shall be exercised on the same input stream as the active model, with their outputs evaluated against genie references or active-model KPIs. This ensures drift or degradation is detected before a model is swapped into service, without impacting live operation

RAN4 shall define standardized criteria for classifying model updates as major or minor within a given deployment scenario. These criteria shall consider the scope of model retraining, changes to architecture, inclusion of new channel models or RF profiles, dataset distribution shifts, and modifications to the inference pipeline. This classification will determine whether a model must pass through the pre-activation validation stage before deployment, while all models, regardless of classification, will be subject to post-activation LCM monitoring

RAN4 shall specify that KPI envelopes used for post-deployment monitoring be scenario-specific and model-specific, ensuring that performance tracking is tailored to the environment and use case the model was designed for. KPI envelopes shall be derived from pre-activation validation results for major updates or inherited from prior deployments for minor updates.

# 3	LCM requirements

## 3.1	On delay requirements for activation/deactivation/fallback

RAN2 has earlier agreed on the following definitions, which are relevant for all use cases:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

- Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

- Activated functionalities refers to functionalities already enabled for performing inference.

For a functionality to be applicable at least there should at least one model available within it. RAN2 also agreed that the UE will indicate to the gNB/LMF whether the AI/ML functionality is available/applicable.

Although the granularity of functionality (e.g., whether it is a use case such as beam management, whether it is a sub-use case such as beam management Case 1, or others) is still under discussion in RAN1, “RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.”

The following was agreed in RAN4#114bis [1] on delay requirements for activation/switching/fallback:

Agreement:

RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2:

      - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference

      - functionality deactivation refers to the process of network deactivating an active functionality

Agreement:

FFS on whether the deactivation requirements will be introduced

      - The starting point is when the UE receives the signalling to commence deactivation

      - FFS when the endpoint for deactivation is

      - FFS whether fallback delay is the same as deactivation delay or has some other components

Agreement:

For beam management use case

      - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML

      - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML

      - FFS on the definition of first reporting

The increasing use of AI/ML-based functionalities in NR introduces new requirements for lifecycle management (LCM), particularly around how quickly such functionalities can be activated, deactivated, or replaced. These procedures are critical in scenarios where the AI/ML system must adapt to dynamic conditions (e.g., channel changes, loss of applicability, or degraded performance). RAN2 has already defined key terms such as supported, applicable, and activated functionalities, which describe the progression from a UE's declared capability to the actual execution of inference. However, for performance-sensitive use cases like beam management, the timing constraints surrounding these transitions remain under discussion, particularly in RAN4.

One of the central challenges is defining the delay requirements associated with these transitions. For example, when an AI/ML functionality is found to be no longer applicable or begins to underperform, the system must either deactivate it or fall back to a non-AI/ML operation. Yet, the precise timing from the signaling trigger to the full operational transition needs to be carefully defined and, possibly, standardized. Furthermore, there is ambiguity on whether fallback delay is identical to deactivation delay or includes additional steps (such as reconfiguring legacy measurements or procedures). In parallel, functionality switching activating one AI/ML functionality while simultaneously deactivating another also lacks a clear definition, even though it may be essential for adaptive deployments.

Another layer of complexity arises when considering how AI/ML and non-AI/ML operations coexist. In some configurations, legacy procedures might run in parallel with AI/ML methods, but in others, the system may need to cleanly switch modes. The lack of a consistent definition of non-AI/ML operation or fallback  could risk misalignment across UEs and gNBs, leading to potential performance issues. RAN4 has therefore proposed working definitions to facilitate progress on this topic. These definitions attempt to clarify what constitutes a switch, a fallback, or a deactivation, but agreement on their associated timing implications is still for further study (FFS).

The LCM delay problem revolves around the need to ensure fast, reliable transitions between AI/ML states, particularly when real-time performance is at stake. The definition of these transitions must be precise enough to allow for testing and consistent implementation across vendors, but also be flexible enough to accommodate a wide range of AI/ML use cases. RAN4’s next steps should focus on refining these working definitions, aligning with RAN1 and RAN2 on granularity and signaling, and determining whether delay requirements for switching, deactivation, and fallback need to be explicitly defined and tested.

AI/ML functionalities differ greatly in their sensitivity to activation and fallback delays. For instance, AI-based beam management during mobility may require transition times within a few slots, while AI-based CSI compression enhancement may tolerate longer latencies. Without differentiating delay expectations by use case, it is difficult to specify meaningful and testable timing requirements. A single delay budget among many use cases risks being either too strict for some functionalities or too loose for critical ones, undermining both efficiency and reliability.

(Use-Case-Aware Delay Classes for Activation and Fallback) RAN4 should define a set of delay classes that reflect the time-criticality of various AI/ML functionalities (e.g., Class 1:   for beam mobility; Class 2: for medium-latency AI tasks; Class 3: for background tasks). Each AI/ML Function Group (FG) or feature, once agreed in RAN1, would be mapped to one of these classes. The class assignment would be signaled during capability exchange or RRC reconfiguration, allowing the gNB and UE to align their expectations.

The boundaries of activation, deactivation, switching, and fallback procedures remain loosely defined across RAN1, RAN2, and RAN4. This leads to ambiguity in measuring when a procedure begins and ends, complicating both implementation and testing. For example, in a fallback scenario, does the timer stop when the AI/ML model halts, or when the legacy method begins? Similarly, switching from one AI/ML function to another in a single signaling message is not clearly distinguished from serial activation/deactivation. By defining a Transition Window a clear start and end point for when an AI/ML functionality change occurs, we can specify and test the total transition delay without needing to standardize or observe the UE's internal implementation details. In other words, RAN4 doesn't need to define exactly how the UE loads, initializes, or unloads models; it only needs to ensure that from the moment the network commands the transition, the UE completes the switch and is ready to operate within a certain delay budget (e.g., 4 slots or 10 ms).

This also simplifies the concept of switching, which is treated as one atomic event: the deactivation of one functionality and activation of another in the same signaling message. Instead of treating switching as two separate procedures (with two different timers or signals), this approach clarifies it as a coupled action: one enters a transition state, and one emerges in the new state. This approach eliminates redundant signaling and can improve the efficiency of state management in AI/ML LCM procedures. Overall, this framework provides a testable, interoperable way to manage timing across diverse AI/ML use cases without taking into account UE-specific operational details.

RAN4 should define a Transition Window for activation, deactivation, and fallback of AI/ML models, starting when the UE receives the relevant command and ending when it is ready for inference or reconfigured to legacy operation. By default, control is RRC-based, with the start at reception of the RRC command and the end at the UE’s RRC Complete message, providing an unambiguous completion signal. If MAC CE/DCI-based control is used, the start is reception of the MAC CE/DCI, but an explicit UE-to-NW readiness indication, via MAC CE acknowledgment, implicit CQI feedback, or explicit signaling with the model ID, must be added to define the end point. This delay should be standardized and tested independently of the UE’s internal steps.

In post-deployment operation, the UE may determine that an active AI/ML model is no longer suitable for use. This can occur when the model becomes not applicable, such as when scenario classification detects that current conditions fall outside its validated domain or when required inputs are unavailable, or when the model is no longer performing adequately, for example if runtime KPI monitoring shows performance drifting outside the KPI envelope. In both cases, continued use of the model could degrade system behavior or violate conformance assumptions.

When such conditions occur, the UE should inform the network through minimal applicability/inapplicability signaling, as discussed in RAN2, to maintain visibility of the UE’s current operational state. This enables the gNB to coordinate lifecycle actions, such as activating a different AI/ML model, reconfiguring parameters, or falling back to legacy operation. Without this signaling, the network is blind to state transitions, making it harder to trace performance changes, debug issues, and coordinate related functions like beam reporting or measurement feedback.

When a UE determines an AI/ML model is no longer applicable or its runtime KPIs fall outside the validated KPI envelope, it shall signal this state change to the network using minimal applicability/inapplicability reporting. This ensures the gNB has visibility into UE operational state, enabling coordinated actions such as adapter switching, model reconfiguration, or fallback to legacy operation.

When a UE either activates an AI/ML model (starts using it for inference) or falls back to a non-AI/ML method, there needs to be a clear and agreed-upon time limit, a delay bound,  for how long that transition can take. Without such bounds, different UEs may behave inconsistently, which can affect network coordination and user experience. The problem is that today there’s no standardized, externally observable way to determine exactly when that transition is complete. Measuring internal UE processing steps isn’t practical for conformance testing, because vendors don’t expose those details, and the internal timelines can vary widely.

Current 3GPP agreements also haven’t fixed the definition of the “end-point” for activation or fallback , for example, whether it’s when the AI/ML algorithm starts making decisions or when the first valid measurement/report is sent. This lack of clarity makes it harder to test devices in a consistent way, ensure they interoperate correctly, and validate their performance under different network conditions.

(Define Activation and Fallback Delay via First Valid Measurement Trigger) RAN4 should define AI/ML activation and fallback delay in terms of the time to the first valid L1/L3 measurement report generated under the new operational state, rather than relying on internal UE processing timelines. For example, in beam management, activation delay would be measured from the network’s activation command to the first AI/ML-based report, and fallback delay from the fallback trigger to the first legacy (non-AI/ML) report. Using an externally observable event as the endpoint removes ambiguity, enables consistent measurement across UEs and scenarios, and supports interoperability and conformance testing in line with RAN2’s applicability and activation framework

### AI/ML beam management

For the beam management use case, UE-sided model, the LCM steps are as below.

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

  - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signalling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality.

- Applicable periodic CSI Report (for beam management) is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported, e.g., by L1 and MAC signalling.

- When a functionality becomes non-applicable the UE reports this to NW, but does not autonomously deactivate the functionality. NW is expected to deactivate active functionality upon receiving the UE’s report.

Further related agreements were made by RAN2 in RAN2#129 (Feb. 2025):

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

From the above agreements, in the case of periodic CSI reporting, the UE autonomously activates applicable functionalities upon reporting applicability via RRCReconfigurationComplete (Step 4). To avoid excessive activation delays, defining activation delay requirements is well justified for periodic CSI reporting with AI/ML, while for the aperiodic case, RAN4 has already agreed in RAN4#114bis to define corresponding activation delay requirements.

When a functionality becomes non-applicable, the UE reports this to the network, which is then expected to deactivate the active functionality upon receiving the report. If this requires the UE to switch to non-AI/ML operation, the transition should be completed as quickly as possible, making a defined deactivation delay requirement necessary to ensure timely and consistent behavior.

Regarding the agreement on whether fallback delay is identical to deactivation delay or includes additional components, for beam management the AI/ML functionality deactivation delay may extend to include extra time, such as the interval until the next valid non-AI/ML report is generated.

For the UE-side AI/ML model in the beam management use case, RAN4 should define activation delay requirements for periodic CSI reporting, measured from the applicability report (e.g., in RRCReconfigurationComplete) to the first valid AI/ML-based report, ensuring timely operation after functionality is deemed applicable. Similarly, deactivation delay requirements should be defined, measured from the network’s deactivation command to the end of AI/ML-based reporting, ensuring prompt transition to non-AI/ML operation when a functionality becomes non-applicable. For fallback cases, the fallback delay should be defined separately if it includes additional components beyond deactivation, for example, the time until the first valid non-AI/ML report is received. These requirements will provide clear, externally measurable timing bounds for activation, deactivation, and fallback, improving interoperability, performance validation, and lifecycle management.

# Conclusion

In the previous sections we made the following proposals:

- Option 1 (pre-activation testing) ensures assurance but can be slow, costly, and impractical for frequent updates. Option 2 (post-activation LCM monitoring) is scalable but risks false triggers, overhead, and delayed detection if not carefully designed

Pre-activation protects the network from immediate catastrophic failures at the moment of deployment, while post-activation protects against gradual or unforeseen degradation during the model’s operational life. Both are needed in the hybrid approach to cover different risks and time horizons

Shadow inference is essential for Option 2 because post-activation monitoring only gathers KPIs from the model actively serving inference. Without shadow mode, standby models remain untested in live conditions, risking undetected drift or degradation until swapped

- For post-deployment AI/ML operation, RAN4 shall adopt a hybrid life-cycle management (LCM) framework that combines pre-activation validation for major updates with post-activation monitoring for all updates. Major updates, defined as architecture changes, new RF front-end profiles, or significant dataset distribution shifts, shall undergo pre-activation validation in lab or on-device test mode to establish or update their KPI envelope. Minor updates may bypass pre-activation validation but must retain the previously established KPI envelope through online monitoring.

For multi-model deployments under Option 2 LCM, post-activation monitoring shall include shadow inference for all non-active models. These standby models shall be exercised on the same input stream as the active model, with their outputs evaluated against genie references or active-model KPIs. This ensures drift or degradation is detected before a model is swapped into service, without impacting live operation

RAN4 shall define standardized criteria for classifying model updates as major or minor within a given deployment scenario. These criteria shall consider the scope of model retraining, changes to architecture, inclusion of new channel models or RF profiles, dataset distribution shifts, and modifications to the inference pipeline. This classification will determine whether a model must pass through the pre-activation validation stage before deployment, while all models, regardless of classification, will be subject to post-activation LCM monitoring

RAN4 shall specify that KPI envelopes used for post-deployment monitoring be scenario-specific and model-specific, ensuring that performance tracking is tailored to the environment and use case the model was designed for. KPI envelopes shall be derived from pre-activation validation results for major updates or inherited from prior deployments for minor updates.

(Use-Case-Aware Delay Classes for Activation and Fallback) RAN4 should define a set of delay classes that reflect the time-criticality of various AI/ML functionalities (e.g., Class 1:   for beam mobility; Class 2: for medium-latency AI tasks; Class 3: for background tasks). Each AI/ML Function Group (FG) or feature, once agreed in RAN1, would be mapped to one of these classes. The class assignment would be signaled during capability exchange or RRC reconfiguration, allowing the gNB and UE to align their expectations.

RAN4 should define a Transition Window for activation, deactivation, and fallback of AI/ML models, starting when the UE receives the relevant command and ending when it is ready for inference or reconfigured to legacy operation. By default, control is RRC-based, with the start at reception of the RRC command and the end at the UE’s RRC Complete message, providing an unambiguous completion signal. If MAC CE/DCI-based control is used, the start is reception of the MAC CE/DCI, but an explicit UE-to-NW readiness indication, via MAC CE acknowledgment, implicit CQI feedback, or explicit signaling with the model ID, must be added to define the end point. This delay should be standardized and tested independently of the UE’s internal steps. Another option is described in Proposal 9 for BM.

When a UE determines an AI/ML model is no longer applicable or its runtime KPIs fall outside the validated KPI envelope, it shall signal this state change to the network using minimal applicability/inapplicability reporting. This ensures the gNB has visibility into UE operational state, enabling coordinated actions such as adapter switching, model reconfiguration, or fallback to legacy operation.

(Define Activation and Fallback Delay via First Valid Measurement Trigger) RAN4 should define AI/ML activation and fallback delay in terms of the time to the first valid L1/L3 measurement report generated under the new operational state, rather than relying on internal UE processing timelines. For example, in beam management, activation delay would be measured from the network’s activation command to the first AI/ML-based report, and fallback delay from the fallback trigger to the first legacy (non-AI/ML) report. Using an externally observable event as the endpoint removes ambiguity, enables consistent measurement across UEs and scenarios, and supports interoperability and conformance testing in line with RAN2’s applicability and activation framework

For the UE-side AI/ML model in the beam management use case, RAN4 should define activation delay requirements for periodic CSI reporting, measured from the applicability report (e.g., in RRCReconfigurationComplete) to the first valid AI/ML-based report, ensuring timely operation after functionality is deemed applicable. Similarly, deactivation delay requirements should be defined, measured from the network’s deactivation command to the end of AI/ML-based reporting, ensuring prompt transition to non-AI/ML operation when a functionality becomes non-applicable. For fallback cases, the fallback delay should be defined separately if it includes additional components beyond deactivation, for example, the time until the first valid non-AI/ML report is received.

# References

[1] R4-2505107, WF on AI ML for air interface study, Ericsson, Apr. 2025.