3GPP TSG-RAN WG4 Meeting #116	R4-2509426

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.5

Source: 	apple

Title: 	RRM core requirement and testing framework for Positioning accuracy enhancement

Document for:	Discussion

# Discussion

In RAN1 #110bis meeting, following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study,

- Case 1: 		UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In RAN #107 meeting, case 2a and case 2b have been removed from the Revised WID[1], and following objectives have been agreed.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In the above use cases, as agreed in RAN4 #110 meeting, RAN4 will not define positioning accuracy requirements for case 3a/3b.

During RAN4#115 the following agreements were reached:

Issue 3-1: Requirements for case 1

Agreement:

- Discuss how to define a measurement delay requirement(reporting delay)

- Do not define any positioning accuracy requirement

Issue 3-2: Report mapping for sample based timing measurement reporting

Agreement:

  - Use the value of the legacy UL RTOA mapping for 1st sample

  - FFS for the additional sample reporting

Issue 3-3: Requirements for case 3b

Agreement:

- Reuse legacy path based Rx-Tx accuracy requirements for case 3b

- Requirement applies only for the 1st path

### Measurement Accuracy and Report Mapping Challenges in Case 3b

In Case 3b, the AI/ML model is deployed at the LMF, while the gNB collects and reports channel measurements (e.g., time-domain channel impulse response, CIR) as inputs to the model. These measurements are critical for high-accuracy uplink-based positioning, but moving from legacy path-based to sample-based reporting introduces new challenges for RAN4 in defining mapping, granularity, quantization, and validation requirements. AI/ML models are sensitive to input distribution drift, so variations in timing alignment, sample count (Nt'), granularity factor (k), power scaling, or inclusion of phase information can degrade performance and hinder generalization. While RAN1 has agreed to support various k and Nt' values, RAN4 must standardize how additional samples beyond the first are mapped, quantized, and validated to ensure consistent interpretation across gNB implementations.

RAN4 has agreed to reuse the legacy UL RTOA mapping for the first sample (Issue 3-2) and to apply legacy path-based Rx–Tx accuracy requirements only for the first path (Issue 3-3). However, for additional samples, open items include defining their time mapping relative to the first sample, granularity or spacing, quantization resolution and scaling for timing and power values, permissible reporting range, and whether phase information should be included and, if so, how it should be represented. Without clear specifications for these parameters, vendor-specific implementations may emerge, undermining interoperability, limiting model portability, and reducing the benefits of standardization.

Proposal 1: RAN4 is invited to define the mapping, granularity, quantization, and reporting range for additional samples, and to decide whether phase information should be included and, if so, its representation. Completing these specifications will prevent vendor-specific behaviors, ensure interoperability, and support robust LMF-side model performance

# Conclusions

In this contribution, we have got the following proposals:

Proposal 1: RAN4 is invited to define the mapping, granularity, quantization, and reporting range for additional samples, and to decide whether phase information should be included and, if so, its representation. Completing these specifications will prevent vendor-specific behaviors, ensure interoperability, and support robust LMF-side model performance

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface