3GPP TSG-RAN WG4 Meeting #116	R4-2509427

# Bengaluru, India, August 25th – 29th, 2025

Source:	Apple

Title:	RRM performance requirements for beam management and positioning accuracy

Agenda item:	7.17.6

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM and Positioning use cases were continued in RAN4#114bis in the scope of Rel-19 WI NR_AIML_air ‎[1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#114bis meeting are summarized in the WF ‎[2].

In this paper, we address RRM performance requirements for AI/ML enabled beam management. We discuss following main aspects:

- Performance requirements for AI/ML BM

- Generalization aspects

Discussion

Performance requirements for AI/ML BM

Measurement error impact and the ground truth

During RAN4#112 and RAN4#112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements of RAN4 #113 are provided below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

Using the agreed evaluation methodology as detailed in R4-2508081 from the last RAN4#115 meeting, we present the simulation results for the following KPIs and Cases:

At least the following performance characteristics are to be provided

KPI 1: Beam prediction accuracy

- The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB

KPI 2: Absolute RSRP accuracy

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the 1st / 3rd / 5th strongest predicted beam(s) and the ground truth L1-RSRP of the same beams

Note 1: the ‘ground truth’ underlined refers to:

- Ideal value without consideration of measurement error (for Case 1 and Case 2a defined in clause 6)

- Measurement result with consideration of measurement error (for Case 2b and Case 3 defined in clause 6)

Note 2:

90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))

- Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement.  (narrow to narrow)

- Scenario 2: Spatial domain prediction with 32 CSI-RS beams in Set A and 8 SSB beams in Set B (wide to narrow)

The following cases are treated as the candidate Options:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 2:

  - Case 2a No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference

  - Case 2b No error will be considered in training dataset and ground truth for training. Error will be considered in model input during inference and ground-truth for inference

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

Below we provide our simulation results:

Case 1 KPI 1:

KPI1: Scenario 1

| top-K/X (dB) | K=1 | K=2 | K=3 | K=4 | K=5 |
| --- | --- | --- | --- | --- | --- |
| X=0 | 0.6 | 0.77 | 0.85 | 0.89 | 0.92 |
| X=1 | 0.72 | 0.85 | 0.9 | 0.93 | 0.95 |
| X=2 | 0.8 | 0.898 | 0.934 | 0.953 | 0.964 |
| X=3 | 0.85 | 0.923 | 0.96 | 0.97 | 0.98 |

KPI1: Scenario 2

| top-K/X (dB) | K=1 | K=2 | K=3 | K=4 | K=5 |
| --- | --- | --- | --- | --- | --- |
| X=0 | 0.72 | 0.87 | 0.93 | 0.953 | 0.97 |
| X=1 | 0.843 | 0.937 | 0.96 | 0.976 | 0.984 |
| X=2 | 0.9 | 0.96 | 0.976 | 0.984 | 0.992 |
| X=3 | 0.94 | 0.978 | 0.989 | 0.993 | 0.995 |

Case 1 KPI 2:

KPI2: Scenario 1                                                                                     KPI2: Scenario 2

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 4.38 | 5.3 | 5.3 |

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 3.1 | 4.2 | 4.9 |

Case 2a KPI 1:

KPI1: Scenario 1                                                                                              KPI1: Scenario 2

| top-K/X (dB) | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.48 | 0.66 | 0.76 | 0.82 | 0.87 |
| 1 | 0.59 | 0.76 | 0.84 | 0.89 | 0.91 |
| 2 | 0.69 | 0.83 | 0.89 | 0.92 | 0.94 |
| 3 | 0.77 | 0.88 | 0.926 | 0.947 | 0.962 |

| top-K/X (dB) | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.49 | 0.68 | 0.77 | 0.84 | 0.88 |
| 1 | 0.59 | 0.77 | 0.85 | 0.9 | 0.93 |
| 2 | 0.68 | 0.84 | 0.9 | 0.94 | 0.96 |
| 3 | 0.75 | 0.89 | 0.94 | 0.96 | 0.98 |

Case 2a KPI 2:

KPI2:Scenario1                                                                                     KPI2:Scenario2

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 7.3 | 8.3 | 8.3 |

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 9.8 | 9.8 | 9.8 |

Case 2b KPI1:

| top-K/X (dB) | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.41 | 0.59 | 0.7 | 0.78 | 0.83 |
| 1 | 0.49 | 0.67 | 0.78 | 0.84 | 0.88 |
| 2 | 0.56 | 0.748 | 0.84 | 0.89 | 0.92 |
| 3 | 0.64 | 0.81 | 0.886 | 0.927 | 0.95 |

KPI1: Scenario 1                                                                                          KPI1: Scenario 2

| top-K/X (dB) | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.38 | 0.58 | 0.69 | 0.77 | 0.82 |
| 1 | 0.47 | 0.65 | 0.76 | 0.83 | 0.87 |
| 2 | 0.55 | 0.73 | 0.82 | 0.87 | 0.91 |
| 3 | 0.62 | 0.8 | 0.87 | 0.9 | 0.93 |

Case 2b KPI 2:

KPI2:Scenario1                                                                                     KPI2:Scenario2

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 7.3 | 8 | 8.5 |

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 9.8 | 9.9 | 9.8 |

Case 3 KPI 1:

KPI1: Scenario 1                                                                                          KPI1: Scenario 2

| top-K/X (dB) | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.43 | 0.61 | 0.72 | 0.79 | 0.83 |
| 1 | 0.52 | 0.69 | 0.78 | 0.84 | 0.88 |
| 2 | 0.6 | 0.76 | 0.84 | 0.89 | 0.91 |
| 3 | 0.68 | 0.82 | 0.88 | 0.91 | 0.94 |

| top-K/X (dB) | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.43 | 0.63 | 0.75 | 0.82 | 0.87 |
| 1 | 0.52 | 0.7 | 0.81 | 0.87 | 0.9 |
| 2 | 0.6 | 0.78 | 0.865 | 0.91 | 0.94 |
| 3 | 0.68 | 0.84 | 0.9 | 0.94 | 0.96 |

Case 3 KPI 2:

KPI2:Scenario1                                                                                     KPI2:Scenario2

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 6.8 | 7.7 | 7.7 |

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 7 | 8.4 | 8.8 |

We also simulated the case which follows case3 assumptions, but the ground truth contains no errors. We call this Case 3a

Case 3a: (clean output labels for inference only)

KPI1:Scenario1

| top-K/X (dB) | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.44 | 0.64 | 0.75 | 0.82 | 0.87 |
| 1 | 0.55 | 0.74 | 0.83 | 0.87 | 0.9 |
| 2 | 0.65 | 0.82 | 0.88 | 0.92 | 0.94 |
| 3 | 0.73 | 0.87 | 0.92 | 0.94 | 0.96 |

KPI2:Scenario1   ( RF (x= +-4.5dB) + LLS)

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 6.92 | 7.6 | 7.7 |

KPI2:Scenario1 ( RF only Errors)

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 5.4 | 6.5 | 6.5 |

KPI2:Scenario1   ( RF only with x= +-2.25 dB)

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 5. | 6. | 6. |

KPI2:Scenario1   ( RF only with x =+1dB)

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 4.6 | 5.4 | 5.4 |

KPI2:Scenario1   ( LLS only)

| top-K predicted | 1 | 3 | 5 |
| --- | --- | --- | --- |
| RSRP accuracy (dB) | 5.4 | 6.7 | 6.7 |

Fig. 1: PDF of prediction errors and values

From Fig. 1, it can be observed that there is no evidence of significant systematic bias.

Fig. 2: RSRP Prediction errors as a function of various levels of noise sources

When we select the predicted Top-1 beam out of 32 purely because it has the highest predicted RSRP, we are not only choosing a strong beam, we are also picking the maximum of many noisy estimates. Mathematically, if the true RSRP of beam i is μᵢ and the prediction error is εᵢ with mean 0 and variance σᵢ², the predicted value is yᵢ = μᵢ + εᵢ. By taking argmax(yᵢ), we increase the likelihood that the chosen beam’s εᵢ is positive (overestimated). Even if the errors are zero-mean overall, conditioning on being the maximum shifts the expectation upward, so E[ε_(Top-1)] > 0. This is the “winner’s curse”,  the best-looking estimate is, on average, slightly too good to be true.

When we pick the Top-1 predicted beam, we are not just selecting the strongest candidate, we are also favoring the one that happened to be overestimated due to random noise in the predictions. This means the “winner” often looks slightly better in the test results than it is really in reality.

In Fig2, we plot the RSRP Prediction errors as a function of various levels of noise sources for case 3a and for scenario 1.

We also plot the distribution of RF errors and LLS baseband errors for reference.

Based on the results in Fig 2, let’s make the following definitions:

B_K = Baseline AI/ML model 90th percentile absolute error (in dB) for top-K beam prediction with no RF or LLS errors.

Δ_RF(x,K) = Additional 90th percentile error (in dB) due to RF error with variance x (in dB²), for top-K.

  Δ_LLS(K) = Additional 90th percentile error (in dB) due to LLS error at SNR = –3 dB (no RF), for top-K.

  Δ_LLS|RF(x,K) = Additional 90th percentile error (in dB) due to LLS when RF errors are also present, for top-K.

  P90 = 90th percentile absolute error between AI/ML predicted RSRP and ground truth RSRP.

  K = Number of top beams considered (K = 1, 3, or 5 in your figure).

  Spec = Requirement limit for P90 absolute error = 6.0 dB.

In order to meet the legacy measurement accuracy (ignore measurement uncertainty for now)  we would require:

B_K + Δ_RF(x,K) + Δ_LLS(K) ≤ Spec

From this, the maximum allowable LLS error for a given RF profile is:

Δ_LLS,max(x,K) = Spec - (B_K + Δ_RF(x,K))

From figure 2, we have the following numerical results in dB:

Baseline (B_K):
K=1 → 4.4dB, K=3 → 5.3 dB, K=5 → 5.3 dB

RF only (Δ_RF):

- x = 1.0 dB → K=1: 0.2 dB, K=3: 0.1 dB, K=5: 0.1 dB

- x = 2.25 dB → K=1: 0.6 dB, K=3: 0.7 dB, K=5: 0.7 dB

- x = 4.5 dB → K=1: 1.0 dB, K=3: 1.2 dB, K=5: 1.2 dB

LLS only at SNR = –3 dB (Δ_LLS):
K=1: 1.0 dB, K=3: 1.4 dB, K=5: 1.3 dB

Based on the simulation results we can derive the following LLS margins for different RF profiles and top-K beams as shown in the table below:

Negative cap means even zero LLS error cannot meet spec for that combination (would require reducing B_K or RF variance).

We are proposing to investigate and focus solely on the most practical scenario of a modified version of Case 3 for simulation alignment purposes, where the inference target labels (e.g., RSRP or beam index) are assumed to be error-free and derived directly from idealized ground truth data. This ensures that evaluation focuses solely on the impact of AI/ML model behavior and not on noise introduced by training targets. Additionally, to better understand model performance beyond point statistics, it is proposed to evaluate and report the full probability density functions (PDFs) of the prediction errors (e.g., RSRP_pred − RSRP_true) rather than relying solely on fixed metrics like the 90th percentile. Analyzing these error distributions helps identify potential prediction bias, asymmetries, and non-Gaussian tails that may influence downstream RRM decisions. Such a dual approach, clean targets and full error characterization enables a more transparent framework for assessing AI/ML model generalization and conformance across deployments.

RAN4 is invited to adopt and focus on a modified Case 3a simulation setup using error-free ground truth labels to isolate model performance from training noise. Additionally, we propose plotting full error PDFs (e.g., RSRP_pred − RSRP_true) to identify bias, skewness, or non-Gaussian behavior, rather than relying solely on percentile metrics. This supports robust AI/ML evaluation and simulation alignment.

Based on our numerical analysis above we have the following proposal:

In order for RAN4 to meet the P90(|error|) ≤ 6 dB requirement (FR2), it is proposed to (i) reduce SNR-sensitive errors by ensuring higher-SNR side conditions for the reference signals used in AI/ML inputs, (ii) further improve SNR through measures such as RS power boosting or 1–2 non-coherent repetitions, (iii) revisit RF assumptions by defining deployment RF profiles with tighter variance where feasible, and (iv) apply noise-aware training with augmentation matched to the declared RF/LLS error models to improve model robustness and reduce the baseline prediction error.

We provide an analysis below to highlight the superposition of all non-idealities as the output of the AI/ML model:

Let Xgenie  denote the ground truth value ("genie" X) and Xmeasured  denote the measured value of X. Let Δ=Xgenie-Xmeasured denote the measurement error (requirement accuracy).  Xpred=fAI/MLXsubsetmeasured the AI/ML predicted value of X using a subset of the measured X.  The prediction error of the model under clean inputs ϵpred=Xgenie-Xpred(clean). The input measurement error: ϵinput=Xgeniesubset-Xmeasuredsubset

Assuming the model is Lipschitz continuous with constant L, then the total error between the predicted X and the genie X is bounded by: Xgenie-Xpred≤ϵpred+L⋅ϵinput. This represents the total inference-time error between Xpred and Xgenie accounting for both: the model's prediction error when trained on clean inputs ϵpred  and the additional error due to noisy inputs at inference L ϵinput

For Case 2b when we compare the difference between the predicted values and the genie values containing measurements errors, we provide the following analysis: We want to bound the difference Xpred-Xmeasured. We can write: Xpred-Xmeasured≤Xgenie-Xmeasured+Xpred-Xgenie. That is, Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured  is the full measurement error across all dimensions of X,  and ϵinput=Xgeniesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model

Assuming the ML model is Lipschitz continuous with constant L, then the total error between the predicted X and the measured X is bounded by: Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured  is the full measurement error across all dimensions of X,  and ϵinput=Xtruesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model

Comparing predicted RSRPs to their corresponding measured values using an absolute accuracy metric mostly reflects relative prediction error. However, this relative error can exceed the acceptable RSRP margin defined in legacy systems.

To understand how the total prediction error in Set A (the predicted beam set) is related to the measurement error in Set B (the input beam set), we can decompose the overall error into two additive components:

This is the baseline error the AI/ML model makes when fed perfect (noiseless) features from Set B: E_model = f(X_true) - Y_true where:

- X_true is the true (noise-free) measurement vector from Set B (e.g., true RSRPs),

- Y_true is the ground truth label or measurement for Set A (e.g., genie RSRP ),

- f(·) is the AI/ML model performing the spatial prediction.

If the input measurements from Set B are noisy (e.g., due to L1 noise or estimation error), the model is actually given: X_noisy = X_true + ε_meas. Then the model outputs: Y_pred = f(X_noisy) = f(X_true + ε_meas). The error due to input noise is the difference between this output and the ideal output: E_noise = f(X_true + ε_meas) - f(X_true). This term depends on:

- The local sensitivity of the model to input perturbations (i.e., gradients or Jacobian),

- The nonlinearity of the function f(·),

- The magnitude and structure of ε_meas.

The total prediction error, combining both modeling error and input noise propagation, is: E_total = f(X_true + ε_meas) - Y_true   (Set B= X_true, set A = Y_true)

= [f(X_true) - Y_true] + [f(X_true + ε_meas) - f(X_true)]

= E_model + E_noise

Therefore we see that E_total is affected both by the model design and by input measurement quality. The measurement error in Set B (ε_meas) directly impacts Set A prediction fidelity.

### Under certain conditions, E_noise can be smaller than ε_meas, meaning the model effectively suppresses or attenuates input noise during inference. This is often a design goal in AI/ML systems when input measurements are noisy, as in RSRP-based beam prediction. Let’s compare the two conceptually and analytically.

- ε_meas: The input measurement error in Set B — noise added to true features (e.g., due to fading, estimation bias, quantization).

- E_noise = f(X_true + ε_meas) - f(X_true): The output error caused by this input noise, i.e., how the AI model amplifies or suppresses the input disturbance.

In deep models, locally we can linearize f(·) around X_true using first-order Taylor approximation:  f(X_true + ε_meas) ≈ f(X_true) + J_f(X_true) · ε_meas. Where J_f(X_true) is the Jacobian matrix of partial derivatives. Then: E_noise ≈ J_f(X_true) · ε_meas

‖E_noise‖ ≤ ‖J_f(X_true)‖ · ‖ε_meas‖. If we can guarantee that  ‖J_f(X_true)‖ < 1, the model shrinks the noise.

If we model the AI/ML model with noisy inputs and true labels then: When we train a model on inputs of the form: X̃ = X_true + ε_meas with the corresponding target output still being the clean label: Y = f_target(X_true), the model is forced to learn a function f(·) that maps noisy inputs back to clean outputs. In doing so:

- The model learns to ignore the noise directions in the input space that do not affect the correct output.

- This drives the Jacobian matrix J_f(X) (∂f/∂X) to compress those noise directions

- In practical terms, this makes the model locally Lipschitz with a small constant, meaning small perturbations in input produce smaller or equal perturbations in output.

Performance metrics

Following agreement was reached in RAN4#114bis meeting:

Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values - Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Based on the latest agreements on the beam ID prediction KPIs we can conclude that the evaluation of success rate of correct prediction should consider other strong-enough top-K predicted beams, apart from the top-1 beam.

For beam ID prediction, the following was agreed in the previous meeting[1]:

The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values

The proposal for top-N, N>1 options provide the following analysis:

Soption3=1Nt n=1Ntlmaxbk ∈ set ARSRPbk≥maxbk ∈ set ARSRPbk-X   (1)

And the following proposal:

- The threshold for ith ranked predicted beam, bi can be considered as the RSRP of the corresponding ith ranked genie-aided beam, bi – X

- The success rate of correct prediction can be computed as a weighted average that considers all top-K predicted beams

Weighted success rate, Sw=1Nt n=1Nti=1Kwi,n lRSRPbi,n≥RSRPbi,n-X  (2)

Based on the following observation:

For a fair evaluation, the threshold for ith ranked predicted beam, bi could be set as RSRP of the corresponding ith ranked genie-aided beam bi – X. Hence, the condition to be checked for the ith ranked beam is Ci=lRSRPbi≥RSRPbi-X, where X is a fixed value for all i

Case1: The bi  is the  ith ranked predicted from AI/ML model

Case2: The bi  is the  ith ranked after post-processing:   bi →   maxk(RSRPbi,N). This would correspond to the scenario where the AI/ML model would only predict beam indices and the NW would re-transmit those indices for UE measurements. (during operation in the field, not in testing)

We would like to introduce two distinct flavors of beam ID prediction accuracy testing Case 1 and Case 2 which differ in how the UE provides its beam prediction output, and whether or not predicted RSRP values are reported. This distinction has important implications on the scope and complexity of conformance testing. In particular, if the UE reports only beam indices, testing focuses solely on beam ID selection and ranking. However, if the UE additionally reports predicted RSRP values, separate RSRP accuracy checks on those values become necessary to ensure safe integration with RRM procedures.

In Case 1 (Model-Ranked, beam ID prediction accuracy and ranking accuracy), the UE's AI/ML model outputs a ranked list of Top-K beam indices directly based on predicted RSRP or a classifier based model. This list is trusted by the gNB without additional validation or over-the-air probing. It enables ultra-low-latency response and minimal overhead but depends critically on the fidelity of the AI model’s ranking and RSRP prediction. Since the UE provides a ranked list directly from its AI/ML model, the gNB can begin with a high-probability beam immediately, before RSRP measurements arrive. No probing or additional signalling is required to obtain measurements before beam selection. This is especially beneficial in latency-sensitive use cases. Therefore, it reduces the need for extra probing and overhead in scenarios with many candidate beams (e.g., massive MIMO or mmWave systems). Case 1 (Model-Ranked Beam Index Reporting) is fundamentally stricter to pass because it places full reliance on the AI/ML model to identify not only which beams are likely to be strongest, but also their correct order of priority. This means that both beam ID prediction accuracy and ranking accuracy are under scrutiny

In Case 2 (Probe-Ranked, beam ID prediction accuracy), the UE's AI/ML model outputs an unranked list of K candidate beam indices. During real operation the gNB probes these beams over the air, and the UE measures their L1 RSRPs to determine the final ranking. In this case UE reports an unranked Top-K candidate set of beams, the gNB transmits downlink probes, and the UE performs local ranking based on measured RSRP. This approach leverages traditional measurement mechanisms for final ranking, ensuring robustness and simplifying test scope when predicted RSRP is not reported making it ideal for early deployment and interoperability. For the testing purposes NW doesn’t need to probe those beams because it already knows the genie RSRP beams.

Case 2 (Probed Beam Ranking Post-Prediction) introduces a level of robustness by separating the beam selection (done by the AI model) from the final ranking (based on real measurements). The AI/ML model is used to reduce the beam search space by suggesting a set of promising candidates, but the UE ranks these candidates using actual measured RSRPs before finally reporting RSRP to the gNB. Because the final decision-making is based on physical-layer measurements, the model is not penalized for imperfect ranking or small RSRP prediction errors. This makes Case 2 easier to pass in testing, since it only verifies whether the strong genie beams are included among the predicted candidates without requiring precise ordering or RSRP scoring.

Below we present the following example demonstrating the difference between Case 1 and Case 2, considering x=3dB and K=2 (as the K defined in WF, not the proposal):

| Beam ID | Genie RSRP RSRPbi,n | Reported ranking |  |
| --- | --- | --- | --- |
| 1 | -90dBm |  |  |
| 2 | -90.5dBm | 2 |  |
| 3 | -92dBm |  |  |
| 4 | -93.1dBm | 1 |  |

Case 1: Based on the proposal in [2] for N = 2 (which is K in (2)) case, then we have

i=1:  lRSRPb1,n≥RSRPb1,n-X=l-93.1dBm≥-90dBm-3=0

i=2:  lRSRPb2,n≥RSRPb2,n-X=l-90.5dBm≥-90.5dBm-3=1

The success rate is 50%.  (assuming equal weights)

Case 2: There is a post-processing step:  :   bi →   maxk(RSRPbi,N).

i=1:  lRSRPb1,n≥RSRPb1,n-X=l-90.5dBm≥-90dBm-3=1

i=2:  lRSRPb2,n≥RSRPb2,n-X=l-93.1dBm≥-90.5dBm-3=1

The success rate is 100%.

We would like to make the following proposal:

For UEs reporting only beam IDs, we propose an evaluation framework for beam index prediction accuracy in which the UE predicts a Top-K set of beam candidates via AI/ML, and the final ordering is determined offline using genie RSRP values. The metric checks whether the UE’s predicted Top-K set includes the correct beams, in the offline-determined order, for multiple beam positions, within an allowed RSRP margin (X) from the corresponding strongest genie beams.

We would like to discuss the definition of the threshold value X and how to choose a realistic value. To ensure fair and robust evaluation of AI/ML beam prediction under imperfect ground truth conditions, we introduce a threshold model defined as x = x₀ + Δ. Here, x₀ captures the acceptable deviation in beam quality, while Δ compensates for uncertainty in the reference ground truth.

The threshold parameter x₀ represents the baseline margin used to define "strong-enough" predicted beams in relation to the best genie-aided beam. Conceptually, x₀ accounts for acceptable deviations due to spatial granularity of beams, multipath fading, and system-level variability that would still allow a predicted beam to be operationally acceptable even if it is not exactly the strongest. For example, two beams may differ by a few dB in RSRP but offer nearly identical link quality, making them interchangeable in practical deployments. Setting x₀ to values like 0 dB (strict match), 3 dB, or 5 dB reflects different tolerances for this equivalence zone. Thus, x₀ acts as a performance margin, balancing between prediction strictness and practical flexibility in beam usage by the NW. (e.g. x₀ =1-2dB)

However, when the so-called "ground truth" RSRP values used for evaluation are themselves subject to uncertainty such as measurements errors, this introduces additional ambiguity into the success criterion. The parameter Δ is therefore introduced to absorb this uncertainty, compensating for the possible mismatch between the used ground truth and the true genie-level beam quality. Conceptually, Δ reflects the confidence interval or error budget associated with the ground truth reference. If, for instance, the ground truth RSRP is known to deviate by up to ±4 dB from the actual value due to RF noise, calibration error, then Δ = 8 dB is required to ensure that valid predictions are not unfairly penalized. Together, the effective threshold x = x₀ + Δ ensures that the evaluation metric remains both practically meaningful and robust to reference uncertainties.

### For example, if true genie RSRP of the genie beam = R₁ and the true genie RSRP of the predicted beam = R₂. Then the  errors of RSRP(genie beam) = R₁ ± 4 dB and  RSRP(predicted beam) = R₂ ± 4 dB. So the reported difference becomes:  [R₁ ± 4] − [R₂ ± 4] = (R₁ − R₂) ± 8 dB. To ensure that a valid prediction is not unfairly penalized due to this uncertainty, the threshold x should be: x ≥ x₀ + 8 dB. This assumes that the errors are uncorrelated which is the worst case.  Worst-case bound (conservative, additive):If each RSRP has ±4 dB bounded error, and they are independent (ρ = 0), then the worst-case total error in the difference is: Δ = 8 dB . However, this is a deterministic bound.

In the fully correlated case both R₁ and R₂ are shifted by the same error (e.g., both overestimate by +3 dB). Their difference remains unaffected: (R₁ + ε) − (R₂ + ε) = R₁ − R₂. In this ideal case, Δ = 0 dB because the error cancels out in the subtraction.

Lets now discuss the partially correlated errors (realistic case):

- The errors have some common component (e.g., from a shared calibration bias, or receiver implementation).

- The variance of the difference is Var(R₁ − R₂) = Var(R₁) + Var(R₂) − 2·Cov(R₁, R₂).  If correlation coefficient ρ = Corr(R₁, R₂), and each has variance σ² = (4 dB)² = 16. Then we have: Var_diff = 2σ²(1 − ρ) = 32(1 − ρ) and Std_dev_diff = √[32(1 − ρ)]

Therefore if  ρ = 0 → Std_dev ≈ 5.66 dB (Δ ≈ 5.66)  and if ρ = 0.5 → Std_dev ≈ 4 dB (Δ ≈ 4) and if  ρ = 0.75 → Std_dev ≈ 2.83 dB (Δ ≈ 2.8). These values are based on statistical bounds.

We can follow the methodology described above to properly select a value for X.

RAN4 to adopt a beam prediction accuracy metric based on a RSRP threshold model x = x₀ + Δ, where x₀ is the acceptable performance gap between predicted and genie beams, and Δ accounts for RF-induced uncertainty in RSRP measurements and it should be derived statistically from simulations modeling realistic RF error distributions and correlation across Tx beams.

Below, we provide our inputs for selecting an appropriate number for N and discuss if we need N>1.

As AI/ML models are integrated into NR beam management, evaluating beam prediction accuracy based only on the Top-1 beam we believe is insufficient to ensure robustness, reliability, and RRM-safe behavior. For both Case 1 (Model-Ranked) and Case 2 (Probe-Ranked), increasing the evaluation scope to N ≥1 beams could significantly enhance the practical value of conformance testing without increasing complexity in any prohibitive way. Therefore, we provide the following reasons in favor of N >= 1

#### 1. Beam diversity and fallback are fundamental in real deployments

RAN systems often maintain a list of backup beams for redundancy for example, in PDCCH duplication, DL recovery, or fast beam reselection after blockage. Evaluating only the Top-1 predicted beam does not verify the quality of these additional fallback beams. A model that always predicts the best beam correctly but fails to identify the second- or third-best beams (i.e., poor Top-N accuracy beyond rank 1) would pass an N=1 test but perform poorly in real-world scenarios where conditions change rapidly.

#### 2. N > 1 ensures robustness against beam blockage, occlusion, or mobility transitions

At mmWave and FR2 frequencies, beam quality can degrade rapidly due to UE hand movement, body shadowing, or any environmental dynamics. If only the strongest predicted beam is tested (N=1), a sudden degradation can leave the device with no reliable alternative unless other beams were also predicted accurately. Testing Top-2 or Top-3 beams could provide some more assurance that the AI model has a good view of the channel and not a narrow overfit to a single path.

#### 3. N > 1 validates candidate set coverage more thoroughly

In Case 2, the UE reports a Top-K unranked beam set which is then probed and re-ranked. If we test only whether the Top-1 genie beam is present, a model that always includes just the strongest beam (and two irrelevant ones) can still pass. But in real deployments, the network expects the Top-K list to contain multiple strong candidates, not just a single useful beam. Requiring that the Top-N genie beams are covered (e.g., N=2 out of K=3) ensures that the model consistently proposes a useful beam pool, not just a lucky hit.

#### 5. Measurement and computational overhead is minimal

From a testing perspective, increasing N from 1 to 2 (or even 3) introduces almost no additional cost:

- All beam RSRPs (for ground truth and predictions) are already captured during testing

- Rank comparisons and RSRP gap checks are considered post-processing operations , no extra over-the-air signalling or test configuration is needed.

- Evaluation can be batch-processed across the same dataset used for N=1.

We propose setting N > 1 when evaluating Top-K beam prediction accuracy to reflect realistic gNB usage of multiple fallback beams. Considering more than one strong beam improves robustness, and avoids overfitting to a single beam index. This approach aligns with diverse deployment scenarios and enables more resilient AI/ML evaluation.

For Case 2 ("Measured-Ranked Top-K"), the AI/ML model is responsible only for predicting a candidate set of beam indices. The ranking of those beams and any RSRP values associated with them comes from actual measurements performed by the UE. Since the UE measures the RSRP after the gNB transmits the predicted beams (e.g., through CSI-RS or SSB probing), there is no need for the model to predict RSRP values. As a result, all ranking and reporting are based on physical-layer truth, making RSRP prediction redundant in this case.

This simplifies model design and avoids testing additional RSRP accuracy metrics. The focus shifts entirely to whether the predicted beam set includes the strongest beams (i.e., beam ID prediction accuracy), not how well the model approximates signal strength. Therefore, RSRP prediction accuracy should not be a test requirement in Case 2.

RSRP prediction accuracy should not be a test requirement in Case 2.

### One key question we must address is whether a separate RSRP accuracy test is needed in addition to the beam ID prediction test in Case 1. Below we provide our inputs:

For Case 1 (Model-Ranked): The UE reports ranked beam IDs and predicted RSRP values. If RSRP values are reported, a separate absolute RSRP accuracy test is required to validate their reliability. This is because beam ID accuracy (i.e., selecting and ranking the right beams) does not guarantee that the predicted RSRP values are correct. In fact, a model may get the ranking right while being systematically biased in RSRP, which can lead to incorrect RRM decisions (e.g., misconfigured event thresholds, premature handovers, or failed power control).Thus, beam ID and RSRP accuracy serve complementary purposes and must be tested independently if both outputs are used.

Conversely, if we evaluate only RSRP prediction accuracy, is there still a need to separately test for beam ID prediction accuracy (Case 1)? We are trying to answer this question below:

Even if the UE reports accurate predicted RSRP values for the beams it selects, RSRP prediction accuracy alone may not be sufficient to guarantee correct beam selection. This is because the accuracy check is only applied to the RSRP values of the predicted beams that is, we compare the predicted RSRP of beam ID-i to the measured (genie) RSRP of the same beam ID-i.  However, this test alone does not verify whether the beams chosen by the UE are actually the best ones in the scene. A model can pass the RSRP accuracy test while still failing to include the true Top-K beams, the beams with the highest measured RSRPs  in its prediction set. However, if we can guarantee that the RSRP beam prediction accuracy meets the RSRP measurement accuracy as in legacy, then we could argue that we don’t need RSRP beam prediction accuracy since the same issue is already part of legacy (due to measurement errors)

We present an example below where we assume a measurement accuracy of ∆=+-6dB to draw an analogy to legacy measurement accuracy. In this example:

- The  Genie Top-3 (truth): beams 2, 5, 7 (−52/−53/−54 dBm).

- Legacy measured Top-3: still 2, 5, 7 (ranking preserved with small errors).

- AI/ML predicted Top-3: beams 3 (−54), 4 (−55), 10 (−55) , all are within ±6 dB on their own genie values, yet the model omits the true Top-3. We can see that even though the RSRP prediction accuracy is satisfied the top-K predicted beams are omitted.

The fundamental difference between legacy and AI/ML is the following:

In legacy, it’s guaranteed to pick the best beams because of one simple fact: The UE measures the RSRP of every candidate beam directly before making its Top-K list.

We can make the following observations:

- Every beam’s true signal strength (within measurement error bounds, e.g., ±Y dB) is known.

- The UE’s ranking is based on these direct measurements, so the strongest genie beams will almost always appear in the reported Top-K, unless the measurement error itself is so large that it changes the order, which is statistically rare if Y is small.

So in legacy, RSRP accuracy requirements on the reported beams are almost equivalent to ensuring beam-ID correctness, because the list was built from actual measurements over all beams, not predictions.

It doesn’t hold in AI/ML because the model never measures all candidate beams before making its Top-K selection, it predicts them from a subset of inputs (e.g., Set B beams)

We can make the following observations:

- The model only produces predicted RSRP values for the beams it chooses to output.

- The RSRP accuracy requirement is then checked only on those predicted beams, not on the full beam set.

- A strong genie beam that the model never considered won’t be part of the accuracy check, so it can be completely missed , even if the reported beams all pass the ±Y dB RSRP accuracy requirement.

It’s possible, and maybe quite easy for an AI/ML model to miss the strongest beams because of how it infers rather than measures:

- Partial or indirect input information

  - The model doesn’t “see” the full set of beams; it predicts Set A beams from Set B beams or other features.

  - Measurement errors in Set B can scramble the relative ordering of beams that would otherwise map well to Set A, leading the model to miss the true strongest beam.

- Prediction error distribution

  - AI/ML models don’t make uniform small errors, they can make large, localized mistakes.

  - A single mapping error or underprediction can push the strongest genie beam’s predicted RSRP far below the Top-K threshold, so it never gets selected.

- Mismatch between training and deployment : The mapping learned during training assumes the propagation environment, gNB configuration, and beam shapes remain consistent. In real deployment, these can change due to factors like new channel types, reconfigured gNB beams, or altered antenna patterns. While testing is done under the training assumptions, real-world mismatches can cause the model to overlook even the strongest beams. This means a model that passes the lab test could fail to deliver the same accuracy in the field.

In legacy (non-AI/ML) beam management, this problem essentially doesn’t occur, because the UE measures all relevant beams directly in real time and always selects the beam with the highest measured RSRP at that moment.

However if we could truly guarantee that the predicted RSRP accuracy in AI/ML is the same as the measured RSRP accuracy in legacy, for every predicted beam the UE reports, then in principle, a separate beam prediction accuracy metric would not be strictly necessary.

This is because for the following reasons:

- In legacy, measuring all beams and picking the Top-K ensures that the reported beams are indeed the ones with the highest true RSRP at that moment.

- In AI/ML, if the predicted RSRPs are as accurate as real measurements in legacy, then sorting them will produce the same Top-K beams as legacy, meaning beam-ID accuracy would be implicitly satisfied.

If we could truly guarantee that the predicted RSRP accuracy in AI/ML is the same as the measured RSRP accuracy in legacy, for every predicted beam the UE reports, then in principle, a separate beam prediction accuracy metric would not be strictly necessary.

If RSRP prediction accuracy cannot be guaranteed, falling back to Case 2  is a practical safeguard for the following reasons.

- Case 2 does not depend on the predicted RSRP values for ranking; it only checks whether the UE’s Top-K predicted beams include the strongest genie beams, with the ordering determined offline using genie RSRPs (during test) or extra measurements (in deployment)

- This removes the risk that poor predicted RSRP values will distort the ranking in real time, while still ensuring that the UE points the network toward beams that are genuinely strong.

- It effectively says: “We don’t fully trust your RSRP predictions, so we will only validate whether your beam shortlist overlaps with the true strongest beams.”

Therefore, we are proposing the following flowchart for the test procedure for AI/ML BM as shown in the flow chart below:

RAN4 is invited to adopt a unified AI/ML beam management testing framework that categorizes UE performance outcomes according to the reliability of both RSRP prediction accuracy and beam ID prediction accuracy. If a UE reports only beam IDs, Case 2 is applied, where success is evaluated using the weighted Option 3 formula. If a UE reports RSRP values and its RSRP prediction accuracy meets or exceeds legacy measurement accuracy, only RSRP prediction accuracy is tested. If RSRP accuracy cannot be guaranteed, testing falls back to Case 2 to ensure robustness in beam selection.

In Case 2 (Probe-Ranked Top-K), it is generally beneficial to set K > N, where K is the number of beam indices predicted by the AI/ML model and reported by the UE, and N is the number of strongest genie beams checked during testing. Since the UE ranks the beams only after actual RSRP measurements are collected through gNB probing, a larger K provides the AI model with more flexibility to include the correct N strongest beams, even if its predictions are imperfect. This setup enhances robustness against prediction errors, supports beam diversity for fallback scenarios, and reduces the likelihood of failing the Top-N accuracy check. Additionally, it aligns with real-world deployments where the network expects multiple good candidates for beam management, while keeping the measurement overhead reasonable.

For Case 1, where the UE reports a ranked Top-K list of beam indices directly from the AI/ML model, it is both sufficient and optimal to set K = N in conformance testing. Since the test metric evaluates only the Top-N predicted beams against the Top-N genie beams typically through ranking accuracy or RSRP gap comparisons any additional predicted beams beyond rank N (i.e., K > N) are excluded from the test and have no influence on the result. Predicting more than N beams does not improve test performance, yet increases model complexity and potentially introduces confusion in interpreting test results. By aligning K with N, we can ensure clarity in the test interpretation, and avoid unnecessary prediction or signaling overhead. Therefore, we recommend adopting K = N as the default for Case 1 testing.

RAN4 to adopt beam ID prediction accuracy (Case 2) as the sole conformance metric when the UE reports only beam indices. The test shall be performed offline using the genie RSRP values for all beams in Set A, with no re-measurement required. For each snapshot, the genie Top-N beams are identified, and the UE’s Top-K predicted beams are compared against them. A predicted beam is considered a match to a genie beam if its genie-RSRP is within x = x₀ + Δ dB of the genie beam’s RSRP.

For Case 2 (Probe-Ranked Top-K), we recommend K > N, allowing the UE to provide a broader beam pool for post-measurement ranking. A slightly larger K (e.g., K = N+1 or N+2) improves robustness and fallback without adding significant overhead during test

- When N > 1, a set of weights may also be applied to reflect the relative importance of the ranked beams. A prediction is considered successful if the weighted sum of the Top-N predicted beams based on whether each beam’s RSRP is within x = x₀ + Δ of the corresponding genie beam exceeds a defined threshold (e.g., normalized sum ≥ 1). This parameterized formulation is general and flexible: all other beam prediction accuracy definitions used in RAN4 to date can be derived as special cases by selecting appropriate values of N, K, x₀, Δ, and the weighting function. This enables consistent and configurable evaluation across different use cases and input quality assumptions.

Generalization Aspects

In the RAN4#114-bis meeting, the following agreement was achieved on the generalization aspects.

Relation between generalization testing and additional conditions signalling Agreement: - There shall be consistency between applicable conditions signalling and testing - No test will be defined that implies a change of network condition or associated ID during the test. Non-static scenarios Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario

In this section we present an end-to-end simulation blueprint for a RAN4 study aimed at resolving generalization problems in AI/ML-based BM.

Problem statement and study objective
AI/ML beam-selection will be trained and benchmarked in a single “friendly” lab environment. However, the live network changes band, SINR (deployment scenario), UE speed and channel type, and the prediction accuracy can collapse. RAN4 therefore needs proof that AI/ML prediction can generalize across the full radio diversity operators deploy. The proposed study will compare three different approaches and their underlying models

  - Universal model: one large neural network trained on every dataset it sees, with no specialization.

  - Per-slice model: a completely separate full-size network for every deployment slice, guaranteeing accuracy but at huge memory and certification cost.

  - Backbone + Experts: the proposal under test, where a single physics-aware backbone is frozen and each slice adds only a small adaptation layer. The goal could be to show that Backbone + Experts meets RAN4 KPI thresholds and beats the other two on size, while meeting generalization.

To judge generalization we must move the radio “knobs” that truly shape propagation. This slice universe lets us create controlled yet diverse scenarios. By training on one set of slices and testing on others, we can measure how well an AI/ML model transfers its learning when any single axis or several at once changes.

Defining the slice universe
Generalizations can only be judged if we deliberately change the radio “knobs” that matter. We describe every deployment by a 5-axis code:

- Frequency layer (F) – low sub-6 GHz, high sub-6 GHz, low mmWave, high mmWave; four buckets capture two octaves of path-loss change.

- Antenna / beam class (A) – small arrays (≤ 8 TX), medium arrays (9–32 TX) and massive arrays (> 32 TX); this axis drives beam-width, antenna spacing, etc

- UE-speed tier (V) – pedestrian, city-traffic, high-speed rail; speed alters temporal correlation and Doppler. [vlow, vmedim, vhigh]

- Environment (E) – dense-urban street canyon, suburban macro, rural motorway and indoor hotspot; each has distinct scatterer density. CDL/TDL

- SNR (S): it could be defined based on ranges: Low, medium, high

Let’s consider one example where the whole set of slices as defined by the cartesian product is 4(F) × 3(A) × 3(V) × 4 (E) x 3 = 432 theoretical slices. RAN4 will never test all 432, but the taxonomy ensures every real-world cell can be mapped to a canonical slice code.

How to Choose the “training coverage” level (so that we test the generalization later):

### There are two practical strategies for down-selecting slices before we start training:

1 – “Exploratory” (very low budget, ≈ 8 slices)
When we just need a quick sanity check, anchor each axis at its two extremes and cross them:

- Frequency: choose the lowest band we operate and the highest (e.g., 47 GHz).

- Antenna size: take the smallest panel in the network (≤ 8 TX) and the largest massive-MIMO panel (> 32 TX).

- Speed: include a pedestrian trace (< 10 km h⁻¹) and a high-speed-rail trace (> 120 km h⁻¹).

- Environment: pick one outdoor macro setting (e.g., suburban, UMa) and one dense indoor/hotspot (InH).

- SNR:  pick low and high

The cartesian product 2 × 2 × 2 × 2 x 2= 32 slices captures the widest physics gap on every axis with the smallest possible dataset. We can use this dataset when we want a fast test to see if the modelling concept holds.

2 “Balanced” (mid-size)
Sample slices are data-driven rather than by eyeballing the extreme values:

- Encode every one of the all possible slices as a numeric vector
[ freq_GHz,  log2(#TX),  speed_km h−1,  env_id , SNR]

- Cluster them with K-means (choose K).

- Attach a weight to each slice if available

- Rank clusters by the summed weight of their members and keep the centroids of the top K clusters.

This way we end up with a modest-sized training set that is (a) evenly spread across the physics space thanks to K-means distance minimization and (b) skewed toward the slices that will actually be more beneficial according to some predefined criteria.

Below we are discussing a simulation workflow for addressing the generalization challenge and evaluating different models.

### Option 1: Universal model: single network for every slice

In the simulation we first evaluate a naïve baseline by training a single universal model on an aggregated dataset that combines logs from all the chosen training scenarios. To prevent dominant slices from overshadowing rare ones, we apply balanced mini-batching with equal sampling from each slice in every training step. The model is optimized over many epochs using AdamW and an appropriate learning rate. This simulation produces one checkpoint. Checkpoint refers to a saved snapshot of the trained AI/ML model on the aggregated dataset at a given point during or after training. It contains the learned parameters (weights and biases) of the model, as well as optimizer states if needed.

### Option 2: Many stand-alone models:  one full network per slice

### In this simulation setup, we also evaluate the per-slice training approach, where a separate full model is trained individually on each slice of the dataset. For each slice scenario, we create an independent training dataset using only the dataset from that specific slice. The model is trained on this single-slice data with standard mini-batching, optimized over many epochs using AdamW with an appropriate learning rate. At the end of training, we save a checkpoint of the model for each slice. These checkpoints are later used for offline KPI evaluation to assess how well models perform when trained exclusively on one slice and tested across others.

### Option 3: Shared backbone + tiny slice-specific experts

In this simulation setup, we also evaluate a parameter-efficient approach using a shared backbone combined with small slice-specific experts. A single backbone model is trained to capture general radio characteristics across all slices. For each individual slice, we attach a lightweight-adapter and fine-tune only this adapter while keeping the backbone frozen. Training follows standard mini-batching over many epochs with AdamW and a appropriate learning rate. After training, we save the common backbone along with the set of slice-specific adapters, which are later used for offline KPI evaluation. This method aims to balance generalization with slice-specific adaptation while keeping the total model size compact.

- Stage A – learn backbone Φ

  - Train on the pooled data exactly like the universal case.

  - Freeze Φ once validation loss stabilizes

- Stage B – fine-tune for all adapters

  - For each slice k: attach a fresh adapter Ψₖ, train just 2-3 epochs on that slice, freeze, export

  - Repeat until all adapters are done.

- Simulation outputs: one backbone file + (number of chosen slices) adapters.

Take-away for the simulation workplan

- Universal model tests raw generalizability with minimal engineering effort.

- Stand-alone models give an upper-bound on accuracy but are heavy.

- Backbone + experts aims for near-upper-bound accuracy at a fraction of the storage and training cost, exactly what we want to quantify before any formal RAN-4 test plans are drafted.

In order to describe the evaluation methodology for generalization, we make the following definitions:

### Seen Slices = Slices used for training, but tested on held-out (unseen) data

- During training, we use part of the data from each of the Seen Slices (e.g., 80% for training).

- For testing, we evaluate the model on the remaining 20% (or any split) of data that was never used in training from those same slices.

- This gives us a valid estimate of in-distribution generalization testing whether the model learned meaningful patterns rather than memorizing the training data.

### Unseen Slices = Slices never used during training, fully reserved for testing

- Unseen Slices are slice-types (combinations of deployment parameters) that are entirely excluded from the training phase.

- Neither the model nor any of its components (universal, stand-alone, or expert) have been exposed to any data samples, patterns, or statistics from these slices during training.

- The model is evaluated on fresh data drawn from these Unseen Slices, representing truly out-of-distribution (OOD) testing.

The NMSE performance on Unseen Slices directly measures the model’s ability to generalize to deployment scenarios it was never trained on. This simulates real-world robustness when a trained model encounters new environments, frequencies, or mobility patterns.

Let:

- J= Index set of Seen Slices (used for training, tested on held-out data). Measures in-distribution performance on known slice types.

- O = Index set of Unseen Slices (never used for training, fully held out). Measures out-of-distribution performance on unknown slice types.

The total number of NMSE test error values is:

Nt=Nseen+Nunseen=J+O

Let u∈RNt be the NMSE row-vector for the universal model; element uj corresponds to test file or slice j.

Let S∈RNseen×Nt  denote the NMSE matrix for the fully stand-alone fleet, so that Sij is the error obtained when stand-alone model i is applied to test slice j

Let E∈RNseen×Nt  be the analogous NMSE matrix for the backbone + expert system.

We can use the following aggregated statistics to judge generalization:

Universal model averages

NMSEseenU=1Ij∈Iuj,   NMSEunseenU=1Oi∈Oui,  and define the generalization gap as:

ΔU=NMSEunseenU-NMSEseenU. (scalar)

Stand-alone diagonal and cross averages

We train Nseen​ separate models one model per Seen Slice

- We evaluate each trained model on all slices in the testing set:

  - The Nseen slices (in-distribution)

  - The Nunseen​ slices (out-of-distribution)

Resulting in a Nseen×(Nseen+Nunseen)  NMSE matrix S∈RNseen×Nt

The diagonal entries Sii indicate the accuracy on Seen Slices (Self-Performance): each model tested on its own slice (e.g., Model 5 on Slice 5). We want low NMSE on the diagonal, showing the model fits well to its trained slice. This validates in-distribution fitting.

Off-diagonal entries ​Sij, i≠j, Cross-Slice Generalization: Model i tested on a different slice j.

- We can analyze the following:

  - If off-diagonal NMSE is high:
→ The stand-alone model overfits its own slice and generalizes poorly.

  - If off-diagonal NMSE is low:
→ Some generalization exists across slices.

The goal of this study is to answer the following question: How many stand-alone models would be needed to “cover” the full set of possible slices with acceptable performance and by coverage we mean low NMSE.

### If we see: Diagonal ≪ Off-Diagonal NMSE (Poor generalization)

- If Diagonal NMSE ≪ Off-Diagonal NMSE, this shows:

  - Each stand-alone model performs well on its own slice

  - But generalizes poorly to other slices (seen or unseen)

- This means:

  - Models are highly slice-specific

  - We may need a large number of specialized models: possibly close to one per slice

  - The stand-alone modeling approach does not generalize efficiently

### If we see: Diagonal ≈ Off-Diagonal NMSE (Good generalization)

- If we observe Diagonal NMSE ≈ Off-Diagonal NMSE, that suggests:

  - Each model can generalize well to slices beyond its training slice

  - We may need fewer models (e.g., grouping slices into clusters)

  - The system could work with shared models across similar slices

### Let’s provide an example with numerical values (assume 144 total slices):

- If we have 16 stand-alone models which fail on most of the 4 unseen slices (high off-diagonal NMSE),
→ We cannot expect the 16 models to cover the other 144 – 16 = 128 slices
→ We would likely need many more models or a better generalization strategy (e.g., Backbone + Experts)

- If some stand-alone models perform well on multiple unseen slices,
→ We might cluster slices based on those generalization patterns
→ Estimate coverage as 1 model per generalizable cluster rather than per slice

### We can use the following rule of thumb Interpretation:

- Diagonal NMSE ≪ Off-Diagonal NMSE
→ We need ~1 model per slice (poor generalization)

- Diagonal NMSE ≈ Off-Diagonal NMSE (for both seen and unseen slices)
→ We can cover multiple slices with fewer models (good generalization)

### What Stand-Alone Models tells us: they tell us how specialized each model must be to achieve good performance on its own slice. However, by testing off-diagonal NMSE, we could see how poorly a slice-specific model performs on other slices. Therefore:

- If off-diagonal NMSE is high, stand-alone models suggest we need one model per slice

- If off-diagonal NMSE is low in clusters, we might cluster slices and reduce the number of models.

However:

- The stand-alone study doesn’t tell us whether you can share a backbone between slices.

- Nor does it inform whether lightweight specialization (experts) is enough for good performance.

If the study with standalone models show poor generalization performance, then we can adopt a better generalization strategy explained below:

### Backbone + Experts Simulation Study:

- The Backbone + Experts approach tests whether a single shared backbone can generalize common features across slices, while a small slice-specific expert handles the specialization.

- We employ the following training procedures:

  - One backbone model on all Seen Slices mixed dataset.

  - One expert per Seen Slice (small parameter set and few iterations).

- Employ the following test procedures:

  - Test how well the backbone + expert combination works on:

    - The Seen slices (with their own expert)

    - The Unseen slices (with a random expert or no expert possibly fallback)

- If Backbone + Experts generalize well:
→ We may only need a single backbone and a small set of experts for similar slice groups.
→ This reduces total models dramatically (e.g., 1 backbone + ~10 experts instead of ~100 full models).

- If Backbone + Experts fail on unseen slices:
→ The radio domain doesn’t support generalization well.
→ We may still need many specialized models (but at least now we know).

### Therefore the bottom line and conclusions from above discussion are:

- Stand-alone models = Worst case upper bound on model count (likely 1 per slice if no generalization).

- Backbone + Experts = Test if we can collapse models to a small number of shared structures.

Based on the above analysis we have the following proposal:

- RAN4 is invited to study the model robustness across scenarios of AI/ML models for beam management (BM) by evaluating three training approaches: (1) a universal model trained on a combined dataset of multiple deployment scenarios, (2) stand-alone models trained per scenario, and (3) a shared backbone model with lightweight scenario-specific experts. The study will assess performance on both training scenarios (Seen) and scenarios not used in training (Unseen) through systematic testing across diverse deployments. The outcome will clarify whether BM requires many specialized models or if a shared architecture with minimal specialization can handle realistic deployment variations, guiding model deployment strategies and standardization in RAN4.

Conclusion

In this paper we share our views on RRM performance requirements for AI/ML enabled beam management. Specifically, we cover following aspects:

- When we pick the Top-1 predicted beam, we are not just selecting the strongest candidate, we are also favoring the one that happened to be overestimated due to random noise in the predictions. This means the “winner” often looks slightly better in the test results than it is really in reality.

If we could truly guarantee that the predicted RSRP accuracy in AI/ML is the same as the measured RSRP accuracy in legacy, for every predicted beam the UE reports, then in principle, a separate beam prediction accuracy metric would not be strictly necessary.

- RAN4 is invited to adopt and focus on a modified Case 3a simulation setup using error-free ground truth labels to isolate model performance from training noise. Additionally, we propose plotting full error PDFs (e.g., RSRP_pred − RSRP_true) to identify bias, skewness, or non-Gaussian behavior, rather than relying solely on percentile metrics. This supports robust AI/ML evaluation and simulation alignment.

In order for RAN4 to meet the P90(|error|) ≤ 6 dB requirement (FR2), it is proposed to (i) reduce SNR-sensitive errors by ensuring higher-SNR side conditions for the reference signals used in AI/ML inputs, (ii) further improve SNR through measures such as RS power boosting or 1–2 non-coherent repetitions, (iii) revisit RF assumptions by defining deployment RF profiles with tighter variance where feasible, and (iv) apply noise-aware training with augmentation matched to the declared RF/LLS error models to improve model robustness and reduce the baseline prediction error.

For UEs reporting only beam IDs, we propose an evaluation framework for beam index prediction accuracy in which the UE predicts a Top-K set of beam candidates via AI/ML, and the final ordering is determined offline using genie RSRP values. The metric checks whether the UE’s predicted Top-K set includes the correct beams, in the offline-determined order, for multiple beam positions, within an allowed RSRP margin (X) from the corresponding strongest genie beams.

RAN4 to adopt a beam prediction accuracy metric based on a RSRP threshold model x = x₀ + Δ, where x₀ is the acceptable performance gap between predicted and genie beams, and Δ accounts for RF-induced uncertainty in RSRP measurements and it should be derived statistically from simulations modeling realistic RF error distributions and correlation across Tx beams.

We propose setting N > 1 when evaluating Top-K beam prediction accuracy to reflect realistic gNB usage of multiple fallback beams. Considering more than one strong beam improves robustness, and avoids overfitting to a single beam index. This approach aligns with diverse deployment scenarios and enables more resilient AI/ML evaluation.

RSRP prediction accuracy should not be a test requirement in Case 2.

RAN4 is invited to adopt a unified AI/ML beam management testing framework that categorizes UE performance outcomes according to the reliability of both RSRP prediction accuracy and beam ID prediction accuracy. If a UE reports only beam IDs, Case 2 is applied, where success is evaluated using the weighted Option 3 formula. If a UE reports RSRP values and its RSRP prediction accuracy meets or exceeds legacy measurement accuracy, only RSRP prediction accuracy is tested. If RSRP accuracy cannot be guaranteed, testing falls back to Case 2 to ensure robustness in beam selection

RAN4 to adopt beam ID prediction accuracy (Case 2) as the sole conformance metric when the UE reports only beam indices. The test shall be performed offline using the genie RSRP values for all beams in Set A, with no re-measurement required. For each snapshot, the genie Top-N beams are identified, and the UE’s Top-K predicted beams are compared against them. A predicted beam is considered a match to a genie beam if its genie-RSRP is within x = x₀ + Δ dB of the genie beam’s RSRP.

For Case 2 (Probe-Ranked Top-K), we recommend K > N, allowing the UE to provide a broader beam pool for post-measurement ranking. A slightly larger K (e.g., K = N+1 or N+2) improves robustness and fallback without adding significant overhead during test

- RAN4 is invited to study the model robustness across scenarios of AI/ML models for beam management (BM) by evaluating three training approaches: (1) a universal model trained on a combined dataset of multiple deployment scenarios, (2) stand-alone models trained per scenario, and (3) a shared backbone model with lightweight scenario-specific experts. The study will assess performance on both training scenarios (Seen) and scenarios not used in training (Unseen) through systematic testing across diverse deployments. The outcome will clarify whether BM requires many specialized models or if a shared architecture with minimal specialization can handle realistic deployment variations, guiding model deployment strategies and standardization in RAN4.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.