3GPP TSG-RAN WG4 Meeting #116	R4-2509432

Bengaluru, India, August 25th – 29th, 2025

Source:	APPLE

Title:	On RAN4 Impacts for RRM measurement prediction in AIML Mobility

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis in the scope of Rel-19 SI ‎[1] and ‎‎[2]. Agreements achieved during this meeting are summarized in the WF ‎[3].

Some of the issues related to AI/ML Mobility RRM measurement prediction use case require further discussion. In this paper, we provide some additional views on issues related to RRM requirements a

Discussion

Performance Metrics

The following scenarios have been prioritized as high priority in RAN2:

| Scenario | Prioritization | Evaluation scenario combination | Objective | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | Measurement reduction | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | Measurement reduction | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | Mobility optimization | Intra-cell |

Cell Level Relative Accuracy

During RAN4#115 meeting, the following agreement was made on relative RSRP accuracy.

Issue 2-1: Relative RSRP accuracy definition

Agreement:

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 1 and cell 2 are on the same frequency

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

The relative error becomes the difference between the prediction errors of cell 1 and cell 2. The core of the issue is this: even if the ground truth is available and we're evaluating prediction error differentially, the validity of the comparison assumes that both predictions (or measurement + prediction) correspond to the same moment in time.

But in practice:

- If the prediction for cell 1 is for time t₁,

- And the measurement or prediction for cell 2 is for a different time t₂,

- Then their errors are being evaluated at different time contexts.

So even though ground truth cancels the biases in absolute RSRP levels, it cannot cancel the time skew if the prediction or measurement times differ. Any fast-fading, mobility, or dynamic environment changes between t₁ and t₂ will introduce artificial “error” in this difference, even if both models were perfect at their own time points.

Let’s say the UE is in motion and the channel conditions are rapidly changing (e.g., due to urban fading or beam switching).

Setup:

- Cell 1 is measured at time t₁.

- Cell 2 is predicted at a later time t₂.

- The goal is to compute relative RSRP accuracy between Cell 1 and Cell 2.

Ground truth and reported values:

At t₁ = 1000 ms:

- RSRP_Cell1_pred = −80 dBm (predicted)

- RSRP_Cell1_GT = −82 dBm (ground truth)

At t₂ = 1150 ms (Δt = 150 ms later):

- RSRP_Cell2_meas = −85 dBm (measured)

- RSRP_Cell2_GT = −87 dBm (ground truth)

In this example, Cell 1 is measured at time t₁ = 1000 ms, and Cell 2 is predicted at a later time t₂ = 1150 ms. At t₁, the predicted RSRP for Cell 1 is −80 dBm, while the ground truth is −82 dBm. At t₂, the measured RSRP for Cell 2 is −85 dBm, while the ground truth is −87 dBm. Using the relative accuracy formula, we compute:
[(−80) − (−85)] − [(−82) − (−87)] = 5 − 5 = 0 dB.
This yields a relative error of 0 dB, which appears to be “perfect” and suggests that the comparison between Cell 1 and Cell 2 is accurate.

The problem is that this calculation assumes both RSRP values correspond to the same moment in time, but here they are taken 150 ms apart. In a fast-changing radio environment, such as when the UE is moving quickly, undergoing beam changes, or experiencing blockage, the channel conditions can shift significantly within that short time. As a result, Cell 1’s signal may have dropped well below Cell 2’s by t₂, even though the earlier comparison made at t₁ showed them to be in the same relative strength.

The risk is that a small relative error can give a false sense of confidence in the measurement or prediction. In this case, the system might wrongly conclude that Cell 1 remains stronger and delay a needed handover or hold the UE on a now weaker serving cell. The key takeaway is that relative accuracy is only meaningful if both inputs are time-aligned; otherwise, it can hide real changes in cell dominance and lead to incorrect mobility decisions.

Relative accuracy is designed to check how accurately the UE can predict or measure the difference in RSRP between two cells, regardless of any absolute bias in each individual cell’s level. If this difference matches ground truth closely, the relative accuracy error will be small, and the tester may conclude the UE’s predictions are reliable for ranking cells.

The hidden risk in the time-misalignment case:
If cell 1’s value is taken at t₁ and cell 2’s value is taken at t₂ (e.g., 150 ms later), the relative error can still be very small  giving a false sense of accuracy , even though the actual ranking of cell strengths at the same instant might have reversed. This can lead to incorrect mobility decisions, such as handing over to a cell that is no longer stronger when both are evaluated at the same time.

Relative RSRP accuracy evaluates how well the predicted difference between two cells matches the ground-truth difference, providing a measure of consistency between cell comparisons. While a small relative-accuracy error suggests that the prediction and measurement are mutually reliable, it does not reveal which cell is actually stronger. This creates a risk when the two values are taken at different times (t₁ for one cell and t₂ for the other): channel changes, mobility, or fading can cause the relative ranking of cells to change between these moments. As a result, even with perfect relative accuracy, the network could incorrectly conclude that one cell is stronger and make a sub-optimal handover decision.

A similar definition of relative RSRP accuracy exists in legacy, but it does not account for any time alignment. As per the FFS item on whether and how to consider the time difference between the two reported RSRPs when applying this definition, RAN4 should clarify what differs in the AI/ML case compared to legacy.

The problem under FFS concerns whether and how the time difference between two RSRP values, either measured or predicted, should be considered when applying the definition of intra-frequency L3 RSRP relative accuracy. In practice, the two RSRPs used in the accuracy formula may be reported or predicted at different times, due to the filtering nature of L3 measurements and the asynchronous nature of reporting or prediction instances.

This matters because the radio environment can vary significantly over time, especially in scenarios with UE mobility, dynamic interference, or rapid channel variations. If two RSRP values are compared without accounting for their time difference, the resulting relative accuracy could be skewed by changes in the environment rather than reflecting the actual prediction performance.

This opens a critical question for further study (FFS) in RAN4 whether the test specification should require strict time alignment of the two RSRPs or allow for a defined maximum time difference between them. Resolving this is key to ensuring the validity and fairness of the relative accuracy assessment for AI/ML-based mobility prediction models.

Therefore, the key considerations FFS (For Further Study):

- Should the definition of relative RSRP accuracy enforce that both RSRP values be time-aligned (measured or predicted at the same or near-same time)?

- If not perfectly time-aligned, should there be a defined maximum allowed time difference (Δt) between them?

- Does the time difference tolerance need to vary by scenario (e.g., fast-moving UEs vs stationary)?

- How does this impact testing in RAN4 when validating prediction performance?

Below we list various options for enabling the discussion on this FFS issue:

Option 1 – Strict time alignment (almost zero tolerance):
Require that both RSRPs (predicted or measured) used in the relative accuracy calculation correspond to the same timestamp or originate from overlapping measurement/prediction windows within a very small time margin (e.g., X ms).
Pros: Clean KPI interpretation; ensures comparison reflects only prediction error and not channel variation.
Cons: May limit flexibility in asynchronous reporting scenarios; harder to meet in mobility cases with different reporting triggers.

Option 2 – Fixed maximum Δt (some tolerance):
Permit a controlled maximum time difference between the two RSRPs (e.g., Δt ≤ 100 ms), excluding any pair beyond this limit from the KPI calculation.
Pros: More practical in mobility cases where perfect time alignment is difficult; avoids excessive data loss when reports are slightly misaligned.
Cons: Requires careful calibration of Δt value to balance fairness and feasibility; residual channel changes within Δt may still bias results.

Option 3 – Scenario-dependent Δt:
Set the maximum allowable time difference based on the mobility scenario or UE speed class (e.g., ≤ 50 ms for high-speed rail, ≤ 200 ms for pedestrian/stationary).
Pros: Matches time-alignment tolerance to realistic radio dynamics; avoids overly strict limits in slow-changing environments.
Cons: Adds complexity to test specifications; requires reliable classification of UE speed/mobility category during testing.

Option 4 – Post-processed time compensation (if Δt is known):

This option allows comparisons of RSRPs with larger time differences by applying a correction factor or error budget to account for the expected RSRP change over Δt. The intent is to maximize usable data and reduce bias caused by timing offsets without forcing strict exclusions. The approach can be tailored for different mobility profiles, with the trade-off that it requires accurate modeling of RSRP time variation. This modeling may be as simple as an empirical bound on the typical RSRP change per unit time in a given scenario. For example: if the absolute time difference is within a small threshold, the raw RSRP difference is used directly; if it exceeds that threshold, the expected variation margin is subtracted (e.g., Relative Error = Raw Error − (α ⋅ Δt) or a more advanced model). The risk is that errors in this model can propagate into KPI evaluation.
A key question is whether Δt can be reliably known. In principle, if either the UE or gNB timestamps each reported or predicted RSRP, then the time difference can be computed as Δt = |t_RSRP1 − t_RSRP2|. For measurement reports such as L3 RSRP, the UE’s reporting is aligned to the L3 filter output, and Δt can be derived from timestamps or from the reporting periodicity. For AI/ML-based predictions, the prediction process would need to produce timestamped outputs, which can be built into the design,for instance, aligning predictions to the same occasions as measurement reporting.

The challenge arises with asynchronous events, such as when one RSRP comes from a periodic report and the other from an event-triggered prediction. Without explicit timestamp tagging, aligning these may be difficult. Therefore, if RAN4 chooses to allow relative RSRP accuracy KPIs with Δt > 0, the conformance framework must either (a) require timestamp availability, or (b) define reporting periodicity and alignment rules that implicitly bound Δt. This ensures that any post-processed compensation is based on a known and verifiable time offset.

RAN4 is invited to consider that if the definition of intra-frequency relative RSRP accuracy allows for a non-zero time difference between the two RSRPs used in the calculation, either explicit timestamps should be required as part of the conformance testing framework to ensure traceable and bounded time alignment, or the reporting periodicity and measurement alignment rules should be clearly defined to implicitly control the maximum allowed time difference (Δt). This will ensure that any variation in the relative accuracy metric reflects the prediction or measurement performance, rather than uncontrolled environmental changes due to asynchronous reporting or prediction instances.

RAN4 is invited to study mechanisms for handling time differences between the two RSRP values used in intra-frequency relative RSRP accuracy calculations. The following options are proposed:

Option 1: Enforce strict time alignment, requiring both RSRPs to be measured or predicted at the same or near-identical time.

Option 2: Define a fixed maximum Δt threshold, excluding RSRP pairs exceeding this limit from KPI evaluation.

Option 3: Use scenario-dependent Δt thresholds based on UE mobility class or deployment scenario to better match realistic radio dynamics.

Option 4: Permit post-processing compensation for larger Δt by applying a bounded correction factor that accounts for expected RSRP variation over time, allowing more relaxed alignment constraints while preserving KPI validity.

Beam Level Relative Accuracy

From RAN4#115 we concluded on FFS ” Relative RSRP accuracy for Beam level measurements “

The relative beam-level RSRP accuracy could be defined as the difference between the predicted (or measured) beam-level RSRP of a serving or neighbor beam and a reference beam, compared to the corresponding difference in their ground truth RSRP values, where both beams belong to cells operating on the same frequency layer. Mathematically: [Predicted RSRPBeam 1-Reported or Predicted RSRPBeam 2-Ground Truth RSRPBeam 1-Ground Truth RSRPBeam 2]

Beam 1 and Beam 2 are two beams of interest, either from the same cell or from different intra-frequency neighbor cells.

It remains for further study (FFS) how the testing of relative RSRP accuracy should extend to beam-level RSRP measurements or predictions within the same cell because several open technical questions and ambiguities have not yet been fully addressed by RAN4 or RAN2, including:

The study of relative RSRP accuracy at the beam level introduces complexities beyond those encountered at the cell level. The proposed definition, comparing the difference between the predicted (or measured) RSRP of Beam 1 and Beam 2 against the same difference in their ground truth values , is conceptually straightforward. However, beams represent much narrower spatial channels than cells, meaning their RSRP values vary far more rapidly due to finer angular resolution, local multipath changes, and small-scale fading effects. This makes beam-level dynamics inherently more volatile than cell-level measurements, raising concerns that accuracy thresholds or filtering rules designed for cell-level KPIs may not translate directly. Without accounting for these faster fluctuations, a beam-level relative accuracy metric could unfairly penalize otherwise correct predictions or fail to reflect their operational usefulness in beam management.

A further challenge lies in the ambiguity of the reference definition. At the cell level, the reference is typically unambiguous, one serving cell compared against a neighbor. At the beam level, the choice is less clear. Should the reference always be the serving beam within the serving cell, or could it be any beam in the same cell? Should the metric consider “best-beam vs. secondary-beam” comparisons within a cell, or only “best beam” comparisons across cells? Each choice leads to different statistical characteristics and potential biases in the KPI. For example, measuring relative accuracy between best and secondary beams may highlight intra-cell angular prediction fidelity, while using only best-beam references may emphasize inter-cell competitiveness in beam selection. This ambiguity needs resolution to ensure a consistent and interpretable KPI definition.

Temporal and filtering misalignment risks are particularly acute at the beam level. Because beam-level RSRP can change significantly over tens of milliseconds, even small reporting or filtering misalignments between Beam 1 and Beam 2 can distort the relative accuracy calculation. Cell-level metrics are often tolerant of looser alignment because large-scale fading and shadowing dominate slower variations, but beam-level signals are much more sensitive to instantaneous multipath changes. Therefore, the question arises whether beam-level relative accuracy should mandate stricter time alignment or specific L1/L3 filtering definitions to prevent misinterpretation. If these aspects are not tightly specified, the resulting KPI may conflate true prediction errors with artefacts of timing and filtering mismatches.

Finally, beam-level relative accuracy intersects with the dynamic nature of beam switching behavior in NR. UEs, especially in FR2 with narrow beams, may switch serving beams every tens of milliseconds in response to channel changes or gNB scheduling. This raises the question of how prediction accuracy should be validated: should it be assessed against the instantaneous best beam at the reporting time, or should a tolerance window be allowed given the high volatility of beam choices? For AI/ML models, a prediction that is correct at Time t₀ might no longer be valid at Time t₀ + Δt simply due to mobility or beam schedule changes, even if the model’s underlying spatial inference was sound. Determining an acceptable “lookahead” interval, and whether the KPI should penalize predictions for failing to track rapid beam changes, is critical for defining a fair and meaningful beam-level relative RSRP accuracy metric.

RAN4 is invited to study the definition and testing of relative RSRP accuracy at the beam level for intra-frequency scenarios, focusing on four key areas: (1) determining whether existing cell-level thresholds, filtering, and reporting rules are suitable for the faster dynamics of beam-level RSRP or if tighter beam-specific definitions are needed; (2) clarifying reference beam selection, including whether it should always be the serving beam, any beam in the same cell, or a best-vs-secondary beam comparison; (3) assessing the need for stricter time alignment or Δt compensation methods given rapid variability of beam-level channels; and (4) defining how KPI evaluation should account for frequent intra-cell beam switching

Inter-frequency Relative Accuracy

In RAN4#115 the following agreement was reached:

- For inter-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

The agreed definition of relative accuracy for inter-frequency L3-RSRP compares the RSRP difference between two cells, one on the serving frequency (cell 1) and one on a different frequency but in the same FR (cell 2), against the corresponding ground truth difference. This approach is designed to cancel out common-mode errors such as large-scale fading bias or calibration offsets, focusing the KPI on the UE’s ability to preserve the relative ranking between the two frequencies. The reported RSRP of cell 2 can come either from direct measurement or from prediction, depending on whether an inter-frequency measurement gap was taken.

However, the definition leaves open a For Further Study (FFS) item regarding how to handle time difference (Δt) between the two reported RSRPs. In practice, inter-frequency measurements and predictions are not always time-aligned, cell 1’s RSRP may be current while cell 2’s RSRP could come from a prediction made several milliseconds earlier (or later), depending on measurement gap scheduling, prediction windows, and reporting triggers. If the UE is in motion or the radio environment changes rapidly, this time skew means the two RSRP values may reflect different channel states, undermining the assumption that the difference in ground truth at those times is directly comparable.

This misalignment creates a risk of misinterpretation in KPI evaluation. A small relative accuracy error could misleadingly suggest that the UE’s measurement or prediction performance is sound, even if the underlying time mismatch would cause the network to make a suboptimal mobility decision (e.g., handing over to a weaker cell). Conversely, a large error could reflect legitimate channel changes over Δt rather than poor prediction quality. Resolving whether to enforce strict time alignment, allow a bounded Δt, or apply Δt compensation is therefore essential to ensure that the inter-frequency relative accuracy metric reflects measurement/prediction quality rather than timing artifacts.

In inter-frequency scenarios, the measurement dynamics differ fundamentally from intra-frequency. Inter-frequency RSRP measurements depend on measurement gaps, which are scheduled periodically and can be separated by hundreds of milliseconds. This inherently creates larger time offsets (Δt) between serving cell and neighbor cell measurements compared to intra-frequency, where measurements can be taken continuously or at short periodic intervals. As a result, enforcing very tight time alignment requirements, as sometimes discussed for intra-frequency, may not be realistic without altering the basic measurement process.

Reporting behavior in inter-frequency is also more asynchronous. Neighbor cell measurements are only available when the measurement gap occurs, which may range across hundreds of milliseconds depending on RRC configuration and measurement gap patterns. Serving cell RSRP can be measured continuously or during separate configured gaps, leading to variable and sometimes large Δt between the two values used in the relative accuracy calculation. In contrast, intra-frequency measurements are often closely aligned in time, enabling much tighter timing constraints.

Below we provide two examples to demonstrate the risks of time misalignment:

Example A : Small error but wrong mobility decision (misleading “good” KPI):

- Times: t₁ = 1000 ms (serving-cell time), t₂ = 1300 ms (neighbor inter-freq gap).

- Ground truth: at t₁: C1 = −90 dBm, C2 = −94 dBm (C1 stronger by +4 dB). At t₂: C1 = −95 dBm, C2 = −88 dBm (C2 stronger by +7 dB).

- UE reports: Predicted C1 at t₁ = −89 dBm (≈GT), Measured C2 at t₂ = −87 dBm (≈GT).

- Inter-freq relative-accuracy per the current form (using GT at each report’s own time):

( −89−(−87))−( −90−(−88))=(−2)−(−2)=0 dB(−89−(−87))−(−90−(−88))=(−2)−(−2)=0 dB

KPI looks perfect (0 dB error). But if a decision is made at t₁, the instantaneous truth is C1 stronger by +4 dB(−90 vs −94). Comparing C1@t₁ with C2@t₂ falsely suggests C2 is stronger (−87 vs −89), risking a premature inter-frequency handover. Small KPI error, bad decision, because the two inputs came from different times.

Example B : Large error but due to timing, not model quality (misleading “bad” KPI):

- Times: t₁ = 1000 ms (serving), t₂ = 1300 ms (neighbor).

- Ground truth: at t₁: C1 = −90 dBm, C2 = −94 dBm (C1 stronger by +4 dB). At t₂: C2 rises to −88 dBm (channel improved); C1 at t₁ unchanged for this check.

- UE reports perfectly for their own times: Pred C1@t₁ = −90 dBm, Meas C2@t₂ = −88 dBm.

- If the KPI requires same-time GT (e.g., compares UE’s pair to GT at t₁):

UE pair: (−90−(−88))=−2 dB;GT@t1:(−90−(−94))=+4 dBUE pair: (−90−(−88))=−2 dB;GT@t1​:(−90−(−94))=+4 dB

Relative error = −2−(+4)=−6 dB−2−(+4)=−6 dB → large “fail.” Yet each report was accurate for its own time; the “error” comes from Δt = 300 ms and real channel evolution, not a bad predictor.

Therefore we can see that a small relative-accuracy error can mask a bad decision if the two cells’ values are taken at different times; a large error can merely reflect legitimate channel change over Δt, not poor prediction. Hence RAN4 must choose: enforce strict time alignment, allow a bounded Δt, or apply Δt compensation, so the KPI reflects prediction/measurement quality, not timing artifacts.

The use case for inter-frequency relative RSRP accuracy is also different. While intra-frequency relative accuracy supports fast, timing-sensitive operations such as handovers or beam switching requiring minimal Δt, inter-frequency use cases often focus on mobility preparation, gap reduction, or other processes where larger time differences are tolerable. In principle, AI/ML-based prediction logic could be designed to predict neighbor cell RSRP exactly for the time when the measurement gap occurs, but this would require explicit coordination between prediction triggering and RRC-configured gap scheduling.

RAN4 is invited to address the impact of time misalignment (Δt) on inter-frequency relative RSRP accuracy. Two risks are noted: (1) Small error masking poor decisions, KPI can appear perfect even if Δt would lead the network to select a weaker cell; (2) Large error from timing artifacts , KPI may show large deviations caused by real channel changes over Δt, despite each value being accurate at its own time. To ensure the metric reflects true prediction/measurement quality rather than Δt effects, RAN4 should study whether to (i) enforce strict time alignment, (ii) allow a bounded Δt with exclusions, or (iii) apply compensation models for larger Δt. Clear Δt handling rules are needed for conformance testing

Ground Truth Derivation

In the RAN4#114 meeting, the following agreements were made for FR1 Ground truth definition for RSRP accuracy:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Following agreement was reached in RAN4#114bis meeting:

Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Following is the RAN4#114 agreement for AIML Beam management usecase:

Agreement: - Use CDL-based channel model as starting point

Alternative 1, the ground truth for FR1 can be derived by TE based on the transmitted or received power:

Alternative 1 proposes deriving the FR1 ground truth (GT) from the transmitted or received power at the test equipment (TE), consistent with the legacy L3 RSRP accuracy method. This works well in the AWGN channel model used in past conformance tests, where signal power is constant over time and space. However, in fading CDL channels, which RAN4 has agreed to consider for FR2 beam management this method becomes problematic. The rapid variations in channel conditions make it difficult to align the exact time periods of measured and predicted samples, which can introduce GT inaccuracies.

Additionally, the TE does not have access to the UE’s proprietary L1 filtering parameters, such as filtering coefficients and the number of samples, further complicating accurate GT derivation. While the main advantage of Alternative 1 is that it is cheat-proof, since the UE cannot influence TE-based calculations, it is not well suited for AI/ML mobility in fading channels. The combination of channel variability and hidden UE filtering behavior can lead to significant mismatches between the derived GT and the UE’s actual measurement context.

Alternative 1, where the FR1 ground truth is derived at the TE from transmitted or received power (legacy approach), is not well suited for AI/ML mobility scenarios. In fading CDL channels, rapid signal fluctuations make precise time alignment between measured and predicted samples difficult, leading to potential inaccuracies in ground truth. Furthermore, the TE lacks knowledge of the UE’s proprietary L1 filtering parameters, such as filtering coefficients and sample counts, which further limits its ability to replicate the UE’s measurement context.

Alternative 2, the ground truth for FR1 is derived based on the measurements reported by UE based on certain conditions:

Alternative 2 derives the FR1 ground truth from the measurements reported by the UE under specific conditions. This approach avoids the complexity of accounting for channel variations or UE implementation-specific parameters (such as L1 filtering details) since these factors are inherently handled by the UE when generating measurement reports. As a result, the TE receives ground truth values already aligned with the UE’s internal processing, simplifying the derivation process.

A potential challenge with this method arises when a timing window is configured, as some UEs may not be capable of measuring and predicting simultaneously. For temporal domain Case A, this is not an issue since measurements are always performed and can be reported as ground truth. For temporal domain Case B, where some measurements are skipped, continuous measurement values are not always available. This can be addressed by enabling the UE to perform legacy measurements and AI/ML predictions in parallel (when capable) or by running two separate test iterations, one for legacy measurement reporting (to establish ground truth) and one for AI/ML prediction, under identical channel conditions.

S(I)NR variations during the test may also pose a challenge for this alternative, as prediction and measurement could occur under slightly different conditions. This can be mitigated by maintaining an average SINR during the test and ensuring that ground truth measurements are taken at higher SINR levels for accuracy.

Additionally, safeguards must be in place to prevent UE cheating, which can be achieved through dedicated test procedures, as outlined in supporting documentation.

Alternative 2, where the FR1 ground truth is derived from UE-reported measurements under defined conditions, is better suited for AIML Mobility because channel variations and L1 filtering effects are inherently accounted for by the UE when producing the ground truth measurements

Below we describe the anti-cheating strategies the testing procedure could employ to mitigate the cheating risks for Alt2:

### 1) “Two parallel channels in one run” (twin-cell single-run)

Concept:  The TE generates two cells with matched configs and the same instantaneous channel realization with some power offset only known to TE. In the same run, the UE (a) predicts RSRP for Cell-A over PW, and (b) measures RSRP for Cell-B over the same PW; Cell-B’s stream is the GT. Because prediction and GT are simultaneous on different cells, the UE can’t simply replay predictions as “measurements,” and there’s no run-to-run fading mismatch. This mirrors the “two neighbor cells” safeguard discussed for Alt-2, but does it concurrently instead of in two separate runs.

Strengths: Removes run-to-run channel drift; closes the “replay my predictions as GT” loophole.
Caveats. The rig must synchronize channels tightly; true identity of instantaneous fading across two RF paths can be challenging OTA.

In the twin-cell single-run method, the UE must output its prediction for time t at time t – Lookahead, before the actual measurement at t exists. This timing constraint means the UE cannot simply copy a future measured value into the prediction field , the measurement is not yet available when the prediction is due. The test bench timestamps predictions and measurements, so any attempt to “back-fill” predictions with later measurements would be immediately detectable.

### Conversely, copying the prediction into the measurement field would fail because live L1 measurements for the reference cell must include the real-time effects of channel fading, receiver hardware imperfections, and L1 filtering, all captured at the moment of measurement. Replaying a prediction as a measurement strips out these dynamics and would be detected by the test bench’s liveness and plausibility checks. Introducing a known, fixed (or even time-varying) power offset between Cell A and Cell B provides an additional safeguard: if the offset is only known to the TE and applied consistently during the prediction/measurement window, the UE cannot simply reuse its Cell A predictions as Cell B measurements without failing the KPI. To pass, it must perform a genuine L1 measurement on Cell B’s signal  including the offset, which preserves test integrity while keeping both cells’ fading profiles otherwise matched for fair comparison.

### 2) Profile-comparison checks (RSRP vs distance / attenuation / Tx power)

Concept : The TE sweeps a deterministic knob (e.g., distance/attenuation/Tx power) and draws a reference RSRP profile for the emulated path. The UE’s reported “GT” profile is overlaid. Significant deviation signals cheating or misbehavior. This can be done conducted (vary attenuation) or OTA (vary probe Tx power).

Strengths:  Simple to reason about; works with a single UE; good sanity test even when channel identity can’t be guaranteed.
Caveats. A clever UE might mimic a plausible curve unless the sweep is rich enough (multi-segment, randomized order). Also, residual fading/noise can mask small cheats; so we may still need tolerances.

### 3) Multi-UE consistency (collocated UEs)

Concept:  Put two or more collocated UEs under the same conditions. Compare their reported values; if one consistently violates RSRP1 ≈ RSRP2 ± δ (noise/prediction tolerance), it’s suspicious. This would scale to >2 UEs.

Strengths:  Harder to fake consistently across different implementations; it is great for spotting outliers.
Caveats. Requires multiple DUTs and true collocation; different RF chains/calibrations create legitimate offsets, so δ must be well-defined per band/setup.

Based on the discussion above we have the following proposal:

RAN4 to adopt a layered anti-cheating framework for Alt-2 GT in FR1, with the primary method as a single-run twin-cell setup where the TE drives two synchronized cells , the UE predicts Cell-A and measures Cell-B simultaneously, and the setup includes documented channel-correlation methods plus a tolerance model for residual mismatch. At least one secondary check is required: (i) profile-comparison sweep with predefined design and pass/fail tolerance, or (ii) multi-UE collocated consistency with a specified δ per band/BW/test mode.

Measurement Error Model for AI/ML Mobility

### In this section we propose an RF-Error Model for mobility measurements based on TS 38 300 Fig 9.2.4-1.

Figure 1: 3GPP TS 38.300 Measurement Model

### Notation

- c∈C be physical-cell identifier (PCI)

- n∈{1,…,Kc} Tx-beam index of cell c

- t∈N: Layer-1 sampling instant (period Tsamp​)

- f : carrier-frequency layer of beam (c,n)

- Ptruec,n,t be large-scale received power (path-loss + shadowing)

- γ(c,n,t) : instantaneous post-RX SINR of that beam

### Point-A measurement model

The received power can be modelled as :

PAc,n,t=Ptruec,n,t+βUEf+βcellc,f+δbeamc,n,f+εAγc,n,t+ιAINc,t

Below we explain the individual terms of the equation above:

UE-wide bias βUEf

This term captures every calibration error that is common to the whole handset on a given carrier band f: residual front-end gain or phase mis-calibration, antenna-coupler tolerances and slow temperature drift in the RFIC. It is slow varying hours rather than seconds so we treat it as a single constant per UE and per band. βUEf∼N0,σUE2f

Cell-specific bias βcellc,f.

The cell-bias term βcell​ represents residual hardware gain errors in the gNB’s transmit chain things like imperfect PA calibration, cable losses or over-the-air compensation loops that have nothing to do with the true propagation channel. In other words, when a UE reports an RSRP it’s measuring “channel + hardware,” not just “channel.”

All gNB impairments that occur before beam-forming land here: residual PA-gain error after digital pre-distortion, cable or feed-network losses and any left-over from factory over-the-air calibration. Because the same power detector loop controls every beam, the bias is identical for all beams of cell c on band f and changes only on the scale of minutes βcellc,f∼N0,σcell2f  .  However, we can consider this error as part of the ground truth and omit this term.

Per-beam residual  δbeamc,n,f (RF bias errors BM case 1 with truncated gaussian)

Inside the beam-forming network, small amplitude- or phase-mismatches of individual antenna branches, finite-resolution weight look-up tables and thermal gradients create a unique offset for each transmit beam (c,n). Those offsets are fixed for the duration of a simulation run and are statistically independent from beam to beam (unless we deliberately impose correlation). A variance of σbeam2f rms covers typical hardware; setting it to zero collapses the model to common bias: δbeamc,n,f∼N0,σbeam2f

Fast Layer-1 noise εAγ.

Every individual snapshot at Point A also contains fast, zero-mean noise: thermal noise plus any residual in-band interference, channel-estimation imperfection and quantizer in the UE PHY. A fresh sample is drawn for every (c,n,t) . The samples are independent across beams and time. The variance depends on instantaneous post-receiver SINR, shrinking rapidly at high SINR: εAγ∼N0,σA2γ. For example we can use the following model: σA2γ=σ021+e-aγ. This noise can be directly inferred  through LLS simulations instead of modeling  it with a particular model,

Interference indicator INc,t ( likely we don’t need spreading factor )

### While SS-block correlation does provide despreading gain, in practice significant interference remains and must be modelled explicitly. First, an SS block occupies only a handful of OFDM symbols and subcarriers in each 20 ms period, so when the UE correlates and integrates over those SS resources it also picks up any energy from overlapping PDSCH, CSI-RS or data bursts that share the same time-frequency slots those channels aren’t despread by the SS sequence and therefore bleed through as spurious power. Second, the different PSS/SSS sequences used by neighbor cells are only approximately orthogonal once we include real-world multipath and Doppler, so their cross-correlation side-lobes introduce a non-zero error floor. Finally, under moderate to heavy load the aggregate data-channel interference across the SS-block bandwidth can exceed the thermal noise floor. Taken together, these effects leakage from in-band traffic, imperfect SS-sequence orthogonality, and load-dependent interference justify adding an explicit interference term to our RSRP model so that simulated measurement variance faithfully reflects real-world loading and sequence-imperfect suppression.

### A scalar that represents the total received interference power on the UE’s serving PRBs at time.

### Simple proxy: use the measured wideband RSRP of the strongest neighbour that is not muted. Use the wide-band RSRP of the single non-serving cell that is strongest at the UE’s location:

IN1c,t=maxc'≠cPRSRPc',t

Rich proxy: sum over all neighbour cells weighted by their current PRB-load.

IN3c,t=m=13PRSRPcm,t

Treat the interference offset as zero-mean Gaussian with variance proportional to IN ​: ιAIN∼N0, σI2IN where σI2IN=kI IN

where ​ kI  (in linear power units) or  ​kIdB (in dB²) is a tunable knob that converts neighbour-cell power into measurement variance

All noise components can be aggregated and derived from LLS simulation results.

Independence.
The interference term is re-drawn each time step (like εA​) because PRB collisions vary sub-frame to sub-frame, yet it is correlated across beams that share the same PRB set. The simplest assumption is to add it once per cell and copy it to all beams of that cell for time t

Beam-to-cell consolidation (A1 → B)

When the UE collapses the Kc​ individual beam streams (point A1) into one cell-level stream (point B), it first averages the per-beam residuals. Denote this arithmetic mean by δc,f=1Kcn=1Kcδbeamc,n,f and describe the statistical similarity between any two beam-residuals with the intra-cell correlation coefficient ρf=Corrδbeam,i,δbeam,j i≠j.  Because the common cell bias βcell(c,f) is independent of every δbeam​, the variance of the consolidated bias that feeds stream (point) B becomes Varβcell+δ=σcell2f+ρf σbeam2f where ρ(f) is the correlation between any two beam residuals. If ρ(f)=0 the beam offsets are reduced, leaving  σcell2f and reduced σbeam2f,  if ρ(f)=1 they add coherently, so the full σcell2f+ρf σbeam2fterm survives.

Each Point-A snapshot is the sum of a fixed hardware offset and a one-shot noise draw:PAt=Ptruet+βUE+βcell+δbeam+εAt. The bracketed term contains the slow biases they do not change with time in the observation window. By contrast, the fast noise  is redrawn independently at every step.

If the UE applies an 8-sample sliding average, the sequence that feeds Layer-1 beam filtering becomes

PAt=Ptrue,avgt+βUE+βcell+δbeam+18k=07εAt-k+18k=07ιAINt-k

Because the eight noise terms are independent and zero-mean, their average tends toward zero (variance shrinks by a factor of eight). The slow-bias bracket, however, is identical in every summand, so it passes through unchanged. The sliding window therefore “sees” almost no fast noise but continues to see exactly the same constant offset on every sample. If interference statistics fluctuate quickly (independent per sub-frame) its variance also shrinks by 1/8; if PRB collisions are bursty we may add a correlation factor the same way we used ρ(f) for beam residuals.

### Statistical structure

- βUEand βcell​ are static in time, independent across UEs, cells and layers.

- δbeam is static, independent across beams unless ρ>0

- ​εAt is i.i.d. over beams and time; any temporal correlation is introduced only by later L1/L3 filtering.

Key tunable parameters (“knobs”) for the AI/ML error model

UE-bias spread σUE2f 
Controls the RMS calibration error that is common to the whole handset on carrier layer f. βUEf∼N0,σUE2f

Cell-bias spread σcell2f (this term will not be needed if it is part of GT)
Sets the RMS residual gain error that is common to every beam of gNB c on layer f.  βcellc,f∼N0,σcell2f

Per-beam residual spread δbeamc,n,f
Magnitude of the beam-unique mismatch after beam-forming δbeamc,n,f∼N0,σbeam2f

Intra-cell beam correlation ρ(f)
Correlation coefficient between any two beam residuals of the same cell on layer f.
ρ=0 ⇒ independent Tx beams; ρ=1 ⇒ identical residual on all beams.
Its only role is in the consolidated-bias variance Varβcell+δ=σcell2f+ρf σbeam2f

Fast-noise law σA2γ or parameters (σo2γ}, a)
Maps instantaneous SINR to the one-shot Layer-1 noise variance.
Either use a link-level lookup table or the analytic form σA2γ=σ021+e-aγ

L1 averaging length Tavg  (samples M)
Number of point-A snapshots averaged to form A1.

Interference term: σI2IN=kI IN

### We present a procedure for generating N independent UE trajectories

- Draw the slow-varying biases

For each trajectory i=1,…,N and for every carrier layer f:  βUE,if∼N0,σUE2f

For every cell c (reuse the same value for all trajectories unless each run has its own network) βcellc,f∼N0,σcell2f

Per-beam residuals, drawn once and optionally given a common component so they share the desired correlation ρ(f).  δbeamc,n,f∼N0,σbeam2f

- Loop over every measurement sample (i,c,n,t)

Compute or look up the instantaneous SINR along the trajectory γic,n,t

Draw the fast Layer-1 noise for that beam and time εA,ic,n,t∼N0,σA2γic,n,t.

Form the complete Point-A measurement PA,ic,n,t=Ptruec,n,t+βUE,if+βcellc,f+δbeamc,n,f+εA,ic,n,t+ιA,iINc,t

All noise components above can be aggregated and derived from LLS simulation results.

- Apply Layer-1 averaging

If the RRC configuration averages M consecutive snapshots, divide the fast-noise variance by M (or equivalently average M successive PA,ic,n,t ) to obtain the A1 stream.

After that, propagate the data through the remaining L1/L3 filters defined in TS 38 300 Fig. 9.2.4-1.

RAN4 to adopt a measurement error with a compact four-term model. The first term is a slow UE-wide bias that is constant for one handset on a given carrier band. A second term adds a small residual unique to each transmit beam, whose correlation across beams is controlled by a single knob. The third term is fast thermal-and-estimator noise, redrawn for every measurement occasion with variance taken from the existing SINR-dependent link-level table or through LLS simulations. A fourth term introduces additional zero-mean variance proportional to a chosen neighbor-power metric, modelling interference.

Inter-frequency Inter-cell Predictions

A key AI/ML application in NR inter-cell mobility is predicting inter-frequency measurements, which currently require measurement gaps where the UE pauses serving-cell communication to measure other frequencies. These gaps cause service interruptions and increase power consumption. AI/ML can reduce the need for such gaps, improving continuity and UE energy efficiency. By using measurement data from two frequency layers, models can learn cross-layer signal relationships, enabling prediction of one layer’s measurements from the other or determining if a UE on a macro layer can reliably connect to a micro layer. Clustering techniques can further enhance accuracy by combining multi-cell data as input features.

.

Frequency Correlation Coefficient

Channel correlation coefficient between measured and predicted frequency layers that will impact the prediction accuracy for inter-frequency prediction

Issue 2-1-8: Inter-frequency prediction scenario Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

The channel correlation coefficient between measured and predicted frequency layers plays a critical role in determining the accuracy of AI/ML-based inter-frequency RSRP prediction. This coefficient reflects how similar the propagation conditions are between the two frequency layers, which directly impacts model generalization and reliability.

However, the channel correlation is affected by several key factors, including:

- Differences in antenna patterns between frequencies

- Variations in bandwidth

- Changes in line-of-sight (LOS) probability

- Frequency-dependent scaling of large-scale parameters

- Divergences in cluster delays and angles

- Independent shadowing effects

- Presence of frequency-specific blockage

These variations can significantly reduce the correlation between layers, thereby limiting the predictive power of models trained on one frequency to accurately infer RSRP at another. These parameters have been tabulated in R4-2505766 and they are shown below:

| Step | Issue | Impact on Multi-Frequency Modeling |
| --- | --- | --- |
| Step 1 | Shared parameters except antenna/array/center freq/bandwidth | • Antenna pattern differences cause spatial response mismatch • Bandwidth variation affects PSD |
| Step 2 | Uniform propagation conditions (soft LOS may differ by frequency) | • FR2 typically has lower LOS probability than FR1 |
| Step 4 | LSPs same but DS/AS allow frequency scaling | • DS(f)= 10^(μlgDS(f) + σlgDS(f)x) |
| Steps 5-7 | Identical cluster delays/angles for all frequencies | • Conflicts with Clause 7.5 (noted in standard) |
| Step 6 | Independent per-cluster shadowing, frequency-dependent cluster powers | • Allows realistic power variation but no explicit pathloss linkage |
| Steps 8-11 | Small-scale parameters generated independently per frequency | • Breaks physically existing phase correlations |
| Blockage | Shared blocker positions across frequencies | • Underestimates FR2 blockage probability (wavelength-sensitive) |

RAN4 to conduct simulation studies for calibration and to better align the frequency-domain channel correlation coefficients. This should be based on the modeling assumptions for large-scale and small-scale parameters across multiple frequencies, and antenna pattern differences. The goal is to create a consistent simulation baseline that can support more accurate evaluation of inter-frequency prediction models

Impact of Cluster Approach

AIML model performance can be enhanced when measurements from a group of cells in one carrier frequency are used as an input for AIML model. The rationale behind this is diverse inputs can help to enhance AIML model performance. Clustering is needed in AI/ML-based inter-frequency measurement prediction because relying on measurements from a single serving cell often provides a limited and potentially biased view of the radio environment. In many deployment scenarios, especially in heterogeneous networks with varying cell sizes and antenna configurations, the signal behavior of a single cell may not fully represent the spatial and frequency-domain correlations present across the network. By incorporating measurements from multiple cells within the same frequency layer, whether serving or neighbor cells, the model gains access to richer and more diverse spatial fingerprints.

Another key benefit of clustering is resilience against measurement anomalies. In practice, a single cell’s measurement may be affected by temporary disturbances, such as beam misalignment, or short-term channel fluctuations that could mislead the prediction model. When the model instead consumes a set of measurements from multiple cells, it can average out or filter these anomalies, allowing it to identify the underlying long-term correlation between frequency layers more accurately. This is particularly important in mobility scenarios where channel conditions change rapidly, and the predictive power of any single cell’s measurement may degrade quickly.

Finally, clustering supports better generalization across deployment environments. AI/ML models trained with diverse multi-cell input are less sensitive to changes in topology, cell power configurations, or beamforming patterns because they learn relationships that are consistent across the network, not just within a single link. For inter-frequency prediction, this means the model can more reliably infer a target frequency’s RSRP, even in areas with sparse direct measurements, by leveraging patterns observed in other, spatially correlated cells.

RAN4 is invited to study the impact of clustering on AI/ML-based inter-frequency measurement prediction performance. Multi-cell inputs provide richer spatial and frequency-domain information, improve robustness against anomalies in individual cell measurements, and enable better generalization across deployment scenarios. The study should assess clustering benefits under varying mobility conditions, network topologies, and correlation levels between cells, with the objective of determining whether clustering can reduce measurement overhead while maintaining or improving prediction accuracy

Generalization Issues

In this section we present an end-to-end simulation blueprint for a RAN4 study aimed at resolving generalization problems in AI/ML-based mobility prediction.

Problem statement and study objective
AI/ML beam-selection and hand-over algorithms will be trained and benchmarked in a single “friendly” lab environment. However, the live network changes band, panel size or mobility pattern, and the prediction accuracy can collapse. RAN4 therefore needs proof that AI/ML prediction can generalize across the full radio diversity operators deploy. The proposed study will compare three different approaches and their underlying models

  - Universal model: one large neural network trained on every dataset it sees, with no specialization.

  - Per-scenario model: a completely separate full-size network for every deployment slice, guaranteeing accuracy but at huge memory and certification cost.

  - Backbone + Experts: the proposal under test, where a single physics-aware backbone is frozen and each slice adds only a small adaptation layer. The goal is to show that Backbone + Experts meets RAN4 KPI thresholds and beats the other two on size, update agility and conformance overhead.

To judge generalization we must move the radio “knobs” that truly shape propagation. This slice universe lets us create controlled yet diverse scenarios. By training on one set of slices and testing on others, we can measure how well an AI/ML model transfers its learning when any single axis or several at once changes.

Defining the slice universe
Generalizations can only be judged if we deliberately change the radio “knobs” that matter. We describe every deployment by a five-axis code:

- Frequency layer (F) – low sub-6 GHz, high sub-6 GHz, low mmWave, high mmWave; four buckets capture two octaves of path-loss change.

- Antenna / beam class (A) – small arrays (≤ 8 TX), medium arrays (9–32 TX) and massive arrays (> 32 TX); this axis drives beam-width and spatial consistency. [ a1, a8]

- UE-speed tier (V) – pedestrian, city-traffic, high-speed rail; speed alters temporal correlation and Doppler. [vlow, vmedim, vhigh]

- Environment (E) – dense-urban street canyon, suburban macro, rural motorway and indoor hotspot; each has distinct scatterer density. [e1, e2, e3,,, ]

- SNR (S): it could be defined based on ranges: Low, medium, high [snr low snr medium snr hign ]

Let’s consider one example where the whole set of slices as defined by the cartesian product is 4(F) × 3(A) × 3(V) × 4 (E) x 3 = 432 theoretical slices. RAN4 will never test all 432, but the taxonomy ensures every real-world cell can be mapped to a canonical slice code.

How to Choose the “training coverage” level (so that we test the generalization later):

### There are two practical strategies for down-selecting slices before we start training:

1 – “Exploratory” (very low budget, ≈ 8 slices)
When you just need a quick sanity check, anchor each axis at its two extremes and cross them:

- Frequency: choose the lowest band we operate and the highest (e.g., 47 GHz).

- Antenna size: take the smallest panel in the network (≤ 8 TX) and the largest massive-MIMO panel (> 32 TX).

- Speed: include a pedestrian trace (< 10 km h⁻¹) and a high-speed-rail trace (> 120 km h⁻¹).

- Environment: pick one outdoor macro setting (e.g., suburban, UMa) and one dense indoor/hotspot (InH).

- SNR:  pick low and high

The cartesian product 2 × 2 × 2 × 2 = 16 slices captures the widest physics gap on every axis with the smallest possible dataset. We can use this dataset when we want a fast test to see if the modelling concept holds.

2 “Balanced” (mid-size, ≈ 40 slices)
Sample slices are data-driven rather than by eyeballing the extreme values:

- Encode every one of the all possible slices as a numeric vector
[ freq_GHz,  log2(#TX),  speed_km h−1,  env_id , SNR]

- Cluster them with K-means (choose K).

- Attach a weight to each slice if available

- Rank clusters by the summed weight of their members and keep the centroids of the top KK clusters.

This way we end up with a modest-sized training set that is (a) evenly spread across the physics space thanks to K-means distance minimization and (b) skewed toward the slices that will actually be more beneficial according to some predefined criteria.

Below is the suggested simulation workflow for addressing the generalization challenge and evaluating different models.

### Option 1: Universal model: single network for every slice (scenario)

In the simulation we first evaluate a naïve baseline by training a single universal model on an aggregated dataset that combines logs from all the chosen training scenarios. To prevent dominant slices from overshadowing rare ones, we apply balanced mini-batching with equal sampling from each slice in every training step. The model is optimized over many epochs using AdamW and an appropriate learning rate around . This simulation produces one checkpoint. Checkpoint refers to a saved snapshot of the trained AI/ML model on the aggregated dataset at a given point during or after training. It contains the learned parameters (weights and biases) of the model, as well as optimizer states if needed.

### Option 2: Many stand-alone models:  one full network per slice

### In this simulation setup, we also evaluate the per-slice training approach, where a separate full model is trained individually on each slice/scenario of the dataset. For each slice scenario, we create an independent training dataset using only the dataset from that specific slice. The model is trained on this single-slice data with standard mini-batching, optimized over many epochs using AdamW with an appropriate learning rate. At the end of training, we save a checkpoint of the model for each slice. These checkpoints are later used for offline KPI evaluation to assess how well models perform when trained exclusively on one slice and tested across others.

### Option 3: Shared backbone + tiny slice-specific experts

In this simulation setup, we also evaluate a parameter-efficient approach using a shared backbone combined with small slice-specific experts. A single backbone model is trained to capture general radio characteristics across all slices. For each individual slice, we attach a lightweight  adapter and fine-tune only this adapter while keeping the backbone frozen. Training follows standard mini-batching over many epochs with AdamW and a appropriate learning rate. After training, we save the common backbone along with the set of slice-specific adapters, which are later used for offline KPI evaluation. This method aims to balance generalization with slice-specific adaptation while keeping the total model size compact.

- Stage A – learn backbone Φ

  - Train on the pooled data exactly like the universal case.

  - Freeze Φ once validation loss stabilizes

- Stage B – fine-tune for all adapters

  - For each slice k: attach a fresh adapter Ψₖ, train just 2-3 epochs on that slice, freeze, export

  - Repeat until all adapters are done.

- Simulation outputs: one backbone file + (number of chosen slices) adapters.

Summary of the model options described:

- Universal model tests raw generalizability with minimal engineering effort.

- Stand-alone models give an upper-bound on accuracy but are heavy.

- Backbone + experts aims for near-upper-bound accuracy at a fraction of the storage and training cost, exactly what we want to quantify before any formal RAN-4 test plans are drafted.

We make the following definitions:

### Seen Slices = Slices used for training, but tested on held-out (unseen) data

- During training, we use part of the data from each of the Seen Slices (e.g., 80% for training).

- For testing, we evaluate the model on the remaining 20% (or any split) of data that was never used in training from those same slices.

- This gives us a valid estimate of in-distribution generalization testing whether the model learned meaningful patterns rather than memorizing the training data.

### Unseen Slices = Slices never used during training, fully reserved for testing

- Unseen Slices are slice-types (combinations of deployment parameters) that are entirely excluded from the training phase.

- Neither the model nor any of its components (universal, stand-alone, or expert) have been exposed to any data samples, patterns, or statistics from these slices during training.

- The model is evaluated on fresh data drawn from these Unseen Slices, representing truly out-of-distribution (OOD) testing.

The NMSE performance on Unseen Slices directly measures the model’s ability to generalize to deployment scenarios it was never trained on. This simulates real-world robustness when a trained model encounters new environments, frequencies, or mobility patterns.

Let:

- J= Index set of Seen Slices (used for training, tested on held-out data). Measures in-distribution performance on known slice types.

- O = Index set of Unseen Slices (never used for training, fully held out). Measures out-of-distribution performance on unknown slice types.

The total number of NMSE test error values is:

Nt=Nseen+Nunseen=J+O

Let u∈RNt be the NMSE row-vector for the universal model; element uj corresponds to test file or slice j.

Let S∈RNseen×Nt  denote the NMSE matrix for the fully stand-alone fleet, so that Sij is the error obtained when stand-alone model i is applied to test slice j

Let E∈RNseen×Nt  be the analogous NMSE matrix for the backbone + expert system.

We can use the following aggregated statistics to judge generalization:

Universal model averages

NMSEseenU=1Ij∈Iuj,   NMSEunseenU=1Oi∈Oui,  and define the generalization gap as:

ΔU=NMSEunseenU-NMSEseenU. (scalar)

Stand-alone diagonal and cross averages

We train Nseen​ separate models one model per Seen Slice

- We evaluate each trained model on all slices in the testing set:

  - The Nseen slices (in-distribution)

  - The Nunseen​ slices (out-of-distribution)

Resulting in a Nseen×(Nseen+Nunseen)  NMSE matrix S∈RNseen×Nt

The diagonal entries Sii indicate the accuracy on Seen Slices (Self-Performance): each model tested on its own slice (e.g., Model 5 on Slice 5). We want low NMSE on the diagonal, showing the model fits well to its trained slice. This validates in-distribution fitting.

Off-diagonal entries ​Sij, i≠j, Cross-Slice Generalization: Model i tested on a different slice j.

- We can analyze the following:

  - If off-diagonal NMSE is high:
→ The stand-alone model overfits its own slice and generalizes poorly.

  - If off-diagonal NMSE is low:
→ Some generalization exists across slices.

The goal of this study is to answer the following question: How many stand-alone models would be needed to “cover” the full set of possible slices with acceptable performance and by coverage we mean low NMSE.

### If we see: Diagonal ≪ Off-Diagonal NMSE (Poor generalization)

- If Diagonal NMSE ≪ Off-Diagonal NMSE, this shows:

  - Each stand-alone model performs well on its own slice

  - But generalizes poorly to other slices (seen or unseen)

- This means:

  - Models are highly slice-specific

  - We may need a large number of specialized models: possibly close to one per slice

  - The stand-alone modeling approach does not generalize efficiently

### If we see: Diagonal ≈ Off-Diagonal NMSE (Good generalization)

- If we observe Diagonal NMSE ≈ Off-Diagonal NMSE, that suggests:

  - Each model can generalize well to slices beyond its training slice

  - We may need fewer models (e.g., grouping slices into clusters)

  - The system could work with shared models across similar slices

### Let’s provide an example with numerical values (assume 144 total slices):

- If we have 16 stand-alone models which fail on most of the 4 unseen slices (high off-diagonal NMSE),
→ We cannot expect the 16 models to cover the other 144 – 16 = 128 slices
→ We would likely need many more models or a better generalization strategy (e.g., Backbone + Experts)

- If some stand-alone models perform well on multiple unseen slices,
→ We might cluster slices based on those generalization patterns
→ Estimate coverage as 1 model per generalizable cluster rather than per slice

### We can use the following rule of thumb Interpretation:

- Diagonal NMSE ≪ Off-Diagonal NMSE
→ We need ~1 model per slice (poor generalization)

- Diagonal NMSE ≈ Off-Diagonal NMSE (for both seen and unseen slices)
→ We can cover multiple slices with fewer models (good generalization)

### What Stand-Alone Models tells us: they tell us how specialized each model must be to achieve good performance on its own slice. However, by testing off-diagonal NMSE, we could see how poorly a slice-specific model performs on other slices. Therefore:

- If off-diagonal NMSE is high, stand-alone models suggest we need one model per slice

- If off-diagonal NMSE is low in clusters, we might cluster slices and reduce the number of models.

However:

- The stand-alone study doesn’t tell us whether you can share a backbone between slices.

- Nor does it inform whether lightweight specialization (experts) is enough for good performance.

If the study with standalone models show poor generalization performance, then we can adopt a better generalization strategy explained below:

### Backbone + Experts Simulation Study:

- The Backbone + Experts approach tests whether a single shared backbone can generalize common features across slices, while a small slice-specific expert handles the specialization.

- We employ the following training procedures:

  - One backbone model on all Seen Slices mixed dataset.

  - One expert per Seen Slice (small parameter set and few iterations).

- Employ the following test procedures:

  - Test how well the backbone + expert combination works on:

    - The Seen slices (with their own expert)

    - The Unseen slices (with a random expert or no expert possibly fallback)

- If Backbone + Experts generalize well:
→ We may only need a single backbone and a small set of experts for similar slice groups.
→ This reduces total models dramatically (e.g., 1 backbone + ~10 experts instead of ~100 full models).

- If Backbone + Experts fail on unseen slices:
→ The radio domain doesn’t support generalization well.
→ We may still need many specialized models (but at least now we know).

### Therefore the bottom line and conclusions from above discussion are:

- Stand-alone models = Worst case upper bound on model count (likely 1 per slice if no generalization).

- Backbone + Experts = Test if we can collapse models to a small number of shared structures.

Based on the above analysis we have the following proposal:

RAN4 is invited to study the model robustness across scenarios of AI/ML models for AI/ML for mobility by evaluating three training approaches: (1) a universal model trained on a combined dataset of multiple deployment scenarios, (2) stand-alone models trained per scenario, and (3) a shared backbone model with lightweight scenario-specific experts. The study will assess performance on both training scenarios (Seen) and scenarios not used in training (Unseen) through systematic testing across diverse deployments. The outcome will clarify whether AI/ML for mobility requires many specialized models or if a shared architecture with minimal specialization can handle realistic deployment variations, guiding model deployment strategies and standardization in RAN4.

RAN4 is invited to study the generalization capability of AI/ML-based inter-frequency measurement prediction models during the WI stage (Re-20). The study should evaluate how models trained on measurement data from specific inter-frequency deployment scenarios (e.g., macro–micro, FR1–FR1, FR1–FR2) perform when applied to unseen scenarios with different frequency layer combinations, cell layouts, and propagation environments. This includes assessing the impact of clustering multi-cell measurements as input features, the portability of models across varying network configurations, and the trade-off between prediction accuracy and measurement gap reduction.

Spatial Domain Prediction (Case 6)

### We provide our view on the concept of Spatial Domain Prediction with Network-side Model Perspective: At the network side, spatial domain prediction refers to using AI/ML models that learn mappings between radio measurements, such as serving cell beam RSRPs, SINR patterns and the expected RSRP or other KPIs of neighbor cells or beams, even if those neighbors are not directly measured by the UE at that moment.

The AI/ML model resides at the gNB (or NW function), where it leverages measurement reports collected over time to learn spatial correlation patterns. Once trained, the network-side model can infer neighbor cell or beam performance based on incoming real-time measurements/fingerprints from the serving cell.

Below we provide some Network-side Prediction Examples with spatial prediction:

- Neighbor Cell RSRP Prediction:
“Given the serving cell’s reported beam-level RSRPs  from the UE, the network-side AI/ML model predicts the expected RSRP of Neighbor Cell B based on learned fingerprint/measurement associations from past mobility data”

- Proactive Handover Candidate Identification:
“When the serving cell’s fingerprint matches patterns historically linked to strong neighbor performance, the network predicts that Neighbor Cell C will soon become the best HO target and can proactively prepare resources.”

- Measurement Gap Avoidance:
“The network uses the serving cell’s multi-beam fingerprint to predict neighbor inter-frequency RSRP, minimizing the need for UE-triggered measurement gaps.”

- Neighbor Beam Prediction:
“By analyzing serving-cell fingerprints, the network predicts which neighbor beam would offer optimal link quality if the UE moves toward a certain location.”

### The benefits associated with spatial prediction from the perspective of AI/ML model residing at NW are listed below:

- It reduces dependency on frequent measurement reports and gaps.

- Enables proactive mobility decisions based on learned spatial fingerprints.

- The model can aggregate data across many UEs, enhancing its prediction robustness.

- Keeps the AI/ML intelligence centralized at the network, easing model management and updates.

As a conclusion, Network-side spatial domain prediction means the network uses AI/ML models trained on spatial measurement patterns to predict neighbor cell or beam performance, enhancing mobility management without relying solely on real-time UE measurements.

Differences Between AI/ML FR2 Beam Prediction (Case 1) and AI/ML Mobility Spatial Domain Prediction (Case 6)

The AI/ML use cases for mobility in NR serve distinct purposes, especially when comparing Case 1 – FR2 Beam Prediction for BM and Case 6 – Spatial Domain Prediction for Mobility. While both use AI/ML to predict aspects of the radio environment, their objectives, inputs, and deployment scopes differ significantly.
Case 1 (FR2 Beam Prediction) focuses on optimizing beam selection within the serving cell, typically for initial access, beam tracking, or beam refinement in FR2 (mmWave) environments. The AI/ML model predicts the best beam (or beam set) based on immediate link conditions, with the objective of improving DL/UL link quality.

Case 6 (Mobility Spatial Domain Prediction) targets the prediction of neighbor cell KPIs, such as expected RSRP, to support mobility events like handover preparation, neighbor ranking, or measurement gap reduction. The aim is to proactively assist mobility management by forecasting neighbor performance, even when direct measurements are unavailable. In beam management, index prediction and/or L1-RSRP prediction may be sufficient, but for L3 spatial-domain mobility prediction, RSRP prediction is generally required to compare cells for handover decisions.

Source of Input Features
Case 1 uses serving cell beam-level measurements (e.g., SSB RSRP, CSI-RS), focusing on exploiting real-time serving-cell beam decisions such as beam ID or beam RSRP. In contrast, Case 6 relies on serving cell measurement reports and spatial “fingerprints,” which include multi-beam KPIs, past neighbor reports, and spatially correlated radio patterns. The AI/ML model uses these fingerprints to infer neighbor performance by capturing spatial correlations over time and across cells.

Model Location and Training Focus
In Case 1, AI/ML models are typically co-located with the UE or gNB beam management functions, trained to predict the best serving beam using local measurements. In Case 6, the AI/ML model is generally deployed at the network side (gNB or centralized network function), where it can learn cross-cell spatial correlations from aggregated multi-UE, multi-cell data. This enables prediction of neighbor cell or beam KPIs based on serving-side fingerprints.

Additional Technical Considerations
Case 1 typically operates in high-SINR conditions, while Case 6 may involve neighbor cells with degraded SINR (e.g., −6 dB). Beam widths and codebooks may also differ between serving and neighbor cells; for example, the UE might use a narrow beam for beam management inputs in the serving cell but fixed beam patterns (e.g., eight beams covering 360°) for neighbor cell inputs.

Conclusion: Why Case 1 Findings Cannot Be Reused for Case 6
The two cases differ fundamentally:

- Prediction tasks: Case 1 predicts the optimal serving beam, while Case 6 predicts neighbor KPIs for proactive mobility without necessarily having real-time measurements.

- Spatial scope: Case 1 operates within a single TRP/cell, whereas Case 6 requires understanding cross-cell relationships and spatial patterns.

- KPIs and decision points: Case 1 targets serving beam ID or beam set selection; Case 6 targets neighbor RSRP, beam ranking, or handover candidate selection.

- Deployment strategies: Case 1 models are close to beam control functions; Case 6 models operate at the network level, aggregating fingerprints from multiple UEs and cells.

Example:

- Case 1: Based on SSB-RSRP, predict the serving beam ID to maintain optimal FR2 link quality.

- Case 6: Given the current serving cell’s beam fingerprint, predict whether Neighbor Cell B’s RSRP will exceed the handover threshold within 100 ms.

The two spatial prediction cases serve fundamentally different roles in the NR system. Case 1 (AI/ML BM spatial prediction) is designed for intra-cell beam management using immediate link feedback, while Case 6 focuses on neighbour prediction and proactive mobility support using cross cell measurements and AI/ML model at the network side.

The conclusions, model designs, and deployment approaches validated for Case 1 (AI/ML for Beam Management) cannot be assumed valid for Case 6 (AI/ML for Mobility). Case 6 requires broader spatial awareness, cross-cell learning, and prediction mechanisms that address inter-frequency, inter-cell, and longer time-horizon mobility scenarios beyond the scope of beam management. It is therefore proposed to study these distinctions explicitly within the Re-20 6G WI, ensuring that AI/ML methods for mobility are evaluated under representative spatial, temporal, and multi-cell conditions to confirm their robustness and generalization capability across varied deployment scenarios.

Impacts from observation and prediction windows

Following agreement was reached in RAN4#114 meeting:

Issue 2-1-9: Impacts from observation and prediction windows Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

In temporal domain AI/ML prediction, the relationship between the observation window (OW) length, prediction window (PW) length, UE speed, and measurement reporting repetition time (MRRT) can significantly affect prediction accuracy requirements. The number and type of measurements collected within the OW directly influence the AI/ML model’s ability to learn and forecast. However, the ability to precisely determine the number of usable measurement samples within the OW is limited by several factors.

For Case A (no measurement reduction), the model inputs may be either L1 measurements (sub-cases 1 and 3) or L3 measurements (sub-case 2). While the number of L3 measurement samples in the OW can be derived from the configured measurement periodicity, the number of L1 samples depends on UE-specific parameters such as L1 sampling period and filter coefficients, which are proprietary and unknown to RAN4. Even in the L3 case, the subset of L3 samples actually used by the AI/ML model may remain UE-proprietary. As a result, RAN4 cannot reliably assess the number of measurements performed during the OW, nor quantify their direct effect on prediction accuracy.

This limitation means that while OW, PW, MRRT, and UE speed are important parameters to study, RAN4’s evaluation should focus on observable and standardized factors rather than attempting to account for proprietary measurement sample counts inside the OW.

In case A temporal prediction, OW/PW length, UE speed, and MRRT affect accuracy, but the exact number of measurements in the OW is UE-proprietary. RAN4 should therefore base KPIs on standardized parameters (OW, PW, MRRT, UE speed) without considering internal L1/L3 counts.

In temporal domain prediction case B, the UE may reduce the number of actual measurements by replacing them with predicted values, with the reduction level controlled by the MRRT parameter. When combined with variations in UE speed, OW, and PW settings, the total number of possible test configurations can grow significantly, making exhaustive testing impractical. To manage this complexity, RAN4 should agree on a limited set of pre-selected reference scenarios for OW, PW, MRRT, and UE speed to ensure consistency and comparability across evaluations.

RAN4 is invited to agree on a set of pre-selected reference testing scenarios for temporal domain prediction case B, covering representative combinations of OW, PW, MRRT, and UE speed. This approach will limit the exponential growth of possible test configurations.

In temporal domain prediction case B, achieving a specific Measurement Reduction Rate in the Temporal domain (MRRT) can be done through different skipping patterns, even when the target reduction percentage is the same. For example, a 50% MRRT could be realized by replacing a continuous block of measurement slots with predictions within the Prediction Window (PW), or by alternating measured and skipped slots throughout the Observation Window (OW). While both patterns meet the MRRT target, they have different impacts on the distribution of real measurements, potentially influencing prediction accuracy and robustness.

As discussed in RAN2, skipping patterns are applied at the L1 sample level and depend on the UE’s proprietary L1 filtering implementation. This raises the question of whether the UE should indicate its supported skipping patterns to the network (NW). If such capability information is shared, the NW could configure MRRT explicitly by selecting a specific skipping pattern from those supported by the UE. If not shared, the MRRT configuration would be applied without visibility into the underlying measurement distribution. If the UE does not tell the network (or the test equipment) which skipping pattern it is using, the MRRT setting alone (e.g., “reduce measurements by 50%”) doesn’t reveal when and which measurement slots are actually skipped.

For RAN4 testing, two approaches emerge. Option 1 keeps the skipping pattern fully proprietary, with the Test Equipment (TE) only setting the MRRT target for the Device Under Test (DUT) without knowing exactly which measurements are skipped. While simpler, this lacks robustness and creates opportunities for the UE to pass tests without faithfully implementing the intended measurement reduction behavior. Option 2 allows the UE to inform the NW of supported skipping patterns, enabling the TE to explicitly select the pattern for testing. This adds transparency and control, ensuring the test evaluates the intended conditions, but requires additional signaling and coordination.

RAN4 is invited to study whether MRRT configuration for temporal domain prediction should include visibility into the UE’s measurement skipping pattern. Two options are proposed: (1) Keep the pattern UE proprietary, with the TE applying only the MRRT value and verifying KPI compliance; or (2) Have the UE report supported patterns so the TE can explicitly configure one during testing.

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied, we particularly addressed the following topics:

- Performance metrics

- Ground truth definition for RSRP accuracy for FR1

- Measurement Error Model

- Generalization issues

- Relative accuracy

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

- Relative RSRP accuracy evaluates how well the predicted difference between two cells matches the ground-truth difference, providing a measure of consistency between cell comparisons. While a small relative-accuracy error suggests that the prediction and measurement are mutually reliable, it does not reveal which cell is actually stronger. This creates a risk when the two values are taken at different times (t₁ for one cell and t₂ for the other): channel changes, mobility, or fading can cause the relative ranking of cells to change between these moments. As a result, even with perfect relative accuracy, the network could incorrectly conclude that one cell is stronger and make a sub-optimal handover decision.

Alternative 1, where the FR1 ground truth is derived at the TE from transmitted or received power (legacy approach), is not well suited for AI/ML mobility scenarios. In fading CDL channels, rapid signal fluctuations make precise time alignment between measured and predicted samples difficult, leading to potential inaccuracies in ground truth. Furthermore, the TE lacks knowledge of the UE’s proprietary L1 filtering parameters, such as filtering coefficients and sample counts, which further limits its ability to replicate the UE’s measurement context.

Alternative 2, where the FR1 ground truth is derived from UE-reported measurements under defined conditions, is better suited for AIML Mobility because channel variations and L1 filtering effects are inherently accounted for by the UE when producing the ground truth measurements

The two spatial prediction cases serve fundamentally different roles in the NR system. Case 1 (AI/ML BM spatial prediction) is designed for intra-cell beam management using immediate link feedback, while Case 6 focuses on neighbour prediction and proactive mobility support using cross cell measurements and AI/ML model at the network side.

- A similar definition of relative RSRP accuracy exists in legacy, but it does not account for any time alignment. As per the FFS item on whether and how to consider the time difference between the two reported RSRPs when applying this definition, RAN4 should clarify what differs in the AI/ML case compared to legacy.

RAN4 is invited to consider that if the definition of intra-frequency relative RSRP accuracy allows for a non-zero time difference between the two RSRPs used in the calculation, either explicit timestamps should be required as part of the conformance testing framework to ensure traceable and bounded time alignment, or the reporting periodicity and measurement alignment rules should be clearly defined to implicitly control the maximum allowed time difference (Δt). This will ensure that any variation in the relative accuracy metric reflects the prediction or measurement performance, rather than uncontrolled environmental changes due to asynchronous reporting or prediction instances.

RAN4 is invited to study mechanisms for handling time differences between the two RSRP values used in intra-frequency relative RSRP accuracy calculations. The following options are proposed:

Option 1: Enforce strict time alignment, requiring both RSRPs to be measured or predicted at the same or near-identical time.

Option 2: Define a fixed maximum Δt threshold, excluding RSRP pairs exceeding this limit from KPI evaluation.

Option 3: Use scenario-dependent Δt thresholds based on UE mobility class or deployment scenario to better match realistic radio dynamics.

Option 4: Permit post-processing compensation for larger Δt by applying a bounded correction factor that accounts for expected RSRP variation over time, allowing more relaxed alignment constraints while preserving KPI validity.

RAN4 is invited to study the definition and testing of relative RSRP accuracy at the beam level for intra-frequency scenarios, focusing on four key areas: (1) determining whether existing cell-level thresholds, filtering, and reporting rules are suitable for the faster dynamics of beam-level RSRP or if tighter beam-specific definitions are needed; (2) clarifying reference beam selection, including whether it should always be the serving beam, any beam in the same cell, or a best-vs-secondary beam comparison; (3) assessing the need for stricter time alignment or Δt compensation methods given rapid variability of beam-level channels; and (4) defining how KPI evaluation should account for frequent intra-cell beam switching

RAN4 is invited to address the impact of time misalignment (Δt) on inter-frequency relative RSRP accuracy. Two risks are noted: (1) Small error masking poor decisions, KPI can appear perfect even if Δt would lead the network to select a weaker cell; (2) Large error from timing artifacts , KPI may show large deviations caused by real channel changes over Δt, despite each value being accurate at its own time. To ensure the metric reflects true prediction/measurement quality rather than Δt effects, RAN4 should study whether to (i) enforce strict time alignment, (ii) allow a bounded Δt with exclusions, or (iii) apply compensation models for larger Δt. Clear Δt handling rules are needed for conformance testing

RAN4 to adopt a layered anti-cheating framework for Alt-2 GT in FR1, with the primary method as a single-run twin-cell setup where the TE drives two synchronized cells , the UE predicts Cell-A and measures Cell-B simultaneously, and the setup includes documented channel-correlation methods plus a tolerance model for residual mismatch. At least one secondary check is required: (i) profile-comparison sweep with predefined design and pass/fail tolerance, or (ii) multi-UE collocated consistency with a specified δ per band/BW/test mode.

RAN4 to adopt a measurement error with a compact four-term model. The first term is a slow UE-wide bias that is constant for one handset on a given carrier band. A second term adds a small residual unique to each transmit beam, whose correlation across beams is controlled by a single knob. The third term is fast thermal-and-estimator noise, redrawn for every measurement occasion with variance taken from the existing SINR-dependent link-level table or through LLS simulations. A fourth term introduces additional zero-mean variance proportional to a chosen neighbor-power metric, modelling interference.

RAN4 to conduct simulation studies for calibration and to better align the frequency-domain channel correlation coefficients. This should be based on the modeling assumptions for large-scale and small-scale parameters across multiple frequencies, and antenna pattern differences. The goal is to create a consistent simulation baseline that can support more accurate evaluation of inter-frequency prediction models

RAN4 is invited to study the impact of clustering on AI/ML-based inter-frequency measurement prediction performance. Multi-cell inputs provide richer spatial and frequency-domain information, improve robustness against anomalies in individual cell measurements, and enable better generalization across deployment scenarios. The study should assess clustering benefits under varying mobility conditions, network topologies, and correlation levels between cells, with the objective of determining whether clustering can reduce measurement overhead while maintaining or improving prediction accuracy

RAN4 is invited to study the model robustness across scenarios of AI/ML models for AI/ML for mobility by evaluating three training approaches: (1) a universal model trained on a combined dataset of multiple deployment scenarios, (2) stand-alone models trained per scenario, and (3) a shared backbone model with lightweight scenario-specific experts. The study will assess performance on both training scenarios (Seen) and scenarios not used in training (Unseen) through systematic testing across diverse deployments. The outcome will clarify whether AI/ML for mobility requires many specialized models or if a shared architecture with minimal specialization can handle realistic deployment variations, guiding model deployment strategies and standardization in RAN4.

RAN4 is invited to study the generalization capability of AI/ML-based inter-frequency measurement prediction models during the WI stage (Re-20). The study should evaluate how models trained on measurement data from specific inter-frequency deployment scenarios (e.g., macro–micro, FR1–FR1, FR1–FR2) perform when applied to unseen scenarios with different frequency layer combinations, cell layouts, and propagation environments. This includes assessing the impact of clustering multi-cell measurements as input features, the portability of models across varying network configurations, and the trade-off between prediction accuracy and measurement gap reduction.

The conclusions, model designs, and deployment approaches validated for Case 1 (AI/ML for Beam Management) cannot be assumed valid for Case 6 (AI/ML for Mobility). Case 6 requires broader spatial awareness, cross-cell learning, and prediction mechanisms that address inter-frequency, inter-cell, and longer time-horizon mobility scenarios beyond the scope of beam management. It is therefore proposed to study these distinctions explicitly within the Re-20 6G WI, ensuring that AI/ML methods for mobility are evaluated under representative spatial, temporal, and multi-cell conditions to confirm their robustness and generalization capability across varied deployment scenarios.

In case A temporal prediction, OW/PW length, UE speed, and MRRT affect accuracy, but the exact number of measurements in the OW is UE-proprietary. RAN4 should therefore base KPIs on standardized parameters (OW, PW, MRRT, UE speed) without considering internal L1/L3 counts.

RAN4 is invited to agree on a set of pre-selected reference testing scenarios for temporal domain prediction case B, covering representative combinations of OW, PW, MRRT, and UE speed. This approach will limit the exponential growth of possible test configurations.

RAN4 is invited to study whether MRRT configuration for temporal domain prediction should include visibility into the UE’s measurement skipping pattern. Two options are proposed: (1) Keep the pattern UE proprietary, with the TE applying only the MRRT value and verifying KPI compliance; or (2) Have the UE report supported patterns so the TE can explicitly configure one during testing.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- R4-2504901, WF on AI/ML Mobility, Nokia, RAN4#114bis, Wuhan, China, 7th – 11 April, 2025.