3GPP TSG-RAN4 Meeting#116	R4-2509436

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	5.28.1

Source:	Apple

Title:	Updates to TR38.755 (Study on NR FR1 DL fragmented carriers)

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, the TR [2] was completed with a conclusion of the SI. Subsequently, the SI was concluded in RAN#108. At the same time, there were offline discussion that companies should review the TR and provide comments if anything is missing from the TR or the TR needs some corrections/refinements.

In this contribution, we share our thoughts on the TR refinements.

# 2	Discussion

CA fallback and architecture switching

The text in Section 8.2 “CA fallback and architecture switching” describing the three fallback scenarios can be further improved/refine as shown below:

- Scenario 1: We agree that it does not require additional UE capability in the current CA framework. However, the normal CA reconfiguration from the network to enable the fallback is still required. Also, the statement “Based on the evaluation result in section 7, the performance loss of “one Rx RF chain” is comparable with that of the other CA configurations in the RAN4 specification” is not clear. What it is meant to say is that “Based on the evaluation result in section 7, the performance degradation of operating with “one Rx RF chain” is comparable with that of the other CA configurations due to interference such as UL harmonic, intermodulation, limited cross band isolation, etc. in the RAN4 specification.”

- Scenario 2: The text is clear in conveying the main points about scenario 2. Meanwhile, some wording improvement is needed.

- Scenario 3: Regarding scenario 3a, current RAN4 requirement as copied below may indicate that such fallback behavior is not allowed. Of course, as study items do not have any specification impact, we can still list it as one valid option. However, it is better to clearly state that scenario 3a may not be allowed in current specification and thus could imply specification impact.

## 7.2 Diversity characteristics The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.

“Network initiated” vs. “UE initiated” transition

Furthermore, we have the following comments on whether the transition from “one Rx RF Chain” to Scenario 1~3 is “network initiated” or “UE initiated.”

- Network-initiated transition may or may not have standards impact, and UE-initiated transition is expected to have standards impact. Note that current CA framework is mainly based on the “UE-assisted, network-initiated” model.

- If we want to explore UE-initiated transition further, it may lead to some additional benefits. For example, with the current “UE-assisted, network-initiated” model, any SCell deactivation/release from the network may take tens of milliseconds or longer, resulting in prolonged link quality degradation and packet losses. In this case, UE-initiated SCell deactivation/release allows the UE in one Rx RF chain mode to quickly deal with link quality degradation.

- We propose to capture the above two points in the TR.

Proposal 1: It is proposed to further update the TR to make the aforementioned aspects clear in the TR.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to further update the TR to make the aforementioned aspects clear in the TR.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

TR38.755, "Study on NR FR1 DL fragmented carriers," v19.0.0