# 3GPP TSG-RAN WG4 Meeting #116	                                                	     R4-2509450

# Bangalore, India, 25 – 29 August, 2025

Agenda Item:	7.23.3.1

Source: 	Apple

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

# Introduction

RAN4 requirements for OD-SSB was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1].

In this contribution, we will discuss all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

## OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Sub-issue 2: OD-SSB in FMW collided with MG

- Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

- Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

Considering the complexity of cancelling measurement gap, as being discussed in XR, we propose the following UE measurement behavior:

- If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

With this, there is no impact on gap-based RRM measurement.

Proposal 1: If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs

The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion.

- Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2.

- Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

- Other option is not precluded.

Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The OD-SSB based deactivated SCell measurement procedure is shown as follow.

The open issue is regarding UE measurement behavior after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs. Option 1, i.e., the UE should perform ‘fast measurement’ immediately after T2, is not supported. Network knows exactly when T2 ends. Network could send OD-SSB activation command after T2. UE is not expected to receive the activation before T2 ends.

Proposal 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

## OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity.

- Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if

  - The periodicity of AO-SSB and OD-SSB are same

- It doesn’t impact the WI’s completion.

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, there are several aspects that we need to think about. For instance, does UE need to measure both AO-SSB and OD-SSB? We know that RAN1 agreed UE is not required to measure both AO-SSB and OD-SSB if always-on SSB is CD-SSB on a synchronization raster. But what if AO-SSB is not on a synchronization raster? Besides, how to interpretate serving cell MO and how handle neighbor cell measurement? Specifically, is there any impact on definition of intra/inter-frequency measurement? Can we simply reuse the legacy definition, such as:

- SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the center frequency of either AO-SSB or OD-SSB of the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

- SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

For instance, if UE only measures OD-SSB after OD-SSB activation, is UE expected to skip measurement on neighbor cell whose SSB is on same frequency of AO-SSB? Considering we are coming to the end of core part design, we propose to deprioritize this scenario from RAN4 requirements point of view.

Proposal 3: deprioritize the case when the center frequency locations of always-on SSB and on-demand SSB are different in R19 NES from RAN4 requirement point of view.

## OD-SSB based SCell activation

Issue 1-3-1: The known cell condition in Case 2

Agreement

- The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination.

  - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

- FFS whether apply to Alt Time-C2, if the requirement is defined.

- FFS whether apply to scenario 3, if the requirement is defined.

- FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

We think the known SCell activation delay can also apply to Alt Time-C2. However, we still don’t see the need to support scenario 3.

Proposal 4: The current known SCell activation delay methodology can be reused in Case 2 Alt Time-C2 as well with the following combination.

- When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

Proposal 5: not define requirements for scenario 3 in this release.

## OD-SSB general issues

Issue 1-4-4: Additional Processing time for OD-SSB parameter update

Agreement

- Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

- Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE.

  - The additional processing time is 5ms

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

  - Note 2: No capability is needed

For simplicity, we suggest applying the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates:

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

Proposal 6:  regarding additional processing time for OD-SSB parameter update, apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

Agreement

- RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS.

- Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation.

  - Option 1: No UE additional processing time for OD-SSB deactivation

  - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

We continue supporting option 2.

Proposal 7: regarding UE additional processing time for OD-SSB deactivation, Follow the same values as UE additional processing time for OD-SSB activation.

## RAN2 LS

RAN4 received the following LS from RAN2 regarding L3 measurement of OD-SSB.

For the case where AO-SSB and OD-SSB have the same center frequency, RAN2 had made the following agreement: - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured. Please inform RAN2 if any concern regarding the above agreement.

For simplicity, we propose to follow the specific SMTC for SCell measurement when OD-SSB is activated, but still follow legacy SMTC for neighbor cell measurement regardless of status of OD-SSB from the SCell. This can minimize impact on neighbour cell measurement design and mobility performance.

Proposal 8: no concern on RAN2 agreement. Regarding UE measurement behaviour in RAN4, UE shall follow the specific SMTC for SCell measurement when OD-SSB is activated, but still follow legacy SMTC for neighbor cell measurement regardless of status of OD-SSB from the SCell.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for OD-SSB. After discussion, the following conclusions are provided:

Proposal 1: If FMW covers any measurement gap occasion in time domain, UE is allowed to skip OD-SSB measurement and perform measurement on other carriers within measurement gap occasion(s).

Proposal 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

Proposal 3: deprioritize the case when the center frequency locations of always-on SSB and on-demand SSB are different in R19 NES from RAN4 requirement point of view.

Proposal 4: The current known SCell activation delay methodology can be reused in Case 2 Alt Time-C2 as well with the following combination.

- When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

Proposal 5: not define requirements for scenario 3 in this release.

Proposal 6:  regarding additional processing time for OD-SSB parameter update, apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

Proposal 7: regarding UE additional processing time for OD-SSB deactivation, Follow the same values as UE additional processing time for OD-SSB activation.

Proposal 8: no concern on RAN2 agreement. Regarding UE measurement behaviour in RAN4, UE shall follow the specific SMTC for SCell measurement when OD-SSB is activated, but still follow legacy SMTC for neighbor cell measurement regardless of status of OD-SSB from the SCell.

# Reference

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R2-2504981, LS on L3 measurement of OD-SSB, Apple