# 3GPP TSG-RAN WG4 Meeting #116	                                                	     R4-2509452

# Bangalore, India, 25 – 29 August, 2025

Agenda Item:	7.23.3.2

Source: 	Apple

Title:	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

RAN4 requirements for adaptation of common signal/channel transmission was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. In this contribution we provide further discussion on the open issues. After discussion, corresponding proposals are provided.

# Discussion

Issue 1-3-4: Colliding with MG

- Proposals

  - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

As it has been extensively discussed in R19 XR, cancelling measurement gap at UE side is quite complexity. It takes up to 5ms after DCI to cancel the gap as agreed in XR. However, RAN4 already agreed the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation is 2ms in FR1 and 3ms in FR2, which is shorter than the gap cancellation latency. Therefore, it is not sufficient for UE to cancel measurement gap. Besides, gap cancellation in XR is an optional feature. We prefer not to link the two features together. So, we support the idea to skip the measurement in the overlapping SMTC and keep measurement gap.

Observation 1: gap cancellation takes up to 5ms after DCI, which is longer than the agreed additional delay requirement in TfirstSSB. Besides, gap cancellation is an optional UE feature.

Proposal 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

- Proposals

  - Option 1: R18 Scell activation with L3 reporting

  - Option 2: multiple SCells activation scenario with SSB adaptation

We have no strong view on the above two scenarios. We are not sure if network really wants to implement this feature in such way. Especially for the second one, network needs to adapt the SSB on multiple carriers simultaneously. As we know, UE sends measurement report immediately after event on the corresponding carrier is met. Network usually activates that SCell, rather than waiting measurement report on other carriers and then sending activation command for multiple carriers together.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for adaptation of common signal/channel transmission. After discussion, the following conclusions are provided:

Observation 1: gap cancellation takes up to 5ms after DCI, which is longer than the agreed additional delay requirement in TfirstSSB. Besides, gap cancellation is an optional UE feature.

Proposal 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

# Reference

- R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei