# 3GPP TSG-RAN WG4 Meeting #116	                                                	     R4-2509456

# Bangalore, India, 25 – 29 August, 2025

Agenda Item:	7.25.3.1

Source: 	Apple

Title:	Discussion on event triggered L1 measurement reporting

Document for:	Discussion

# Introduction

L1 event triggered measurement reporting has been widely discussed in previous RAN4 meeting. The latest agreements can be found in [1], according to which there are still some open issues. In this contribution, we continue discussing the open issues.

# Discussion

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi)

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

- Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple)

- Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

We understand the concern and motivation from vivo regarding the above side conditions from UE side. However, we don’t think we should simply add those conditions since it may result in too much constraint to network implementation. For instance, if TCI switch happens during L1 measurement period, i.e. UE needs to compare the target beam with a different serving beam, UE just needs to perform another round of measurement on the new beam as minimum requirements. Therefore,  it is unnecessary to say that “No indicate TCI change during the evaluation period”. Regarding configuration change, it is similar as legacy to some extent. Configuration can only be changed via RRC. After RRC reconfiguration UE needs to follow the updated configuration to perform measurement, including both L1 and L3. To compromise, we can add them in the test case design.

Proposal 1: the following conditions are considered in the test case design for L1-RSRP reporting delay requirements, but not in core requirements.

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///)

  - Details:

    - Option 1a:(vivo)

      - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability.

      - Reporting criteria for LTM event triggered reporting is 4 per frequency layer.

      - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured.

    - Option 1b: E///

      - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

      - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS

- Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO)

- Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

Given that there are clear capability for how many cells/resources that UE can support for LTM L1 RSRP measurement, we don’t see the need to define new reporting criteria per measurement category. However, we have no strong view.

#### Issue 2-7: Impact on other LTM requirements

Agreement in RAN4#114bis:

- For FR1:

  - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

Candidate options:

- Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei)

  - For FR1:

    - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met

    - The related cell switch delay and TCI state activation requirements are applicable

  - The above optimization is not applicable to FR2

- Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia)

- Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report.

- Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia)

- Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple)

  - Number of active TCI does not exceed that UE can support.

- Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK)

- Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE)

- Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///)

  - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

First of all, we have same understanding with Huawei that it is invalid that only event entering condition is configured and leaving condition is not configured. Second, after reviewing all proposals, we list the following points that we can agree:

We are open to whether continue optimizing case 2 and FR2. However, we think it is necessary to add a condition that number of active TCI does not exceed that UE can support.

Proposal 2:  consider the following updates on applicability of cell switch and TCI state activation requirements:

- For FR1, the related cell switch delay and TCI state activation requirements are applicable when the following conditions are met:

  - UE has sent an event-triggered L1 report, and

  - the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and

  - leaving condition of the event is configured and corresponding condition is not met, and

  - Number of active TCI does not exceed that UE can support

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: the following conditions are considered in the test case design for L1-RSRP reporting delay requirements, but not in core requirements.

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

Proposal 2:  consider the following updates on applicability of cell switch and TCI state activation requirements:

- For FR1, the related cell switch delay and TCI state activation requirements are applicable when the following conditions are met:

  - UE has sent an event-triggered L1 report, and

  - the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and

  - leaving condition of the event is configured and corresponding condition is not met, and

  - Number of active TCI does not exceed that UE can support

# Reference

- R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple