# 3GPP TSG-RAN WG4 Meeting #116	                                                	     R4-2509458

Bangalore, India, 25 – 29 August, 2025

Agenda Item:	7.26.3

Source: 	Apple

Title:	Discussion of RRM impact of XR for NR Phase 3

Document for:	Discussion

# Introduction

RRM impact of XR for NR Phase 3 has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are still some open issues. In this contribution, we continue discussing the open issues.

# Discussion

## Issue 3-1: Formulas for measurement delay extension without DRX

<Way forward>

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

  - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

  - Option 3:

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

    - where Tskip = Nskip ´ Max(MGRP , SMTC period)

  - Other options are not precluded

  - where Nskip is the number of  skipped MG occasions in the measurement period.

  - FFS: CSSFinter

We prefer option 2, although the total measurement period looks longer than that of option 1 and 3. Note that option 2 aligns with NR-U design. Problem with option 1 and 3 is that SMTC configuration on different carriers may not align with each other in time domain. Option 1 and 3 have some restriction on measurement scheduling at UE side.

Proposal 1: Formulas for measurement delay extension without DRX:

Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

## Issue 3-2: Formulas for measurement delay extension with DRX

<Way forward>:

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

  - DRX cycle > 320 ms

    - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

- Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

- Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

  - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

  - Where

    - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle)

    - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle

- Other options are not precluded

Following the logic of proposal 1, we support option 2 here.

Proposal 2: Formulas for measurement delay extension with DRX

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

## Issue 3-5: Conditions for measurement skipping

<Way forward>:

- Option 1: No conditions are defined

- Option 2: Define the maximum number of gaps skipped during the measurement period.

- Option 3: Define a maximum separation between two non-skipped gaps for measurement

Note that if too many measurement occasions are cancelled on target carrier (UE cannot measure it for a long time), UE may need to adjust AGC and timing again before it can perform measurement on the carrier successfully. From UE measurement point of view, this is similar to measurement cancellation due to LBT failure. Similarly, max number of occasions shall be defined within a measurement period. If actual number of cancelled occasions exceeds the maximum value, UE shall be allowed to restart a measurement period.

Proposal 3: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 4: max number of occasions cancelled during a measurement period shall be defined. Values from NR-U can be reused.

# Conclusion

In this contribution, we provide discussion on RRM impact of XR for NR phase 3. After discussion, the following conclusions are provided:

Error! Reference source not found.

Proposal 1: Formulas for measurement delay extension without DRX:

Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

Proposal 2: Formulas for measurement delay extension with DRX

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

Proposal 3: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 4: max number of occasions cancelled during a measurement period shall be defined. Values from NR-U can be reused.

# Reference

- R4-2504911, WF on RRM requirements for XR_Ph3, Nokia