3GPP TSG-RAN WG4 Meeting #116	 					     							     R4-2509466

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.25.3.2

Source:	Apple

Title:	On remaining issue for CSI-RS based L1 measurement

Document for:	Discussion

# Introduction

During the past meetings, there have been extensively discussions on CSI-RS based L1 measurement for NR mobility enhancements WI [1-5]. WF in [5] captured all the conclusions and open issues for CSI-RS based L1 measurement in RAN4#115 meeting for further discussion.

This contribution will give further analysis on the remaining open issues and present our proposals.

# Discussion

## Intra-frequency/inter-frequency definition

In RAN4#116 meeting, intra-frequency and inter-frequency definition for CSI-RS based L1 measurement has been agreed as following. In RAN2 specification, there are some descriptions relevant to the definition. Also there might be relevant aspect to RAN1 work. It is proposed to send LS to RAN1/2 for their awareness on such agreement.

#### Issue 3-1: definition on CSI-RS based intra-frequency L1 measurement

Agreement:

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and

  - It is applied for SCS = 60KHz

- at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP.

Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement.

- RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19.

Proposal 1: It is proposed to send LS to RAN1/2 about RAN4 agreement on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

## Whether step2 is skipped for serving cell？

In RAN4#115 meeting, the following was agreed on whether to skip step 2 for serving cell.

#### Issue 3-2: whether step 2 is skipped for serving cell?

Candidate options:

- Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW)

- Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO)

- Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC).

- Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

It seems option A and option B are not mutual exclusive between each other. Option A is to emphasize that “skipping step 2” requirement is not applicable for serving cell and only B is talking about the measurement procedure for serving cell is unchanged. We propose to clarify in the CR that when a serving cell is included in the candidate cell for LTM, skipping step 2 requirement is not applicable.

Proposal 2: Clarify in the measurement requirement applicability that the requirement is not applicable when a serving cell is included in the candidate cell for LTM.

## Handling of resource collision

The following was concluded from RAN4#115 bis meeting.

#### Issue 3-3: Sharing factor for resource collision between serving and neighbour cell

Candidate options:

- For FR2 CSI-RS L1 measurement requirement,

  - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,.

    - Option 1: sharing factor shall be introduced. (HW, QC, E///)

    - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

As discussed in [6], in legacy requirements, when two CSI-RS resources overlap in time domain and require different Rx beam, measurement restriction apply but not define a measurement delay with sharing factor. The reason behind is that the configuration of CSI-RS resources is quite flexible, and NW can avoid the colliding. We are fine to align with legacy requirement handling way with Option 2 and define scheduling restriction requirement. Option 1 is also fine for us. If using option 1, we prefer that serving cell measurement and neighbor cell measurement occasion are shared by 2:1.

Proposal 3: Either option 1 with scheduling restriction or option 2 with sharing factor of 2:1 between serving cell and neighbor cell could be considered.

## How to handle CSI-RS configuration exceeding UE capability

The following agreement has been agreed in RAN4#115 meeting.

#### Issue 3-5: UE measurement behaviour if CSI-RS configuration exceeds UE measurement capability

Agreement:

- RAN4 requirements apply assuming network will not configure CSI-RS based L1 RSRP measurement for LTM candidate cell exceeding UE measurement capability.

  - FFS on whether to capture this agreement in RAN4 spec.

In our view, since the main bullet saying that RAN4 requirement apply assuming network will not configure CSI-RS based L1 RSRP measurement for LTM candidate cell exceeding UE measurement capability. It is natural that we add some requriement applicability in the specificaiton. In Nokia CR R4-2508445, the changes could be made as “ RAN4 The requirements in the clause 9.14a are applicable to FR1 and FR2-1 for LTM when the configured CSI-RS based L1 RSRP measurement for LTM candidate cell doesn‘t exceeding UE measurement capability.”

Proposal 4: clarify in the requiremetn applicability that RAN4 The requirements in the clause 9.14a are applicable to FR1 and FR2-1 for LTM when the configured CSI-RS based L1 RSRP measurement for LTM candidate cell doesn‘t exceeding UE measurement capability.

## Measurement restriction

#### Issue 3-6: Measurement restriction

Candidate options:

- Option 1: Apple, ZTE, Ericsson

  - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

- Option 2: vivo

  - When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

We support option 1. We are also ok for option 2 since it is more UE implementation friendly.

Proposal 5: either Option 1 or Option 2 is ok for measurement restriction.

## Early CSI acquisition

Below agreement was agreed in RAN4#115 meeting.

#### Issue 3-9: early CSI acquisition

Candidate options:

- Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///)

- Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC)

- Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

The baseline CSI acquisition happens after LTM CSC MAC CE. During this period, the connection of serving cell is already dropped and the CSI acquisition of target cell can be performed during the LTM cell switching. There is no RRM requirements specified for baseline CSI acquisition.

As an optional capability, if supported, early CSI acquisition starts before reception of LTM CSC MAC CE. As discussed in [7], the behaviour of CSI acquisition on neighbour cell is similar as that of CSI-RS based L1-RSRP measurement on candidate cell. Interruptions on serving cell will happen similarly as for measurement. While considering the complexity of this issue and the limited time for completing the core requirement in R19, we suggest not to define requirements for early CSI acquisition on candidate cells.

Proposal 6: not define RRM requirement for early CSI acquisition on candidate cells for LTM.

# Conclusion

This contribution proposed our consideration on open issue for CSI-RS based L1 measurement. The following proposals are concluded.

Proposal 1: It is proposed to send LS to RAN1/2 about RAN4 agreement on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

Proposal 2: Clarify in the measurement requirement applicability that the requirement is not applicable when a serving cell is included in the candidate cell for LTM.

Proposal 3: Either option 1 with scheduling restriction or option 2 with sharing factor of 2:1 between serving cell and neighbor cell could be considered.

Proposal 4: clarify in the requiremetn applicability that RAN4 The requirements in the clause 9.14a are applicable to FR1 and FR2-1 for LTM when the configured CSI-RS based L1 RSRP measurement for LTM candidate cell doesn‘t exceeding UE measurement capability.

Proposal 5: either Option 1 or Option 2 is ok for measurement restriction.

Proposal 6: not define RRM requirement for early CSI acquisition on candidate cells for LTM.

# Reference

[1] RP-234036, New WID: NR mobility enhancements Phase 4, Apple, RAN#102

[2] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#112bis

[3] R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, RAN4#113

[4] R4-2504049, WF on RRM requirements for NR_Mob_Ph4_Part1, RAN4#114bis

[5] R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, RAN4#115

[6] R4-250xxxx, MediaTek

[7] R4-250xxxx, Qualcomm