# 3GPP TSG-RAN WG4 Meeting #116	R4-2509478

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.12.3

Source:	Apple

Title:	Discussion on Spatial Channel Modeling

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#115 spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution we share our views on the study for spatial channel models for demodulation performance requirements in RAN4.

# 2. Discussion

## General

In RAN4#115 the following were discussed and agreed:

#### SU and MU MIMO prioritisation Way Forward: Remaining time in Study Item will focus on SU-MIMO, aspects related to MU-MIMO to be captured as is in TR 38.753, to be reviewed as part of pCR review. #### FR2 Agreement: FR2 to be postponed in Rel-19, SID to be updated.

Given that this is the last meeting for this SI, in RAN4#116 we should focus our efforts on drawing observations and conclusions on alignment and comparison aspects of the spatial channel model candidates.

Focus efforts in RAN4#116 on drawing observations and conclusions based on the alignment and comparison aspects of the spatial channel model candidates.

We should also capture items not addressed during this Rel-19 study to serve as a starting point for any further study in Rel-20.

Capture items not discussed or concluded in the Rel-19 study to serve as a starting point for any further study in Rel-20.

## CDL based methodologies

Frequency Generalization

For CDL based methodology we have defined a new CDL channel model for the study. The channel model is for UMa scenario and 3.5 GHz frequency. In RAN4 demod we define band agnostic requirements. But this channel model is only suitable to certain TDD bands. On frequency generalization, the following options were discussed:

Options

Option 1:  To select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

Option 2: Use 3.5GHz for all FR1 bands and duplexing, following legacy and SCM alignment.

We don’t think either of the options can be chosen without further study. We would need to understand the impact to performance by defining one or 2 channel models for FR1 bands. Without further study we also cannot consider UMa-CDL-C1 as a candidate to define demod requirements in Rel-20.

- RAN4 requirements are band agnostic

- UMa-CDL-C1 channel is only for 3.5 GHz band and suitable for some TDD bands.

- Options discussed in last meeting still need further study and evaluation

- UMa-CDL-C1 is not a suitable candidate for defining requirements without further study on frequency generalization.

RAN4 should further study frequency generalization of UMa-CDL-C1 in Rel-20 prior to defining any demodulation performance requirements.

Further study frequency band generalization aspects of UMa-CDL-C1 prior to defining any demodulation performance requirements with it.

Time for Convergence

Another aspect to study for the CDL based methodology was time for convergence of results. We conducted study with different simulation lengths, different seed initialization of UMa-CDL-C1 model, and found that there is no repeatability issue with different seeds. The convergence time is a function of Doppler like legacy TDL channel models.

- For Uma-CDL-C1 channel we observe similar performance with different initial seeds.

- For UMa-CDL-C1 the convergence time is comparable to legacy TDL channel models.

## Alignment of SCM implementations

In RAN4#115 the following was discussed and agreed for alignment:

#### Alignment of SU PDSCH Cases Way Forward: Observations about clustering of results to be captured in the TR. Agreement: SU PDSCH statements on alignment to be made at 70% TPUT, and if appropriate several points of TPUT (including 30%) #### Alignment of SU PMI Cases Agreement: SU PMI statements on alignment to be made at 90% and 70% of Max TPUT.

In RAN4#115 results for alignment was collected in [3] for SU-PDSCH and SU-PMI test cases.

Based on the results with UMa-CDL-C1 in [3] we have the following observations:

- SU-MIMO PDSCH with 4x4 4L – good alignment among companies

- SU-MIMO PDSCH with 8x8 8L – large span in results

- SU-PMI with 8x4 with 4L – Type I codebook – well aligned at least for low TP point

- SU-PMI with 8x4 with 4L – eType II codebook – not very well aligned

For SU-MIMO PDSCH with 4L is not the best use case for spatial channel model. For 4x4 it should be possible to find a TDL channel model with XP antenna array configuration and certain MIMO correlation with similar spatial properties as UMa-CDL-C1. Also, for PMI, eType II codebook was the target for SCM study.

For the more interesting use cases of spatial channel model, we don’t have good alignment in performance.

- There is good alignment in results with Uma-CDL-C1 for 4x4 4L PDSCH, PMI with Type I CB .

- Large span is results is observed with UMa-CDL-C1 for 8x8 8L test case and PMI with eType II codebook – the targeted use cases for SCM.

It should be noted that we typically don’t align SNR operating point with follow PMI in RAN4 and usually only look at TP gain metric.

- It is not typical to align operating SNR for PMI reporting requirements.

## Comparison of methodologies

Metrics for comparison

We should compare the different SCM candidates among each other and with legacy channel model based on operating SNR for PDSCH, and gamma for PMI test cases. Operating SNR could be and additional metric for PMI test cases. Full TP curves might not be needed for comparison as well.

SU-MIMO PDSCH

SNR at 70% Max TP

SU-PMI

Gamma at 90% Max TP with follow Type I PMI

Gamma at 90% Max TP with follow eType TI PMI

[SNR at 70 % Max TP for follow Type I PMI ]

[SNR at 70 % Max TP for follow eType II PMI]

Test metrics for performance comparison:
SU-MIMO PDSCH
     SNR at 70% Max TP
SU-PMI
      Gamma at 90% Max TP with follow Type I PMI 
     Gamma at 90% Max TP with follow eType TI PMI
     SNR at 70% Max TP for follow Type I PMI
     SNR at 70%Max TP for follow eType II PMI

Legacy channel models for comparison

The legacy channel models for comparison should be chosen to match UMa-CDL-C1 in some respects. We don’t think ULA with correlation is suitable for comparison with CDL which employs X-pol antenna. Also, the correlation coefficient should be chosen accordingly. For example, ULA MedA is not suitable for > 2 layer transmission.

- Comparing UMa-CDL-C1 with ULA antenna array configuration is not appropriate.

- Correlation coefficient with TDL channel must be chosen based on number of TX/RX. There is no suitable MIMO correlation in RAN4 for 8x8 with ULA or XP.

Antenna array configuration and correlation coefficient of legacy channel model for comparison must be chosen to match UMA-CDL-C1 in some respects.

# 3. Conclusion

In this paper, we provide our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4. Our observations and proposals are captured below:

General

- Focus efforts in RAN4#116 on drawing observations and conclusions based on the alignment and comparison aspects of the spatial channel model candidates.

- Capture items not discussed or concluded in the Rel-19 study to serve as a starting point for any further study in Rel-20.

CDL based methodologies

- RAN4 requirements are band agnostic

- UMa-CDL-C1 channel is only for 3.5 GHz band and suitable for some TDD bands.

- Options discussed in last meeting still need further study and evaluation

- UMa-CDL-C1 is not a suitable candidate for defining requirements without further study on frequency generalization.

Further study frequency band generalization aspects of UMa-CDL-C1 prior to defining any demodulation performance requirements with it.

- For Uma-CDL-C1 channel we observe similar performance with different initial seeds.

- For UMa-CDL-C1 the convergence time is comparable to legacy TDL channel models.

Alignment

- There is good alignment in results with Uma-CDL-C1 for 4x4 4L PDSCH, PMI with Type I CB .

- Large span is results is observed with UMa-CDL-C1 for 8x8 8L test case and PMI with eType II codebook – the targeted use cases for SCM.

- It is not typical to align operating SNR for PMI reporting requirements.

Comparison of methodologies

Test metrics for performance comparison:
SU-MIMO PDSCH
     SNR at 70% Max TP
SU-PMI
      Gamma at 90% Max TP with follow Type I PMI 
     Gamma at 90% Max TP with follow eType TI PMI
     SNR at 70% Max TP for follow Type I PMI
     SNR at 70%Max TP for follow eType II PMI

- Comparing UMa-CDL-C1 with ULA antenna array configuration is not appropriate.

- Correlation coefficient with TDL channel must be chosen based on number of TX/RX. There is no suitable MIMO correlation in RAN4 for 8x8 with ULA or XP.

Antenna array configuration and correlation coefficient of legacy channel model for comparison must be chosen to match UMA-CDL-C1 in some respects.

# Reference

- R4-2508776, “Way Forward for [115][321] NR_SCM”, Nokia.

- R4-2509479, “Simulation results for Spatial Channel Modeling study”, Apple

- R4-2508621, “Simulation summary for Alignment for NR SCM”, Apple, Ericsson.