# 3GPP TSG-RAN WG4 Meeting #116	R4-2509479

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.12.3

Source:	Apple

Title:	Simulation results for Spatial Channel Modeling study

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#115 spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution present simulation results for spatial channel model study.

# 2. Simulation Results

For the simulation parameters below, we present our results.

Table 1: Simulation Parameters for Spatial Channel Model Study

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |
| --- | --- | --- | --- |
| Duplex mode |  | TDD |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |
| Number of Tx Ports |  | 4 | 8,16,32 |
| Number of Rx Ports/Antennas |  | 4 | 4 |
| Number of layers |  | 4 | 4,2 |
| PMI |  | Random | Follow, Random |
| Waveform |  | CP-OFDM with normal CP |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |
| MCS |  | 13 (64 QAM table) |  |
| PDSCH configuration | Mapping type | Type A |  |
|  | k0 | 0 |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PDSCH aggregation factor | 1 |  |
|  | Resource allocation type | Type 0 |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP (for follow/ random), eType-II (for follow) |
|  | Codebook configuration | For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For 8TX: (N1,N2,O1,O2) = (4,1,4,1) For 16 TX: (N1,N2,O1,O2) = (8,1,4,4) For 32 TX: (N1,N2,O1,O2) = (8,2,4,4) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) | WB PMI for follow Type I WB PMI for random Type I SB PMI for eType II |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer | 8 CSI-RS Ports (4,1) 16 CSI-RS Ports (8,1) 32 CSI-RS Ports (8,2) |
|  | Density (ρ) | 1 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |
| CQI/RI/PMI delay |  | N/A | 7 ms |
| Number of HARQ Processes |  | 8 |  |
| Maximum HARQ transmissions |  | 4 |  |

Channel Models:

- TDLC300

- UMa-CDL-C1: CDL (TR 38.753) based on truncated CDL-C from Table 7.7.1-3 in TR 38.901

Doppler/Speed:

For TDL Models: 10, 100 Hz

For CDL Models: 3, 30 kph

AAV Configuration:

Pass through AAV

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,8,2,1,1) for 16Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (2,8,2,1,1) for 32Tx CSI-RS Ports

SU-MIMO PDSCH Performance

In the table we provide simulation results for alignment and comparison.

Table 2:Simulation results for SU-MIMO PDSCH for alignment and comparison

| Channel Model w/ Ant Corr | Doppler/ Speed | SNR @ 70% Max TP | SNR @ 30% Max TP |
| --- | --- | --- | --- |
| UMa-CDLC-A2 | 3 kph | 15.5 | 8.5 |
| TDLC-300 ULA Low | 10 Hz | 14.0 | 7.4 |
| TDLC-300 XP Med | 10 Hz | 15.3 | 8.4 |
| TDLC-300 ULA MedA | 10 Hz | NaN | 19.1 |
| UMa-CDLC-A2 | 30 kph | 16.3 | 8.7 |
| TDLC-300 ULA Low | 100 Hz | 14.5 | 7.5 |
| TDLC-300 XP Med | 100 Hz | 16.1 | 8.3 |
| TDLC-300 ULA MedA | 100 Hz | NaN | 20.1 |

From the results, we observe the following:

- Performance with XP-Med is comparable to UMa-CDL-C1

- ULA-Med A is not suitable for 4 L transmissions

For SU-PDSCH for 4x4 with 4L -
- Performance with TDLC300-XP-Med is comparable to UMa-CDL-C1 for low and high doppler
- ULA MedA is not suitable for 4L transmission

SU-MIMO PMI-PDSCH Performance

Performance results for SU-PMI with Type I and eType II are presented in the tables below.

Table 3: SU-PMI simulation results with 8TX

| Channel /MIMO Corr | Doppler/Speed | Num Layers | g [TP gain over Random PMI] |  | SNR @ 90% Max TP [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eType II | Type I | eType II | Type I |
| TDLC300 XP-Low | 10 Hz | 2 | 1.78 | 1.49 | 5.5 | 6.6 |
| TDLC300 XP-Med |  |  | 2.55 | 2.81 | 4.0 | 3.6 |
| TDLC300 XP-High |  |  | 3.25 | 6.12 | 4.7 | 3.6 |
| UMa-CDL-C1 | 3 kph |  | 1.88 | 1.97 | 7.5 | 7.0 |
| TDLC300 XP-Low | 10 Hz | 4 | 1.25 | 1.30 | 13.8 | 13.6 |
| TDLC300 XP-Med |  |  | 1.82 | 1.87 | 14.1 | 13.7 |
| TDLC300 XP-High |  |  | 2.13 | 1.99 | 19.2 | 20.0 |
| UMa-CDL-C1 | 3 kph |  | 1.66 | 1.78 | 15.9 | 14.4 |
| TDLC300 XP-Low | 10 Hz | 2 | 0.96 | 0.98 | 8.0 | 7.9 |
| TDLC300 XP-Med |  |  | 2.01 | 2.66 | 5.5 | 3.9 |
| TDLC300 XP-High |  |  | 2.84 | 4.15 | 5.3 | 3.9 |
| UMa-CDL-C1 | 3 kph |  | 1.69 | 1.81 | 8.7 | 7.9 |
| TDLC300 XP-Low | 10 Hz | 4 | 0.90 | 1.12 | 16.7 | 15.1 |
| TDLC300 XP-Med |  |  | 1.80 | 1.83 | 15.6 | 14.5 |
| TDLC300 XP-High |  |  | 2.16 | 1.96 | 19.7 | 21.2 |
| UMa-CDL-C1 | 3 kph |  | 1.42 | 1.75 | 17.3 | 15.1 |

Table 4: SU-PMI simulation results with 16TX

| Channel /MIMO Corr | Doppler/Speed | Num Layers | g [TP gain over Random PMI] |  | SNR @ 90% Max TP [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eType II | Type I | eType II | Type I |
| TDLC300 XP-Low | 10 Hz | 2 | 1.80 | 1.79 | 4.5 | 5.8 |
| TDLC300 XP-Med |  |  | 3.99 | 4.21 | 2.5 | 2.3 |
| TDLC300 XP-High |  |  | 188.39 | 1937.37 | 1.9 | 1.2 |
| UMa-CDL-C-A2 | 3 kph |  | 5.05 | 8.16 | 4.3 | 3.8 |
| TDLC300 XP-Low | 10 Hz | 4 | 1.37 | 1.33 | 13.4 | 13.5 |
| TDLC300 XP-Med |  |  | 2.09 | 2.09 | 11.9 | 11.9 |
| TDLC300 XP-High |  |  | 6.23 | 4.69 | 14.2 | 15.0 |
| UMa-CDL-C-A2 | 3 kph |  | 2.30 | 2.78 | 13.0 | 11.8 |

Table 5: SU-PMI simulation results with 32TX

| Channel /MIMO Corr | Doppler/Speed | Num Layers | g [TP gain over Random PMI] |  | SNR @ 90% Max TP [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eType II | Type I | eType II | Type I |
| TDLC300 XP-Low | 10 Hz | 2 | 1.85 | 1.80 | 3.8 | 5.6 |
| TDLC300 XP-Med |  |  | Inf | Inf | -0.6 | -0.7 |
| TDLC300 XP-High |  |  | Inf | Inf | -0.8 | -2.2 |
| UMa-CDL-C-A2 | 3 kph |  | Inf | Inf | 1.9 | 1.9 |
| TDLC300 XP-Low | 10 Hz | 4 | 1.52 | 1.40 | 13.0 | 13.4 |
| TDLC300 XP-Med |  |  | 266.35 | 175.92 | 9.2 | 9.4 |
| TDLC300 XP-High |  |  | Inf | Inf | 11.4 | 11.9 |
| UMa-CDL-C-A2 | 3 kph |  | 7.78 | 5.80 | 10.2 | 10.9 |

Based on the results, we have the following observations:

- The performance improvement with eType II can be seen in legacy TDL and well as CDL channel models with larger array

- X-pol with Medium / High antenna correlation are not suitable for large number of TX ports/ antenna for > 2layer

eType II performance is better than Type I with larger CSI-RS ports.

The performance improvement with eType II can be seen in legacy TDL and well as CDL channel models

X-pol with High antenna correlation is not suitable for large number of TX ports/ antenna for > 2layer

For eType II we compare performance between different channel models – existing TDL channel models and Uma-CDL-C.

Figure 3: Follow PMI with eType II performance comparison

Based on the results in figure 2, the performance with follow PMI with eType II is not significantly better than existing TDL channel models with/ without correlation.

The performance of follow PMI with eType II is not significantly better than existing TDL channel models.

# 3. Conclusion

In this paper, we provide our simulation results for study of spatial channel models. Our observations are captured below:

SU-MIMO PDSCH performance

- For SU-PDSCH for 4x4 with 4L -
- Performance with TDLC300-XP-Med is comparable to UMa-CDL-C1 for low and high doppler
- ULA MedA is not suitable for 4L transmission

SU-MIMO PMI PDSCH performance

eType II performance is better than Type I with larger CSI-RS ports.

The performance improvement with eType II can be seen in legacy TDL and well as CDL channel models

X-pol with High antenna correlation is not suitable for large number of TX ports/ antenna for > 2layer

The performance of follow PMI with eType II is not significantly better than existing TDL channel models.

# Reference

- R4-2508776, “Way Forward for [115][321] NR_SCM”, Nokia.