# 3GPP TSG-RAN WG4 Meeting #116	R4-2509482

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.13.3

Source:	Apple

Title:	On UE demodulation performance requirements with 8RX

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#115 the UE demodulation enhancements in Rel-19 were discussed in detail and way forward [1] was agreed. In this contribution we share our views on remaining open issues UE demodulation requirements with 8RX.

# 2. Discussion

## General

In RAN4#115 the only open issue is the receiver capability for 8RX UE definition:

Definition of MMSE-IRC receiver capability for 8Rx UE

Options:

- Option 1: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 2: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 3: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 4:
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 5:
Re-use existing definition in Rel-18.

The existing definition for baseline and simplified receivers is:

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

The issue with existing definition is the term “SU-MIMO transmissions”, which is very limiting. We are defining requirements for IUI with MU-MIMO transmission from the gNB. Hence, that change needs to be done. At a minimum the term SU-MIMO transmissions should be removed from definition.

The term “SU-MIMO transmissions” in current definition lends it inapplicable for Rel-19 scenario of MU-MIMO transmission for requirements with IUI.

The Rel-18 definition cannot be used for requirements with IUI.

At a minimum “SU-MIMO transmissions” should be renamed to “MIMO transmission” in the definition.

In the earlier agreement from RAN4#114bis, the MIMO detector assumption which is MMSE-IRC was also included in the definition. That resulted in a lot of discussion. The requirements are defined with assumption of MMSE-IRC receiver which is the baseline receiver assumption in NR. It doesn’t preclude UE to implement different or more advanced receiver as long as it can meet the requirements.

Including MMSE-IRC in the definition doesn’t preclude UE to implement different or more advanced receiver if it can meet the requirements.

The term SU-MIMO in the receiver name is also not needed. We propose to remove it.

There is no need to have “SU-MIMO” in the name.

Based on the above observation. We propose the following definition:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmission with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmission with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmission with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmission with support of only up to 4 layers with two joint 4Rx MIMO detectors.

# 3. Conclusion

In this paper, we provide our views on UE demodulation requirements with 8RX. Our observations and proposals are captured below:

General

- The term “SU-MIMO transmissions” in current definition lends it inapplicable for Rel-19 scenario of MU-MIMO transmission for requirements with IUI.

- The Rel-18 definition cannot be used for requirements with IUI.

- At a minimum “SU-MIMO transmissions” should be renamed to “MIMO transmission” in the definition.

- Including MMSE-IRC in the definition doesn’t preclude UE to implement different or more advanced receiver if it can meet the requirements.

- There is no need to have “SU-MIMO” in the name.

- Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmission with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmission with support of only up to 4 layers with two joint 4Rx MIMO detectors.

# Reference

- R4-2508620, “Way Forward for [115][323] NR_demod_Ph5_Part2_UE”, Nokia.