# 3GPP TSG-RAN WG4 Meeting #116	R4-2509486

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.20.4.2

Source:	Apple

Title:	On RRM Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#115 the other RRM requirements impact for MIMO phase 5 was discussed and WF [1] was approved. In this contribution we discuss RRM requirements impact for objectives other than UE initiated beam management.

# 2. Discussion

UE Reporting enhancement for CJT calibration

For CJT calibration the following agreements were made in RAN4#11:

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration? Agreement: - RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. - RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

In the past few meetings, for CJT calibration, the issue of deciding the number of samples has been discussed. Simulation parameters have been agreed for further evaluation. In RAN4#115 the simulation results for delay and frequency offset were compiled in [2].

We capture the results in terms of fraction of ppm for frequency offset error and fraction of CP for delay offset error in tables below.

Table 1: Error for Frequency Offset

| SCS | SNR | Freq Offset: 350Hz; FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max(CDF5%, CDF95%) fraction of ppm |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MTK |  |  | Ericsson |  |  | Huawei |  |  | Nokia |  |  |
|  |  | #samples |  |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 |
| 15KHz | -3 | 0.051 | 0.030 | 0.023 | 0.011 | 0.006 | 0.005 | 0.056 | 0.034 | 0.026 | 0.019 | 0.014 | 0.004 |
| 15KHz | 0 | 0.030 | 0.019 | 0.014 |  |  |  | 0.030 | 0.026 | 0.013 | 0.013 | 0.003 | 0.003 |
| 15KHz | 3 | 0.020 | 0.013 | 0.009 | 0.005 | 0.003 | 0.002 | 0.026 | 0.016 | 0.010 | 0.009 | 0.002 | 0.003 |
| 30KHz | -3 | 0.104 | 0.055 | 0.047 | 0.020 | 0.013 | 0.009 | 0.116 | 0.052 | 0.046 | 0.033 | 0.014 | 0.015 |
| 30KHz | 0 | 0.065 | 0.038 | 0.029 |  |  |  | 0.081 | 0.045 | 0.027 | 0.021 | 0.015 | 0.015 |
| 30KHz | 3 | 0.041 | 0.024 | 0.019 | 0.009 | 0.005 | 0.005 | 0.053 | 0.027 | 0.022 | 0.013 | 0.002 | 0.004 |

Table 2: Error for Time offset

| SCS | SNR | Tome offset: 0.5 CP; TO estimation error in AD = CP and MD = 256 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max(CDF5%, CDF95%) Fraction of CP |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MTK |  |  | Ericsson |  |  | Huawei |  |  | Nokia |  |  |  |
|  |  | #samples |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 |  |
| 15KHz | -3 | 0.113 | 0.074 | 0.070 | 0.031 | 0.023 | 0.016 | 0.219 | 0.156 | 0.141 | 0.031 | 0.008 | 0.004 |  |
| 15KHz | 0 | 0.066 | 0.047 | 0.039 |  |  |  | 0.125 | 0.094 | 0.078 | 0.031 | 0.008 | 0.004 |  |
| 15KHz | 3 | 0.039 | 0.027 | 0.023 | 0.023 | 0.023 | 0.016 | 0.094 | 0.063 | 0.047 | 0.020 | 0.008 | 0.004 |  |
| 30KHz | -3 | 0.117 | 0.074 | 0.066 | 0.039 | 0.031 | 0.023 | 0.125 | 0.078 | 0.063 | 0.031 | 0.008 | 0.004 |  |
| 30KHz | 0 | 0.063 | 0.043 | 0.039 |  |  |  | 0.078 | 0.063 | 0.047 | 0.031 | 0.008 | 0.004 |  |
| 30KHz | 3 | 0.055 | 0.035 | 0.035 | 0.039 | 0.031 | 0.023 | 0.063 | 0.047 | 0.031 | 0.031 | 0.004 | 0.004 |  |

Based on the results in the tables above the improvement with larger number of samples is in the order of 1/100ths of CP for time offset and 1/100ths of ppm for frequency offset. It is obvious that more averaging would give us better results in terms or accuracy. But it is hard to understand the impact of error when the resolution is so small.

The improvement with > 1 sample is in the order of 1/100 ths of ppm for frequency offset and 1/100 ths of CP for time offset.

The measurement accuracy would improve with more samples, but the impact is unknown given the small resolution.

Based on the above observations, we propose that the core requirements are defined for CJT calibration with assumption of one-shot measurement reporting.

- Define core requirements for CJT calibration reporting enhancement for delay/frequency reporting based on single shot measurement.

Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration?

Way forward:

- Option 1: The scheduling restriction is specified as:

  - For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting.

- Option 2: No scheduling restrictions needed to be applied for CJT reporting.

The proposal on scheduling restriction was motivated based on the UE indicating a 1 bit when the estimated delay offset plus delay spread exceeds CP length. The baseline assumption of CJT transmission is that the MRTD is within CP. In case a scheduling restriction is introduced to avoid any ISI to ensure error free measurement based on TRS, then the scheduling restriction is to avoid any transmission on other TRPs on symbols where TRS is transmitted and adjacent symbols. The necessity of scheduling restriction needs to be further discussed. While the scheduling restriction is needed for ISI free measurement, it also would restrict the scheduling significantly

Baseline assumption for CJT is that MRTD is within CP.

- RAN4 to discuss the necessity of scheduling restriction for CJT calibration reporting.

Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration?

Way forward:

- Option 1: Reuse the legacy measurement restriction in FR1 for delay and frequency offset measurement.

  - Measurement restriction does apply,

    - If UE does not support simultaneousRxDataSSB-DiffNumerology and

    - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols.

  - Measurement restriction does not apply,

    - If UE support simultaneousRxDataSSB-DiffNumerology or

    - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH.

- Option 2: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

The CJT calibration reporting is only in FR1. For other L1 measurements scheduling restriction is based on UE support of simultaneousRxDataSSB-DiffNumerology and SCS of SSB and CSI-RS in FR1. IN FR2 the scheduling restriction is that UE is expected to measure one of the resources, and measurement period might be longer. Since CJT calibration reporting is in FR1 the first option for measurement restriction would be more relevant and appropriate.

CJT calibration reporting is in FR1 alone.

The different options discussed in RAN4#115 are based on FR1 (option 1) and FR2 (option 2)

In FR1 for other L1 measurements the measurement restriction is based on UE support of simultaneousRxDataSSB-DiffNumerology and SCS of SSB and CSI-RS.

- Define measurement restriction for CJT calibration reporting based on UE support of simultaneousRxDataSSB-DiffNumerology and SCS of SSB and CSI-RS.

Asymmetric DL sTRP/UL mTRP

For asymmetric DL sTRP/UL mTRP  the following agreements were made in RAN4#114bis:

Issue 2-4: For Scenario#2, how to update PRACH and PUCCH/PUSCH/SRS transmission requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: For Scenario#2, inter-cell operation, PRACH reference point: - reference point for PRACH transmission is - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL [or UL] TCI state(s) of the reference cell associated with the second TAG For inter-cell multi-TRP, for PUCCH/PUSCH/SRS: - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [or UL TCI states] having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point. Issue 2-5: For Scenario#2, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Autonomous timing adjustment applies to both TAGs or TRPs. - Two independent gradual timing adjustments with potentially different values should be applied to both TAGs. Issue 2-6: For Scenario#2, if the UE is configured with SSB-MTC-additionalPCI, how to define MRTD and MTTD requirements? Agreement: - Reuse Rel 18 values of MRTD and MTTD for Rel 19. Issue 2-7: Whether to define UL TCI state activation requirement for Scenario#2? Agreement: The existing requirements for unified TCI state with mTRP are applicable to Scenario#2 and no new requirements to be introduced. FFS on spec impact

On the timing requirements for 2TA, RAN4 has been discussing based on scenario 1 and scenario 2 –

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward:

- Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG.

- Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

In Rel-18 MIMO evo, for timing requirements with 2 TA, the gradual timing adjustment requirements were applicable to each TAG. In case of asymmetrical DL sTRP/ UL mTRP, at least for scenario 1 there is only 1 DL/UL TRP with corresponding reference timing. In case the UE needs to apply gradual timing adjustment based on propagation delay change at the UE, it can only be applicable to the TRP it receives DL reference from. The same gradual timing adjustment might not be applicable for the other TRP, as UE cannot predict if the same propagation delay change applies to UL only TRP.

In scenario #1, with single timing reference UE can only apply gradual timing to the TRP it receives DL RS for timing reference.

In scenario #1 the propagation delay change necessitating a gradual timing adjustment on DL/UL TRP might not be applicable to UL only TRP.

Hence, the gradual timing adjustment requirements shall be applicable only to TRP with both UL and DL.

- For scenario #1 - The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

UE Reporting enhancement for CJT calibration

- The improvement with > 1 sample is in the order of 1/100 ths of ppm for frequency offset and 1/100 ths of CP for time offset.

The measurement accuracy would improve with more samples, but the impact is unknown given the small resolution.

- Define core requirements for CJT calibration reporting enhancement for delay/frequency reporting based on single shot measurement

Baseline assumption for CJT is that MRTD is within CP.

- RAN4 to discuss the necessity of scheduling restriction for CJT calibration reporting.

CJT calibration reporting is in FR1 alone.

The different options discussed in RAN4#115 are based on FR1 (option 1) and FR2 (option 2)

In FR1 for other L1 measurements the measurement restriction is based on UE support of simultaneousRxDataSSB-DiffNumerology and SCS of SSB and CSI-RS.

- Define measurement restriction for CJT calibration reporting based on UE support of simultaneousRxDataSSB-DiffNumerology and SCS of SSB and CSI-RS.

Asymmetric DL sTRP/UL mTRP

In scenario #1, with single timing reference UE can only apply gradual timing to the TRP it receives DL RS for timing reference.

In scenario #1 the propagation delay change necessitating a gradual timing adjustment on DL/UL TRP might not be applicable to UL only TRP.

- For scenario #1 - The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

# Reference

- R4-2508273, “WF on RRM requirements for NR_MIMO_Ph5_Part2”, MediaTek.

- R4-2508292, “Collection of simulation results for CJT calibration in NR_MIMO_Ph5_Part2”, MediaTek.