# 3GPP TSG-RAN WG4 Meeting #116	R4-2509487

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.20.5

Source:	Apple

Title:	On RRM Performance Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#115 the RRM performance requirements for Rel-19 MIMO Phase 5 was discussed and way forward [1] was agreed. In this contribution we present our views on the performance requirements impact and test cases for this WI.

# 2. Discussion

### UE Initiated/ Event Triggered Beam Reporting

For UE-initiated beam reporting, the following table needs to be further discussed:

|  | QC | Nokia | Samsung | Apple | CMCC | Huawei | Ericsson |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Y |  | Event-2 1 TC for Event-2 No for Event-7 |  | Y | Event-2 FFS on others | Event-2&7 |
| Whether to cover Mode A and Mode B? |  | Mode A FFS on Mode B | Y |  |  |  | Mode A at least |
| Event-trigger for Intra-cell BM for SSB | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y |  |  | FFS | ? |
| Event-trigger for Inter-cell BM for CSI-RS |  | FFS |  |  |  |  |  |
| Whether to cover no time window and eventInstanceCount-r19 > 1? |  |  | Y |  | Y |  | Y |
| FR2? |  |  |  | Y |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI |  |  |  |  |  | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |

On a high level there is consensus to define test case(s) to verify UEIBM.

TCs for different Events

The core requirements have primarily been discussed based on Event 2 and in RAN4#115 there was agreement that the same design would be used for requirements with Event 1 and 7. The events differ in the conditions that trigger the event, while the reporting delay requirements are the same. Also, the beams measured for each of the events are different. For event 1 only current beam – RS of the indicated TCI state is measured; For Event 2 current beam and new beam are measured; For Event 7 the current beam – RS of the qth best activated TCI state and new beams are measured. The main aspects are measurement and comparison against a threshold. It would be sufficient to define performance test case for Event 2 as nothing different is tested by introducing test cases for event 1 and event 7.

- Introducing test case for Event 2 is sufficient and doesn’t test anything different with test cases for Event 1 or Event 7.

Introduce test case for Event 2 alone.

TCs for Mode A/B

The ending point is defined as when UE send first PUCCH which is the same for Mode A and Mode B. There is nothing different tested by introducing test cases for Mode A and Mode B. Only define test for either Mode A or Mode B, Mode A would be preferred.

- Given the ending point of first PUCCH transmission, the requirements are the same for Mode A and Mode B.

Introduce test case for either Mode-A (preferred) or Mode-B.

TCs for different scenarios

The requirements are applicable for inter-cell and intra-cell scenario. For SSB based, we explicitly have inter-cell and intra-cell scenario, while for CSI-RS based they cannot be differentiated. In order to cover both scenarios and minimize tests, we propose to introduce requirements for SSB based with inter-cell and CSI-RS based without explicitly saying inter-cell or intra-cell.

Introduce test cases for (1) CSI-RS based event trigger and (1) inter-cell SSB based event trigger.

TCs for time window-based evaluation

We don’t see it feasible to define any core requirements for window-based evaluation with  eventInstanceCount-r19 > 1and also propose not to define a test case for the same.

Do not introduce test cases for time window based evaluation with eventInstanceCount-r19 > 1.

Applicable frequency range

The core requirements are defined for FR2 with priority and the test case shall also be defined for FR2 alone.

Introduce test cases in FR2 alone.

TCs for beam switching delay

In RAN4#115 it was agreed that existing requirements for TCI state switch delay are applicable for UE-initiated/event-triggered beam management. Also, there is no enhancement to beam switching delay with  UE-initiated/event-triggered beam management. We don’t need to introduce a test case for the same.

Do not introduce test cases for TCI state switching/ beam switching delay for UE-initiated/event-triggered beam management.

### CJT Calibration Reporting

For CJT calibration, we agreed to introduce the following requirements:

- Define core requirements for delay and frequency offset reporting for

  - Measurement period (FFS number of samples)

  - Scheduling restriction (if identified)

The number of samples for CJT reporting is FFS.

For CJT calibration reporting, delay, frequency and phase offset are reported. The following agreements are made for these quantities:

Time offset: AD(Max delay offset)∈CP2,CP;MD(resolution) ∈{32, 64, 128, 256}

Frequency offset: AFO(Max freq offset)∈0.1ppm, 0.2 ppm;MFO(resolution) ∈{16, 32,  256}

Phase offset: n, -uniformly quantized phase between 0 and 2; MΦresolution∈{16, 32}

RAN4 should define mapping tables for the time offset, frequency offset, phase offset reporting based on the agreements of range and resolution.

RAN4 to define mapping tables for time offset, frequency offset, phase offset reporting based on the respective of range and resolution for CJT calibration reporting.

In core requirements we have collected simulation results to determine if we need to define requirements with > 1 sample for measurement. Based on the results, the accuracy is diverse among companies.

- Based on the results collected for FO/ TO accuracy the results are very diverse.

RAN4 should discuss feasibility of defining accuracy requirements for delay and frequency offset reporting for CJT calibration. In Rel-19 RAN4 will not introduce core requirements for phase offset reporting, hence no performance test cases are required.

Test cases for phase offset reporting need not be discussed as there are no core requirements.

RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

### Asymmetric DL sTRP/ UL mTRP

RAN4 has agreed to define timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP. The following scenarios are considered:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

On test scope the following table is used as a starting point –

| High level: | QC | Nokia | Apple | Samsung | CMCC | Huawei |
| --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y |  | Y |
| Whether to define TCs to verify TCI state switching requirements? | Y |  |  | N |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | both | both | both |  | both | Scenarios#1 |

RAN4 shall introduce timing requirements for both scenarios 1 and 2 for 2TA with asymmetrical DL sTRP/UL mTRP. Since the requirements are different, we see the necessity to introduce test cases for both scenario 1 and scenario 2. But it is to be noted that scenario 2 requirements are the same as mTRP requirements with 2 TA.

RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP.

Test cases to cover both scenario 1 and scenario 2.

TCs for TCI state switching delay

In RAN4#115 it was agreed that existing requirements for unified TCI state switching delay for mTRP are applicable for scenario 2. We don’t see the necessity to introduce est case when no new requirement is introduced for asymmetrical DL sTRP/ UL mTRP use case.

Do not introduce test cases for TCI state switching for asymmetrical DL sTRP/ UL mTRP.

Applicable frequency range

The use case and requirements for asymmetrical DL sTRP/ UL mTRP are applicable to both FR1 and FR2. In practice FR2 would most likely use Scenario#2 rather than scenario 1. Instead of introducing test cases for both FR1 and FR2 for both scenarios, define requirements for scenario 1 in FR1 and scenario 2 in FR2.

Introduce test cases for scenario 1 in FR1 and scenario 2 in FR2.

# 3. Conclusion

In this paper, we present our views on the performance part impact and test cases for this WI. Our observations and proposals are captured below:

UE Initiated/ Event Triggered Beam Reporting

- Introducing test case for Event 2 is sufficient and doesn’t test anything different with test cases for Event 1 or Event 7.

- Introduce test case for Event 2 alone.

- Given the ending point of first PUCCH transmission, the requirements are the same for Mode A and Mode B.

Introduce test case for either Mode-A (preferred) or Mode-B.

- Introduce test cases for (1) CSI-RS based event trigger and (1) inter-cell SSB based event trigger.

- Do not introduce test cases for time window-based evaluation with eventInstanceCount-r19 > 1.

- Introduce test cases in FR2 alone.

- Do not introduce test cases for TCI state switching/ beam switching delay for UE-initiated/event-triggered beam management.

CJT Calibration Reporting

RAN4 to define mapping tables for time offset, frequency offset, phase offset reporting based on the respective of range and resolution for CJT calibration reporting.

- Based on the results collected for FO/ TO accuracy the results are very diverse.

Test cases for phase offset reporting need not be discussed as there are no core requirements.

RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

Asymmetric DL sTRP/ UL mTRP

RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP.

Test cases to cover both scenario 1 and scenario 2.

Do not introduce test cases for TCI state switching for asymmetrical DL sTRP/ UL mTRP.

Introduce test cases for scenario 1 in FR1 and scenario 2 in FR2.

# Reference

- R4-2508271, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.