# 3GPP TSG-RAN WG4 Meeting #116	R4-2509489

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.27.7

Source:	Apple

Title:	On UE Demodulation Requirements for NTN Phase 3

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The UE demodulation requirements for Rel-19 NTN Phase 3 WI were discussed in RAN4#115 and way forward [1] was agreed. In this contribution we present our views on the scope of UE demodulation requirements for this WI.

# 2. Discussion

In RAN4#115 the following agreements were made for scope of UE demod requirements:

Issue 3-1-1: PDCCH with repetition for CSS Agreement: - Not define requirements for PDCCH with repetition. Issue 3-1-2: PDSCH with repetition for Msg4/SIB1 Agreement: - Not define requirements for PDSCH with repetition for Msg4/SIB1. Issue 3-1-3: PBCH periodicity extension Agreement: - Not define requirements for PBCH periodicity extension. Issue 3-2-1: PDSCH demodulation requirements for RedCap Agreement: For 2Rx UEs, - Reuse Rel-17 NTN PDSCH demodulation requirements. - Companies can check whether all the Rel-17 NTN features can be applicable for RedCap. Issue 3-2-9: PDCCH/PBCH/SDR/CSI requirements Agreement: - Not define PDCCH, PBCH, SDR, and CSI reporting requirements for RedCap/eRedCap UEs over NR-NTN. - All the existing requirements including PDCCH, PBCH, SDR and CSI reporting requirements in TS38.101-4 can be applicable for Rel-19 NTN access capable RedCap/eRedCap UE according to the UE capability.

For UE demod we continue to discuss requirements and test cases for RedCap and eRedCap for NTN.

Issue 3-2-1: PDSCH demodulation requirements for RedCap

Agreement:

For 2Rx UEs,

- Reuse Rel-17 NTN PDSCH demodulation requirements.

  - Companies can check whether all the Rel-17 NTN features can be applicable for RedCap.

Way forward:

For 1Rx UE,

- FFS whether to add new PDSCH requirements with NTN channel or not.

For 2 RX UEs it was agreed to reuse existing requirements from Rel-17 NTN. Companies were encouraged to check if Rel-17 NTN features are applicable to RedCap UEs. We confirmed that the Rel-17 features of disabled HARQ, increased number of HARQ processes are also applicable to RedCap/eRedCap.

Rel-17 NTN features for NTN are applicable to RedCap/eRedCap UEs supporting non-terrestrial access.

The agreement made in RAN4#115 can be confirmed for requirements for 2RX.

- Confirm agreement in RAN4#115 to reuse Rel-17 NTN PDSCH requirements for 2RX RedCap UE.

For RedCap and eRedCap the UE could support either 1RX or 2RX. To have requirements coverage for all categories of UEs, requirements for both 1RX and 2RX should be introduced.

- Define requirements for 1RX RedCap and eRedCap UEs to have requirements coverage for all categories of UEs.

For RedCap with 1RX use the same test cases as Rel-17 NTN with 1 RX.

Test cases for NTN RedCap –

2RX – Reuse requirements from Rel-17 NTN

1RX – Define requirements with 1RX for the same testcases as 2RX

New test cases for RedCap 1RX

| Test Number | BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | 16 HARQ Proc | Could also be applicable to eRedCap without reduced BW |
| 1-2 | 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 13 | 16 HARQ Proc |  |
| 1-3 | 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | Could also be applicable to eRedCap without reduced BW |
| 1-4 | 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | Disabled HARQ | Could also be applicable to eRedCap without reduced BW |

Simulation effort for defining requirements for RedCap – 4 test cases with 1RX.

The proposed simulation effort in Rel-19 for RedCap NTN – 4 TCs with 1RX.

- Reuse the existing 2RX Rel-17 NTN test cases for 1RX requirements for RedCap NTN.

- Additional test cases for RedCap NTN with 1RX:

| Test Number | BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | 16 HARQ Proc |
| 1-2 | 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 13 | 16 HARQ Proc |
| 1-3 | 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 4 | 32 HARQ Proc |
| 1-4 | 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | Disabled HARQ |

For NTN eRedCap we need to define requirements with -

- Requirements with reduced baseband bandwidth for 1RX/ 2RX

- Requirements without reduced baseband bandwidth for 1RX/2RX

For the requirements without reduced baseband bandwidth some of the RedCap test cases with TBS ≤ 10000 bits can be used. For requirements with reduced baseband bandwidth new test cases would need to be defined for 1RX and 2X UEs.

Test cases for NTN RedCap with TBS ≤ 10000 bits can be applicable to eRedCap NTN UEs without reduced baseband bandwidth.

New test cases and requirements would need to be defined for eRedCap NTN UEs with reduced baseband bandwidth.

For eRedCap without reduced BW the existing test case with MCS13 would not be applicable. We propose to introduce a new TC for MCS 13 with reduced BW to be applicable to eRedCap.

We propose the following test cases for eRedCap NTN:

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | PRB size | TBS | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 50 | 4096 | Without reduced BW Reuse Test 1-3 from RedCap |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 50 | 4096 | Without reduced BW Reuse Test 1-4 from RedCap |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 13 | 16 HARQ Proc | 40 | 9992 | Without reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 13 | 16 HARQ Proc | 25 | 6272 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 25 | 1928 | With reduced BW |

The additional simulation effort for eRedCap NTN is 8 test cases.

The proposed simulation effort in Rel-19 for eRedCap NTN – 4 TCs with 1RX and 4 TCs with 2RX.

- Test cases for 2RX and 1RX requirements for eRedCap NTN:

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | PRB size | TBS | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 50 | 4096 | Without reduced BW Reuse Test 1-3 from RedCap |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 50 | 4096 | Without reduced BW Reuse Test 1-4 from RedCap |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 13 | 16 HARQ Proc | 40 | 9992 | Without reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 13 | 16 HARQ Proc | 25 | 6272 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 25 | 1928 | With reduced BW |

# 3. Conclusion

In this paper, we present our views on the scope of UE demodulation requirements for NTN Phase 3 WI and test cases for RedCap and eRedCap NTN. Our observations and proposals are captured below:

- Rel-17 NTN features for NTN are applicable to RedCap/eRedCap UEs supporting non-terrestrial access.

- Confirm agreement in RAN4#115 to reuse Rel-17 NTN PDSCH requirements for 2RX RedCap UE.

- Define requirements for 1RX RedCap and eRedCap UEs to have requirements coverage for all categories of UEs.

The proposed simulation effort in Rel-19 for RedCap NTN – 4 TCs with 1RX.

- Reuse the existing 2RX Rel-17 NTN test cases for 1RX requirements for RedCap NTN.

- Additional test cases for RedCap NTN with 1RX:

| Test Number | BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | 16 HARQ Proc |
| 1-2 | 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 13 | 16 HARQ Proc |
| 1-3 | 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 4 | 32 HARQ Proc |
| 1-4 | 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | Disabled HARQ |

The proposed simulation effort in Rel-19 for eRedCap NTN – 4 TCs with 1RX and 4 TCs with 2RX.

- Test cases for 2RX and 1RX requirements for eRedCap NTN:

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | PRB size | TBS | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 50 | 4096 | Without reduced BW Reuse Test 1-3 from RedCap |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 50 | 4096 | Without reduced BW Reuse Test 1-4 from RedCap |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 13 | 16 HARQ Proc | 40 | 9992 | Without reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 13 | 16 HARQ Proc | 25 | 6272 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 25 | 1928 | With reduced BW |

# Reference

- R4-2508755, “Way Forward for [115][333] NR_NTN_Ph3_demod”, Ericsson